| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0361-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0235 (Removal of LFNST for chroma components) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 zhi-yi.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This document reports the results of crosschecking JVET- R0235 proposed by Qualcomm. JVET- R0235 proposes to further disable LFNST for chroma in dual-tree partition mode. Thus, the LFNST process for chroma components is completely removed. The verification task was done, and the BD-rate results match those in JVET- R0235.

Verification

Qualcomm provided the source code of their proposed method described in JVET-R0235 [1]. One verification was carried out by setting the following parameters in the provided code:

#define REMOVE_CHROMA_LFNST                              1

#define REMOVE_CHROMA_LFNST_ENCODER                    1

#define INTRA_MODE_ENCODER_SEARCH                       0

The test was conducted under the common test conditions (CTC) defined in [2] by using the VTM8.0 software [3]. Table 1 shows the results using VTM8.0 as the anchor. The BD-rates of the test from our experiment match those in JVET-R0235.

Table 1.  The results of JVET-R0235 using VTM8.0 as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.56% | 1.97% | 95% | 99% |
| Class A2 | -0.04% | 1.58% | 1.60% | 92% | 98% |
| Class B | 0.12% | 1.05% | 1.47% | 97% | 99% |
| Class C | 0.25% | 0.66% | 0.92% | 94% | 101% |
| Class E | 0.22% | 1.08% | 0.98% | 97% | 103% |
| Overall | 0.12% | 1.14% | 1.37% | 95% | 100% |
| Class D | 0.26% | 0.86% | 0.98% | 94% | 106% |
| Class F | 0.23% | 0.54% | 0.49% | 97% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.49% | 0.52% | 100% | 100% |
| Class A2 | 0.02% | 0.34% | 0.61% | 100% | 101% |
| Class B | 0.05% | 0.61% | 0.81% | 100% | 100% |
| Class C | 0.16% | -0.34% | -0.45% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.28% | 0.37% | 100% | 101% |
| Class D | 0.18% | 0.61% | 0.66% | 100% | 102% |
| Class F | 0.23% | -0.16% | 0.17% | 99% | 104% |

References

[1] H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm), “Removal of LFNST for chroma components,” Document of Joint Video Experts Team, JVET-R0235.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).