| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April. 2020 | Document: JVET-R0366-v1 |
| --- | --- |

| Title: | Simplified disLut for GPM |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Qihong Ran, Ruipeng Qiu, Haixin Wang, Junyan Huo, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn rpqiu@stu.xidian.edu.cn hxwang_2@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |  |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |  |

_____________________________

# Abstract

In VVC Draft 8[1], for CUs coded by geometric partitioning mode (GPM), a look-up table disLut is used for weight value derivation and motion vector storing type derivation. This contribution proposes to simplify the table disLut, by changing each value to half. With the proposed simplification, 1 bit could be saved for each value in the table and the intermediate variables, and the derivation of weight value and motion vector storing type keep mathematically equivalent at the same time.

The coding performance is identical with that of VTM8.0. The experimental results are as below:

For RA configuration: 0.00 %, 0.00%, and 0.00%.

For LB configuration: 0.00 %, 0.00%, and 0.00%.

Introduction

When a CU is coded as GPM mode, two main variables in GPM are derived from the following equations: the variable wValue for the weighted sample prediction and sType for the motion vector storing.

1.The variable wValue specifies the weight of the prediction sample and is calculated by the following equations:

| displacementX=angleIdx | (1) |
| --- | --- |
| displacementY=( angleIdx + 8 ) % 32 | (2) |
| partFlip = ( angleIdx >= 13 && angleIdx <= 27 ) ? 0 : 1 xL = ( cIdx = = 0 ) ? x : x * SubWidthC yL = ( cIdx = = 0 ) ? y : y * SubHeightC | (3) (4) (5) |
| weightIdx = ( ( ( xL + offsetX ) << 1 ) + 1 ) * disLut[ displacementX ] + ( ( ( yL + offsetY ) << 1 ) + 1 ) ) * disLut[ displacementY ] weightIdxL = partFlip ? 32 + weightIdx : 32 - weightIdx | (6) (7) |
| wValue = Clip3( 0, 8, ( weightIdxL + 4 ) >> 3 ) | (8) |

The disLut is a look-up table as shown in Table 1.

2.The variable sType for each 4×4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY – 1 ,is calculated by the following equations:

| partIdx = ( angleIdx >= 13 && angleIdx <= 27 ) ? 0 : 1 | (9) |
| --- | --- |
| motionIdx = 4 * xSbIdx + offsetX ≪1+5* disLut displacementX + 4 * ySbIdx + offsetY≪1+5*disLut[displacementY] | (10) |
| sType = abs( motionIdx ) < 32 ? 2 : ( motionIdx ≤ 0 ? 1 - partIdx :partIdx) | (11) |

The disLut is a look-up table as shown in Table 1.

The variables numSbX and numSbY specify the number of 4×4 blocks in the current coding block in the horizontal and vertical directions, respectively.

Table 1. The look-up table disLut[]

| idx | 0 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| disLut[idx] | 8 | 8 | 8 | 4 | 4 | 2 | 0 | −2 | −4 | −4 | −8 | −8 |
| idx | 16 | 18 | 19 | 20 | 21 | 22 | 24 | 26 | 27 | 28 | 29 | 30 |
| disLut[idx] | −8 | −8 | −8 | −4 | −4 | −2 | 0 | 2 | 4 | 4 | 8 | 8 |

Note: the above equation (9) should be a typo, and the correct one should be:

partIdx =  angleIdx  ≥  13  &&  angleIdx  ≤  27 ? 1 : 0

Problem statement

The values in the table disLut are all even numbers, and the results of equation (6), (7) and (10) are all even numbers, too, which means that the last bits are always 0 for these values in the table and the intermediate variables. This causes unnecessary bit consumption.

Proposed method

In this contribution, it is proposed to simplify the array disLut without performance loss.

The derivations of wValue and sType in this contribution are mathematically equivalent with the current method after the simplification of disLut. And at the same time, the unnecessary bits used during the computation can be saved.

The proposed changes based on JVET-Q2001-vE are as below. The added texts are highlighted in yellow, deleted texts are described using texts with red strikethrough.

| weightIdxL = partFlip ? 3216 + weightIdx : 3216 - weightIdx | (1046) |
| --- | --- |
| wValue = Clip3( 0, 8, ( weightIdxL + 42 ) >> 32 ) | (1047) |

…

Table 37 - Specification of the geometric partitioning distance array disLut.

| idx | 0 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| disLut[idx] | 84 | 84 | 84 | 42 | 42 | 21 | 0 | −21 | −42 | −42 | −84 | −84 |
| idx | 16 | 18 | 19 | 20 | 21 | 22 | 24 | 26 | 27 | 28 | 29 | 30 |
| disLut[idx] | −84 | −84 | −84 | −42 | −42 | −21 | 0 | 21 | 42 | 42 | 84 | 84 |

…

| sType = abs( motionIdx ) < 3216 ? 2∶ ( motionIdx <= 0 ? ( 1 - partIdx ):partIdx) ) | (1059) |
| --- | --- |

Experimental results

The experiments were performed based on VTM8.0 software [2] under JVET common test condition [3]. Table 2 and Table 3 present a summary of the results.

Table 2. Results of proposed method under Random Access configuration

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |

Table 3. Results of proposed method under Low delay B configuration

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |

Conclusion

This contribution presents a method to simplify the array disLut and the associated intermediate calculations in GPM processing. With the proposed mathematically equivalent method, the unnecessary extra bits used during the calculation have been saved, which introduces no coding loss compared to VTM8.0. It is recommended to adopt these simplifications to VVC.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 8), Document JVET-Q2001, Version 15 , Brussels, Belgium, January 2020

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-8.0

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).