| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0371 |
| --- | --- |

| Title: | AHG2/9: On max num of subblock merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Jianle Chen, Wei-Jung Chien, Marta Karczewicz | Email: | {hanhuang, cjianle, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the contribution JVET-R0215, it’s reported that the current VVC draft allows an undesired scenario where five_minus_max_num_subblock_merge_cand may be set equal to 5 when sps_affine_enabled_flag is set equal to 1, which turn off the subblock temporal motion vector prediction (SbTMVP) even the values of the SbTMVP enabling flags are true. In this contribution, an alternative solution with minimum text change is proposed. In the proposed method, the range of five_minus_max_num_subblock_merge_cand is set to 0 to 5 – sps_sbtmvp_enabled_flag, inclusive.

# Problem Statement

In the contribution JVET-R0215[1], it’s reported that the current VVC draft allows an undesired scenario where five_minus_max_num_subblock_merge_cand may be set equal to 5 when sps_affine_enabled_flag is set equal to 1, which turn off the subblock temporal motion vector prediction (SbTMVP) even the values of the SbTMVP enabling flags are true. Two solutions are proposed in JVET-R0215. One of the solutions (alternative 2) requires some significant changes to the current VVC draft, while the other solution (alternative 1) disables the ability of turning affine merge off while the sps_affine_enabled_flag is on.

# Solution

In this contribution, an alternative method with minimum change is propsed to fix the issue reported in JVET-R0215. The text change of the propsed method is as following, the added text is highlighted.

five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 5 – sps_sbtmvp_enabled_flag, inclusive. When not present, the value of five_minus_max_num_subblock_merge_cand is inferred to be equal to 5.

# Conclusion

This contribution proposes to change the range of five_minus_max_num_subblock_merge_cand is set to 0 to 5 – sps_sbtmvp_enabled_flag, inclusive.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, “AHG9: Max num of subblock merge candidate signalling”, JVET-R0215, April. 2020.