| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: By teleconference, 15–24 April 2020 | Document: JVET-R0375-v2 |
| --- | --- |

| Title: | AHG2/AHG16: CCLM bug fix in luma reference down-sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Jianle Chen, Vadim Seregin, Marta Karczewicz. 5775 Morehouse Dr., San Diego, CA, 92121, USA. | Tel: Emal: | lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the current VVC text and VTM, the availability check of above-left neighbouring luma samples (avaiTL) in CCLM mode is derived using the availability of the left and above blocks. However, this check is not always correct in raster-scan slice case. This contribution proposes to not use the top-left neighbouring samples in CCLM mode. The experimental results shown that the impact of the proposed fix in terms of coding performance is negligible with an average (Y, U, V) Bd-rate reported as follows:

AI: 0.00%, 0.00%, 0.02% and (EncT, DecT) of (100%, 100%).

RA: -0.01%, -0.09%, -0.03% and (EncT, DecT) of (100%, 100%).

The proposed fix of the VVC bug tracker ticket #796 is also provided in the attached specification text, together with the proposed bug fix.

# Problem statements

In the current VVC text, the availability of above-left neighbouring luma samples (avaiTL) is derived as follows:

The variables availL, availT and availTL are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY − 1, yTbY ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availL.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availT.

- The variable availTL is derived as follows:

availTL  =  availL  &&  availT		(352)

If availTL is equal to true, the above-left neighbouring luma samples will be used to derive the down-sampled luma block and the CCLM parameters, shown as follows:

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

It is noted that when the picture is split into raster-scan slice, the avaiTL may be false when both avaiL and avaiT are equal to true. In this case, the luma down-sampling process uses unavailable top-left samples.

# Proposed methods

This contribution proposes to fix the above issue. Particularly, the top-left samples are not use in down-sampling for the left and above neighbouring luma samples in CCLM mode. Using this method, the special case for the top-left samples in the current VVC is removed. Hence, it makes the design more hardware friendly in pipe-line processing.

The following proposed text are based on JVET-Q2001-vE [1]. Deleted text is noted with red strikethrough, and newly added text is highlighted in green. It is note that the proposed fix of the VVC bug tracker ticket #796 is also provided in the attached specification text, together with the proposed bug fix.

8.4.5.2.13; Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode Inputs to this process are: - the intra prediction mode predModeIntra, - a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture, - a variable nTbW specifying the transform block width, - a variable nTbH specifying the transform block height, - a variable cIdx specifying the colour component of the current block, - chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1. Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1. The current luma location ( xTbY, yTbY ) is derived as follows: ( xTbY, yTbY ) = ( xTbC << ( SubWidthC − 1 ), yTbC << ( SubHeightC − 1 ) ) (351) The variables availL and, availT and availTL are derived as follows: - The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY − 1, yTbY ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availL. - The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availT. - The variable availTL is derived as follows: availTL = availL && availT (352) - The number of available top-right neighbouring chroma samples numTopRight is derived as follows: - The variable numTopRight is set equal to 0 and availTR is set equal to TRUE. - When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1: - The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY , yTbY ) the neighbouring luma location ( xTbY + x, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTR - When availLB is equal to TRUE, numLeftBelow is incremented by one. The number of available neighbouring chroma samples on the top and top-right numSampT and the number of available neighbouring chroma samples on the left and left-below numSampL are derived as follows: - If predModeIntra is equal to INTRA_LT_CCLM, the following applies: numSampT = availT ? nTbW : 0 (353) numSampL = availL ? nTbH : 0 (354) - Otherwise, the following applies: numSampT = ( availT && predModeIntra = = INTRA_T_CCLM ) ? ( nTbW +Min( numTopRight, nTbH ) ) : 0 (355) numSampL = ( availL && predModeIntra = = INTRA_L_CCLM ) ? ( nTbH + Min( numLeftBelow, nTbW ) ) : 0 (356) The variable bCTUboundary is derived as follows: bCTUboundary = ( yTbY & ( CtbSizeY − 1 ) = = 0 ) ? TRUE : FALSE. (357) The variable cntN and array pickPosN with N being replaced by L and T, are derived as follows: - The variable numIs4N is derived as follows: numIs4N = ( ( availT && availL && predModeIntra = = INTRA_LT_CCLM ) ? 0 : 1) (358) - The variable startPosN is set equal to numSampN >> ( 2 + numIs4N ). - The variable pickStepN is set equal to Max( 1, numSampN >> ( 1 + numIs4N ) ). - If availN is equal to TRUE and predModeIntra is equal to INTRA_LT_CCLM or INTRA_N_CCLM, the following assignments are made: - cntN is set equal to Min( numSampN, ( 1 + numIs4N ) << 1 ). - pickPosN[ pos ] is set equal to (startPosN + pos * pickStepN), with pos = 0.. cntN − 1. - Otherwise, cntN is set equal to 0. The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows: - If both numSampL and numSampT are equal to 0, the following applies: predSamples[ x ][ y ] = 1 << ( BitDepth − 1 ) (359) - Otherwise, the following ordered steps apply: - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * SubWidthC − 1, y= 0..nTbH * SubHeightC − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ). - The neighbouring luma samples pY[ x ][ y ] are derived as follows: - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..− 3, y = 0..SubHeightC * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ). - When availT is equal to FALSE, the neighbouring top luma samples pY[ x ][ y ] with x = −1..SubWidthC * numSampT − 1, y = −1..−2, are set equal to the luma samples pY[ x ][ 0 ]. - When availL is equal to FALSE, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = −1..SubHeightC * numSampL − 1, are set equal to the luma samples pY[ 0 ][ y ]. - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ). - When availTL is equal to TRUE, tThe neighbouring top-left luma samples pY[ x ][ y ] with x = −SubWidthC.. −1, −2, y = −1, −2, are set equal to the luma samples pY[ -x ][0] reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ). … 5. When numSampT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to p[ pickPosT[ idx − cntL ] ][ -1 ] with idx = cntL..cntL + cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = 0..cntL + cntT − 1 are specified as follows: - The neighbouring top-left luma samples pY[ x ][ y ] with x = −SubWidthC..−1, y = −1, −2, are set equal to the luma samples pY[ 0 ][ y ]. - The variable x is set equal to pickPosT[ idx − cntL ]. - If both SubWidthC and SubHeightC are equal to 1, the following applies: pSelDsY[ idx ] = pY[ x ][ − 1] (375) - Otherwise, the following applies: - If sps_chroma_vertical_collocated_flag is equal to 1, the following applies: - If bCTUboundary is equal to FALSE, the following applies: pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 − SubHeightC ] + F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ −SubHeightC ] + F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −SubHeightC] + (376) F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −SubHeightC] + F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 − SubHeightC ] + 4 ) >> 3 - Otherwise (bCTUboundary is equal to TRUE), the following applies: pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] + F2[ 1 ] * pY[ SubWidthC * x ][ −1 ] + (377) F2[ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> 2 - Otherwise (sps_chroma_vertical_collocated_flag is equal to 0), the following applies: - If bCTUboundary is equal to FALSE, the following applies: pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC x − 1 ][ −1 ] + F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ −2 ] + F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −1 ] + (378) F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ −2] + F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −1 ] + F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1 ][ −2 ] + 4 ) >> 3 - Otherwise (bCTUboundary is equal to TRUE), the following applies: pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] + F2[ 1 ] * pY[ SubWidthC * x ][ −1 ] + (379) F2[ 2 ] * pY[ SubWidthC * x + 1][ −1 ] + 2 ) >> 2 …

# Experimental results

The proposed bug fix is implemented on top of VTM8.0. The experimental results in AI and RA are summarised in Table 1. The experimental results shown that the impact of the proposed fix in terms of coding performance is negligible with an average (Y, U, V) Bd-rate reported as follows:

AI: 0.00%, 0.00%, 0.02% and (EncT, DecT) of (100%, 100%).

RA: -0.01%, -0.09%, -0.03% and (EncT, DecT) of (100%, 100%).

Table 1. Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% |  | -0.02% |  | 101% |  | 100% |
| Class A2 | 0.00% | 0.04% |  | 0.03% |  | 100% |  | 100% |
| Class B | 0.00% | 0.01% |  | 0.08% |  | 101% |  | 101% |
| Class C | 0.01% | -0.06% |  | 0.00% |  | 100% |  | 99% |
| Class E | 0.00% | 0.01% |  | -0.04% |  | 100% |  | 100% |
| Overall | 0.00% | 0.00% |  | 0.02% |  | 100% |  | 100% |
| Class D | 0.00% | -0.05% |  | -0.11% |  | 100% |  | 98% |
| Class F | 0.02% | 0.02% |  | 0.02% |  | 100% |  | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% |  | -0.08% |  | 0.00% |  | 102% |  |  | 101% |
| Class A2 | -0.02% |  | -0.06% |  | 0.14% |  | 100% |  |  | 100% |
| Class B | 0.00% | -0.11% | -0.06% | 100% | 101% |
| Class C | -0.02% | -0.11% | -0.14% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% |  | -0.09% |  | -0.03% |  | 100% |  |  | 100% |
| Class D | 0.02% | 0.07% | 0.05% | 100% | 99% |
| Class F | 0.04% |  | -0.01% |  | -0.02% |  | 99% |  |  | 98% |

# Conclusions

This contribution proposes a fix a bug in CCLM mode of the current VVC. It is recommended to adopt this fix and the proposed fix of the VVC bug tracker ticket #796 into the next version of VVC.

# Reference

- Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 8)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).