| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0376 |
| --- | --- |

| Title: | Versatile Video Coding for VPCC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dominik Mehlem Christian Rohlfing Institut für Nachrichtentechnik RWTH Aachen University | Tel: Email: | +49-241-80-27676 mehlem@ient.rwth-aachen.de rohlfing@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution presents recently conducted experiments on the video-based point cloud compression (VPCC) test model TMC2 by exchanging the currently used HM HEVC coder implementation with the VTM VVC coder implementation. Experiments show, that compression results with VVC are significantly higher than with HEVC.

# Introduction

The current VPCC design is video codec agnostic. However, for the current testing procedure in the VPCC test model implementation TMC2, one specific coder needs to be used in order to achieve comparable results for all contributions. Therefore, the HM HEVC coder implementation is used for the video-based coding part. It is interesting to see how using a different video coder influences the behaviour and output of the coding process. Especially for VVC it might also be useful to know whether the achieved gains over HEVC can as well be seen on the type of video content that is produced by TMC2.

# VPCC video streams

In VPCC the 3D point cloud data is projected onto a 2D surface in the form of patches of non-rectangular shape. Currently, there are three different video streams that are produced by the VPCC encoder: the occupancy map, the geometry stream and the attribute stream, respectively. The occupancy map is a binary representation of the exact patch boundaries within the 2D representation, the geometry stream is a greyscale video, that specifies the depth of the patches, and the attribute stream consists of the attribute information (e.g. color) of the patches.

# Test results

Tests conducted to achieve the anchor results are in conformance with the VPCC common test conditions and based on the TMC2 test software version 7.0 with 3D motion compensation disabled. All simulations are run on a 32-frame subset as well as the full sequences of the official C2 data set. The tests for the VVC-based encodings are carried out with the official reference software VTM version 7.0. The occupancy map, the geometry stream as well as the attribute stream are now encoded and decoded with this coder. Besides that, the tests are also in conformance with the common test conditions for VPCC without 3D motion compensation.

The results are shown in Tables 1 through 4, respectively. The D1 and D2 PSNR values are quality measures for the geometry whereas Luma, Cb and Cr denote PSNR values for the attributes. The shown percentage values describe the BD-rate gain achieved in comparison to the anchor (HEVC). Using the VVC coder performs significantly better for all sequences and there is no loss observed for any sequence. For the full sequences in the all-intra case, an overall BD-rate gain of roughly 22% is achieved for the geometry and around 20% (luma) for the attributes, respectively. For the random-access case an even higher BD-rate gain of 28% for the geometry and 25% (luma) for the attributes is achieved. Both cases are tested on the full sequences, as well as on a 32-frame subset of each sequence. Simulations on the subset show similar gains as on the full sequences. For the subset in the all-intra case, BD-rate gains of around 22% for geometry and 20% (luma) for attributes are observed. About 28% and 25% (luma) BD-rate gain are obtained in case of random-access for geometry and attribute compression, respectively.

Table 1 - Performance impact of using VVC instead of HEVC (AI, 32 frames)

| Class | D1 | D2 | Luma | Cb | Cr |
| --- | --- | --- | --- | --- | --- |
| loot | -17.1% | -14.9% | -17.8% | -24.0% | -17.3% |
| redandblack | -23.4% | -23.2% | -23.1% | -20.8% | -6.2% |
| soldier | -12.6% | -12.3% | -16.5% | -28.4% | -28.6% |
| queen | -27.9% | -27.8% | -19.3% | -24.7% | -18.4% |
| longdress | -20.1% | -21.1% | -21.3% | -19.4% | -11.8% |
| basketball | -30.6% | -28.5% | -21.1% | -20.1% | -28.5% |
| dancer | -28.4% | -26.4% | -21.2% | -21.0% | -22.8% |
| AVG | -22.9% | -22.0% | -20.0% | -22.6% | -19.1% |

Table 2 - Performance impact of using VVC instead of HEVC (AI, full)

| Class | D1 | D2 | Luma | Cb | Cr |
| --- | --- | --- | --- | --- | --- |
| loot | -16.5% | -14.5% | -17.8% | -24.4% | -18.0% |
| redandblack | -22.8% | -22.5% | -22.9% | -20.5% | -5.9% |
| soldier | -12.5% | -12.5% | -16.6% | -28.3% | -29.8% |
| queen | -26.8% | -26.9% | -19.6% | -27.8% | -22.4% |
| longdress | -21.1% | -21.1% | -21.5% | -21.6% | -12.8% |
| basketball | -30.2% | -28.0% | -21.1% | -19.7% | -27.5% |
| dancer | -28.5% | -26.3% | -20.9% | -20.0% | -22.9% |
| AVG | -22.6 | -21.7% | -20.1% | -23.2% | -19.9% |

Table 3 - Performance impact of using VVC instead of HEVC (RA, 32 frames)

| Class | D1 | D2 | Luma | Cb | Cr |
| --- | --- | --- | --- | --- | --- |
| loot | -25.0% | -24.7% | -24.1% | -37.6% | -29.3% |
| redandblack | -26.7% | -27.1% | -24.5% | -29.6% | -19.5% |
| soldier | -25.5% | -25.7% | -26.5% | -40.6% | -41.9% |
| queen | -33.1% | -33.4% | -23.7% | -35.4% | -31.2% |
| longdress | -25.8% | -26.6% | -25.7% | -28.9% | -23.4% |
| basketball | -33.4% | -32.5% | -25.1% | -29.5% | -31.8% |
| dancer | -30.9% | -29.7% | -24.1% | -21.9% | -20.9% |
| AVG | -28.6% | -28.5% | -24.8% | -31.9% | -28.3% |

Table 4 - Performance impact of using VVC instead of HEVC (RA, full)

| Class | D1 | D2 | Luma | Cb | Cr |
| --- | --- | --- | --- | --- | --- |
| loot | -24.4% | -23.8% | -33.4% | -33.4% | -27.5% |
| redandblack | -26.0% | -26.3% | -24.2% | -27.1% | -18.7% |
| soldier | -23.6% | -23.9% | -25.2% | -37.3% | -36.8% |
| queen | -32.1% | -32.3% | -23.8% | -37.6% | -32.9% |
| longdress | -27.4% | -27.6% | -26.7% | -31.9% | -25.6% |
| basketball | -32.9% | -31.7% | -24.9% | -26.4% | -29.0% |
| dancer | -30.8% | -29.6% | -24.1% | -21.9% | -20.3% |
| AVG | -28.2% | -27.9% | -24.7% | -30.8% | -27.3% |

Detailed results of the conducted experiments are attached to this contribution.

# Conclusion

Utilizing VVC instead of its predecessor HEVC has been tested to compress point cloud data with the VPCC standard. For traditional video, the used next generation video coder achieves substantial gain in comparison to HEVC. The same could be verified by using VVC in combination with VPCC. It is shown, that the VVC coder performs significantly better for all sequences of the C2 test set.

# Patent rights declaration(s)

RWTH Aachen University does not have any current or pending patent rights relating to the technology described in this contribution.