| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0379-v1 |
| --- | --- |

| Title: | Palette mode support in VVC main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Ru-Ling Liao, Mohammed Golam Sarwer Yung-Hsuan Chao, Wei-Jung Chien, Jianle Chen, Marta Karczewicz Patrice Onno, Christophe Gisquet, Guillaume Laroche Hong-Jheng Jhu, Yi-Wen Chen, Xiaoyu Xiu, Xianglin Wang | Tel: Email: |  |
| Source: | Alibaba Inc., Qualcomm Inc., Canon, Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution it is proposed to allow palette mode in the VVC main profile. It is our belief that the surge in use of online collaboration tools will last beyond the unprecedented time in which we live today, and will have long-lasting effect going into the future. It is asserted that VVC hardware decoders need to be future-proof to meet the needs of not only present but also future applications so as to avoid the situation where an application would have to choose between coding efficiency vs. hardware support (as narrated in “the DingTalk dilemma” recently encountered by one proponent). Therefore, providing high screen content coding efficiency is an essential requirement of the up-and-coming VVC standard. It is claimed that the VVC palette mode is a well-known coding tool and the experiments have shown that it provides attractive complexity vs. performance trade-off among the screen content coding tools available in VVC. Detailed study of its decoding flow revealed no particular implementation difficulty based on today’s hardware capabilities. It is further claimed that the VVC standard will be put at a disadvantage compared to other coding formats available on the market today if palette mode was not included in the VVC main profile.

# Proposal

In this contribution, it is proposed to allow the palette mode in the VVC main profile with reasons outlined as follows.

- The palette mode in VVC is based on a well-known concept

The palette mode in VVC was first adopted at the 15th JVET meeting, in a form that was substantially similar to the palette mode in the HEVC SCC extension, a concept that has been well documented in the form of the HEVC standard specification and academic papers. The palette mode in VVC was further refined at the subsequent JVET meetings, mainly to reduce complexity and to harmonize with the general VVC design, without significantly altering the concept of the design that is reportedly well known.

- Online collaboration is more important than ever, so is efficient coding of screen content

The importance of screen content coding has been well recognized for years, and that importance has especially skyrocketed recently in this extraordinary time. Around the globe people are adapting to collaborating online rather than in person for the sake of safety and health of the local and global communities. Workers are sharing ideas through online collaboration tools, students are taking online courses where their teachers teach knowledge by sharing their screens, conferences and meetings including the JVET, VCEG and MPEG meetings are held via video conferencing, where the one must-have form of video source is the many pages of presentations, excel sheets, and documents that need to be shared. In March 2020, several video conferencing applications grew significantly on mobile [1]. Daily mobile users of Zoom, Microsoft Teams, and Slack reached 4.8 million, 1.56 million and 0.5 million, respectively. The total number of daily users on Microsoft Teams (i.e. desktop and mobile users combined) reached 44 million, more than double four months ago. The video conferencing traffic is deemed to have higher priority than other type of video application; as a result, both Netflix and YouTube had to lower the peak video quality to avoid the internet breakdown [2]. Even as the pandemic gets under control (which we all hope would happen as soon as possible), the power of online collaboration in helping us build a robust modern society is here to stay. Thus, as an up-and-coming mainstream video codec standard, VVC must offer as much coding efficiency of the screen content as well-known coding technologies allow.

- Hardware decoders are made once but used repeatedly, and the VVC main profile must be future-proof

As a recent true story, during the outbreak, million of concurrent users in China, most of whom students, used DingTalk Live, a popular office collaboration tool offered by the Alibaba Group, to take online courses for their learning need. This surge in users brought about significant bandwidth burden and consequently significant need to reduce bandwidth usage. Screen content was identified as the highest priority content to be optimized, and detailed study on the efficiency of screen content coding tools was conducted in real product environment and found to be very effective. Unfortunately, because there is no existing hardware decoder support for screen content coding tools, it was ultimately decided to forego the bandwidth saving that these coding tools could have provided, because having to resort to software decoding was considered too burdensome.

The main profile is the most widely implemented and main profile hardware decoders are ubiquitous in devices. Hardware implementation is done once but can be used forever. It is asserted that the VVC main profile decoders should be future-proof and should enable high coding efficiency for all essential applications.

- Standards from ITU-T and ISO/IEC MPEG must be competitive against other coding formats in the marketplace

The ITU-T VCEG and ISO/IEC MPEG are not the only organizations developing video coding standards that aim to have a wide impact on the video industry at large. As the implementers get more choices of other coding formats, the proponents believe that the commercial success of the VVC standard relies heavily on its ability to achieve superior compression performance for all mainstream applications and all types of content. Further, one must consider the power of messaging. If another coding format developed a couple of years ago is able to claim that they support a wider set of screen content coding tools (including the palette mode) than VVC, the proponents believe that claim puts VVC at a disadvantage when it comes market messaging.

- The VVC palette mode is an efficient tool for screen content, and it is implementable

The palette mode is shown to provide remarkable coding efficiency on the screen content sequences in the same level as IBC and transform skip+BDPCM (as shown in Section 2), with manageable encoder runtime increase. Without inverse transform and intra/inter prediction, the tool also shows benefits in reducing decoder complexity.

Upon finalization, the VVC standard will become the state-of-the-art. Naturally its superior performance will come at the cost of some implementation complexity increase, as long as such increase is not unbearable for today’s hardware capability. As on today’s market there are already hardware implementations of a codec that supports palette mode, the proponents studied the difference between the palette mode in VVC and what has already been implemented, and concluded that there is no significant difference in implementation difficulty between these two (see further detail on this in Section 3).

In conclusion, it is the view of the proponents that excluding the palette mode from VVC main profile, which is a well-known coding tool with impressive performance for a very important category of video content and has no apparent severe implementation issues, does not serve the video industry in a constructive manner, and could even hinder VVC’s adoption by some implementors. Therefore, the proponents request to allow palette in VVC main profile.

It is noted that minimum specification changes are needed to allow palette in VVC main profile. The only specification change is to remove the sentence of palette mode constraint (i.e. “Referenced SPSs shall have sps_palette_enabled_flag equal to 0.”) in the A.3.1 Main10 profile definition section.

# Comparison of screen content coding tools in VVC

In this section, the BD-rate, encoder and decoder runtime of all the screen content tools for 4:2:0 content in VVC draft 8, including IBC, PLT, and transform skip + BDPCM, are evaluated on test sequences in class F and class TGM. The YUV BD-rate is calculated from PSNRYUV = (6*PSNRY + PSNRU + PSNRV) / 8. Since BDPCM cannot be enabled without transform skip mode, the two tools were evaluated jointly. For anchor, all the screen content tools are turned off in VTM8.0.

The bar chart in Figure 1 shows the BD-rate of each individual tool versus anchor under the AI, RA, and LDB configurations.

Figure 1:YUV BD-rate of screen content tools in VVC draft 8

The bar charts in Figure 2 are the encoder and decoder runtime of each individual tool versus the anchor under the AI, RA, and LDB configurations.

Figure 2: Encoder/Decoder runtime ratio of screen content tools in VVC draft 8

From the data shown, palette mode is able to achieve significant coding efficiency gain with relatively low encoder complexity. The tool also shows benefits in reducing decoder runtime since it doesn’t require inverse transform and intra/inter prediction. Given screen content coding is mostly used in applications such as video conferencing and online collaboration that requires low latency, it is asserted that the low encoder complexity of the palette mode is very desirable.

# Complexity study of the palette mode in VVC

Given AV1 supports the palette mode, and has hardware implementations available on the market, our study on VVC palette implementation complexity was focused on comparing the two palette modes in terms of their similarities and differences.

Figure 3. AV1 palette mode decoding flow

Figure 4. VVC palette mode decoding flow

Figure 3 and Figure 4 illustrate the decoding flow of the AV1 palette and the VVC palette, respectively. The two decoding flows include similar steps such as parsing the palette table, parsing the palette index map, construction of the palette table and the prediction process to fill the sample values of the CU (for example using a look-up table), as in the shaded boxes. There are two main differences in the figures:

- Unlike VVC palette where there is no residual, AV1 palette mode allows residuals to be sent, and inverse quantization and inverse transform will be performed on the residuals.

- In VVC palette, the predictor is updated after the current CU is decoded and stored for decoding of the next palette CU, whereas AV1 performs predictor construction before parsing based on neighboring CUs’ palette tables. Of course, it is noted that the flows in the figures are just conceptual, and different implementations may choose to implement the steps in alternative orders.

- Not shown in the figures are other minor differences, such as the max sizes of palette predictor and palette table, whether a palette entry includes luma and chroma sample values jointly or luma/chroma sample values separately, etc.

After working closely with our in-house hardware design team to study such differences, the proponents concluded that the VVC palette is not significantly more complicated than the AV1 palette and that VVC palette is implementable.

# Reference

[1] S. Mukherjee and A. Rana, “Zoom takes lead over Microsoft Teams as virus keeps Americans at home: Apptopia”, March 2020, https://www.reuters.com/article/us-health-coronavirus-zoom/zoom-takes-lead-over-microsoft-teams-as-virus-keeps-americans-at-home-apptopia-idUSKBN21I3AB

[2] H. Gold, “Netflix and YouTube are slowing down in Europe to keep the internet from breaking”, March 2020, https://www.cnn.com/2020/03/19/tech/netflix-internet-overload-eu/index.html

# Patent rights declaration(s)

Alibaba Inc., Qualcomm Inc., Canon, and Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).