| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0380 |
| --- | --- |

| Title: | Scaling list for adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

In this contribution, modifications of scaling matrix derivation are proposed. In VVC Draft 8, adaptive colour transform (ACT) for 4:4:4 can be controlled at the CU level flag (cu_act_enabled_flag). Although the energy distribution of transform coefficients in RGB and YCgCo domain would be different, in VVC draft 8, user defined scaling lists are always shared for G and Y, for B and Cg, for R and Co, which may cause undesirable behaviour of quantization process. To avoid the undesirable behaviour, this contribution proposes to introduce additional flag (scaling_matrix_act_disabled_flag) in APS to disable scaling matrix for the block coded with ACT.

Introduction

In VVC Draft 8, adaptive colour transform (ACT) is introduced for 4:4:4 RGB video signals. ACT can be controlled at the CU level flag (cu_act_enabled_flag). When ACT is applied to the current CU, colour space of the residual signals is converted from RGB (GBR) to YCgCo. In general, correlation between residual signals of three components in RGB domain is stronger than that in YCgCo domain, which means that the energy distribution of transform coefficients in RGB and YCgCo domain would be different.

However, in VVC draft 8, user defined scaling lists are always shared for G and Y, for B and Cg, for R and Co, which may cause undesirable behaviour of quantization process.

In similar to scaling_matrix_lfsnt_disabled_flag, new flag in APS for disabling scaling matrix for the block coded with ACT is introduced in this contribution.

Proposed solution

Proposed solution simply disables scaling matrix for block coded with ACT by signalling scaling_matrix_for_act_disabled_flag in scaling_list_data(). When scaling_matrix_for_act_disabled_ flag is set equal to 1, the intermediate scaling factor m[ x ][ y ] is set equal to 16 even if scaling_list_enbled flag is equal to 1.

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| if ( scaling_list_chroma_present_flag ) |  |
| scaling_matrix_for_act_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| … |  |

scaling_matrix_for_act_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with ACT. scaling_matrix_for_act_disabled_flag equal to 0 specifies that the scaling matrices may apply to the blocks coded with ACT.

In the scaling process for transform coefficients, one condition for deriving intermediate scaling factor m[ x ][ y ] is added as follows:

If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16.

- sps_scaling_list_enabled_flag is equal to 0.

- ph_scaling_list_present_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- scaling_matrix_for_act_disabled_flag is equal to 1 and cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).