| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0384 |
| --- | --- |

| Title: | Alternative film grain characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin | Tel: Email: | anorkin@netflix.com |
| Source: | Netflix |  |  |

_____________________________

# Abstract

This document proposes an alternative film grain characteristics SEI message. The syntax for the proposed SEI message is compatible with that of the mandatory film grain post-processing algorithm in the AV1 video codec. Adopting the proposed SEI would enable re-use of the post-processing modules between the codecs, thus improving the compression efficiency on movie and TV content without additional costs for the manufacturers. The document proposes the alternative film grain characteristics SEI message syntax and semantics and explains the film grain synthesis algorithm.

# Introduction

At the Brussels meeting, several documents proposed to adopt the film grain synthesis algorithm that is used in the AV1 video codec. In particular, JVET-Q0424 [3] proposed mandatory film grain synthesis support in VVC with signalling the film grain parameters in the APS and including film grain synthesis algorithm in the normative scope of VVC. In JVET-Q0614 [6], the film grain synthesis from AV1 was proposed to be included in VVC as an SEI message, along with including it in HEVC and H.264/AVC standards as well. Finally, JVET-Q0533 argued that the exact set of the parameters from the AV1 film grain synthesis algorithm should be used to enable re-use of the post-processing hardware modules between two codecs. During the discussion at Brussels meeting, it was agreed that the SEI approach is more preferable.

The SEI message proposed in this document is compatible with the film grain synthesis SEI in AV1. Using AV1 film grain post-processing would improve the coding efficiency on the movie and TV content without additional burden for many implementations since VVC and AV1 implementations could then re-use the same post-processing block. This document explains the AV1 film grain synthesis algorithm and suggests the syntax and semantics necessary to invoke the AV1 film grain synthesis algorithm.

As mentioned earlier, AV1 [1] specifies a film grain synthesis algorithm, which is applied as post-processing before outputting a reconstructed picture. This film grain synthesis algorithm is mandatory for implementation in AV1 decoding systems. The film grain is usually implemented as part of the post-processing pipeline, and implementations of the AV1 film grain synthesis algorithm should therefore be readily available at many manufacturers of the video decoding systems. Re-using the AV1 film grain synthesis algorithm in VVC would therefore not incur significant implementation costs.

This contribution proposes for the VVC video codec to use the same parameters and the reference film grain synthesis algorithm as the AV1 video codec to enable re-use of this post-processing block among the codecs implementations. An software patch with implementation of the example film grain signaling SEI in VVC is attached with the contribution.

# Motivation for film grain synthesis

The film grain is present in much of the movie and TV content. Often, the film grain is part of the creative intent and as such needs to be preserved when encoding the video.

Since film grain is a realization of a random process (with some degree of correlation in the spatial domain) it is difficult to compress it efficiently. Randomness makes prediction difficult, motion estimation less precise, and the prediction residual noisy. It often requires prohibitively high bitrates to reconstruct film grain with sufficient quality. Moreover, some encoding tools, such as the de-ringing filter may suppress the film grain.

The film grain SEI messages exist in H.264. These SEI messages, however, being optional in H.264/AVC, are not supported by most decoders. The encoders cannot rely on support of these SEI messages to provide more efficient compression since if the film grain synthesis algorithm is not implemented by the decoding system, the reconstructed video may have sub-par quality.

# Film grain modeling/synthesis framework

The framework for film grain estimation and synthesis is shown in the figure below.

Figure 1. Film grain estimation and synthesis.

As one could see from Figure 1, film grain is removed from the video before encoding, its parameters are estimated based on the noisy and de-noised versions of the video sequence, and sent along with the compressed video bitstream. After decoding, film grain is synthesized and added to the reconstructed video frames.

The details of the algorithm are as follows. Synthesized noise is added separately to Y, Cb, and Cr color components. The noise is estimated in smooth areas of the picture. Then, the noise standard deviation is modeled using piece-wise linear function for each of Y, Cb, and Cr components.

In luma, grain strength is modulated with a function of luminance as shown below:

Y’ = Y + f (Y) * GrainL,																									(1)

where Y’ is the resulting signal after grain application, Y is the reconstructed signal (without the film grain), and GrainL is the grain sample. f(Y) is the piece-wise linear function that scales with the noise intensity depending on the value of the luma component.

Chroma modeling for noise is done as follows. For chroma component (e.g. Cb), the noise is modulated using the following formula:

Cb’ = Cb + f( z ) * GrainCb, 							(2)

z =  Cb * mCb + Y  * mY,Cb + OffsetCb,  						 (3)

Parameters mCb , mY,Cb, and OffsetCb are used to model the chroma grain dependency on luma values.

Cr is calculated similarly. Here, the value of Y is the average value corresponding to values of Y at co-located positions as the chroma sample.

# Modeling of film grain pattern

The film grain pattern is modeled with an auto-regression process. This auto-regression model is used to synthesize the template with film grain at the decoder. Let G(x, y) be a zero-mean film grain sample at the current position. For lag L = 2, the grain sample G is calculated as follows (see Fig. 3):

G(x, y) = a0 G(x − 2, y − 2)  + a1 G(x − 1, y − 2) + a2 G(x, y − 2) +… + z,	(4)

where a0… an are AR-coefficients, G(x + k, y + m) are previous film grain sample values in the causal neighborhood, and z is the unit-variance Gaussian noise. The Gaussian variable z can be obtained from a predefined set stored at the decoder side.  In the case of chroma components, there is one more coefficient to capture correlation with luma grain.

In Fig. 2, the value of lag (ar_coeff_lag) is equal to 2. The value of lag is signaled in the bitstream and taking values from 0 to 3.

Figure 2. Sample of film grain (green square with cross) with the neighborhood used for noise modeling.

# Film grain reconstruction

Using AR coefficients and a Gaussian sequence, AR process is applied to a 64x64 template, called the film grain template. With a pseudo-random generator, the decoder generates offsets for a 32 x 32 block (+ 2 additional lines and rows when optional overlap is used) inside the 64x64 template. The 32x32 block of the film grain is scaled with intensities of reconstructed samples, added to the reconstructed component (luma or chroma), and the clipping is applied. The block sizes for chroma components are twice smaller for 4:2:0.

The pseudo-random generator is a shift-back linear-feedback shift register (LFSR) based on XOR of the length of 16 bits. The XOR-ed values are at positions 16, 14, 13 and 11, which corresponds to the feedback polynomial of x16 + x15 + x13 + x4 + 1. The offset is generated using 8 least significant bits on the register, 4 bits for y offset and 4 bits for x offset (generating offsets from 0 to 15, with luma offsets being in multiples of 2 of these numbers).

The pseudo-random generator is initialized in the beginning of each row of 32x32 blocks to allow parallel row processing. The generator is initialized based on a random_seed element.

Film grain application process can use optional overlap between the noise blocks, which helps to reduce possible block artifacts in case of larger grain sizes. In this case, luma block sizes are 34 x 34 with the two last rows and two last columns overlapping with the blocks to the left and to the right. The size of chroma blocks is 16x16 when no overlap is used and 17 x 17 with 1 sample overlap when 4:2:0 subsampling is used.

The scaling for a color component can use a look-up table (LUT) with the 256 8-bit values for 8-bit video and 1024 8-bit values for the 10-bit video. The table is filled based on the points of the piece-wise linear scaling function in the beginning of noise signaling for each frame.

Then the sample-wise operations for adding noise to the samples of 32 x 32 block are applied as follows:

Y´(x, y) = clip3( Y(x, y) + ((GL(x + sx, y + sy) * f (Y) + 2shift−1 ) >> shift ), a, b), 	(5)

where a and b define the range, x and y are coordinates inside the block, sx and sy are shift values inside the block, and parameter shift controls scaling of the film grain.

## Overlap between grain blocks

The operation used for overlapped samples over horizontal block boundaries in luma is:

Gcur(x, 0) = (27 * Gup(x, 32) + 17 * Gcur(x, 0) + 16) >> 5,	(6)

Gcur(x, 1) = (17 * Gup(x, 33) + 27 * Gcur(x, 1) + 16) >> 5,	(7)

where Gcur(x, 0) are samples of row 0 of the current block and Gup(x, 32) denotes samples of row 32 of the upper block.

The overlap operation between chroma blocks is applied as follows:

Gcur(x, 0) = (23* Gtop(x, 16) + 22 * Gcur(x, 0) + 16) >> 5		         (8)

More details on AV1 film grain synthesis algorithm can be found in [2].

# Alternative film grain characteristics SEI message syntax

The film grain synthesis algorithm uses the following parameters to synthesize film grain, as in [1]. The syntax and semantics for the proposed SEI message are provided below.

When film grain is enabled at the coded video sequence level, a flag apply_grain shall be signaled for each picture. If the apply_grain flag is equal to 1 for the picture, a grain_seed syntax element of u(16) need to be signaled or derived for the current picture.

| alternative_film_grain_characteristics_SEI( ) { | Descriptor |
| --- | --- |
| apply_film_grain_flag | u(1) |
| if (apply_film_grain_flag) { |  |
| film_grain_seed | u(16) |
| update_film_grain_model_flag | u(1) |
| if (update_film_grain_model_flag) { |  |
| num_y_points | u(4) |
| for ( i = 0; i < num_y_points; i++ ) { |  |
| point_y_value[ i ] | u(8) |
| point_y_scaling[ i ] | u(8) |
| } |  |
| chroma_scaling_from_luma | u(1) |
| if ( chromaGrainScalingSignaling ) { |  |
| num_cb_points | u(4) |
| for ( i = 0; i < num_cb_points; i++ ) { |  |
| point_cb_value[ i ] | u(8) |
| point_cb_scaling[ i ] | u(8) |
| } |  |
| num_cr_points | u(4) |
| for ( i = 0; i < num_cr_points; i++ ) { |  |
| point_cr_value[ i ] | u(8) |
| point_cr_scaling[ i ] | u(8) |
| } |  |
| } |  |
| grain_scaling_minus8 | u(2) |
| ar_coeff_lag | u(2) |
| if ( num_y_points ) { |  |
| for ( i = 0; i < numPosLuma; i++ ) |  |
| ar_coeffs_y_plus128[ i ] | u(8) |
| } |  |
| if ( chroma_scaling_from_luma ¦¦ num_cb_points ) { |  |
| for ( i = 0; i < numPosChroma; i++ ) |  |
| ar_coeffs_cb_plus128[ i ] | u(8) |
| } |  |
| if ( chroma_scaling_from_luma ¦¦ num_cr_points ) { |  |
| for ( i = 0; i < numPosChroma; i++ ) |  |
| ar_coeffs_cr_plus128[ i ] | u(8) |
| } |  |
| ar_coeff_shift_minus6 | u(2) |
| grain_scale_shift | u(2) |
| if ( num_cb_points ) { |  |
| cb_mult | u(8) |
| cb_luma_mult | u(8) |
| cb_offset | u(9) |
| } |  |
| if ( num_cr_points ) { |  |
| cr_mult | u(8) |
| cr_luma_mult | u(8) |
| cr_offset | u(9) |
| } |  |
| grain_blocks_overlap_flag | u(1) |
| clip_to_restricted_range | u(1) |
| } |  |
| } |  |
| } |  |

# Alternative film grain characteristics SEI message semantics

This SEI message provides the decoder with an alternative model for parameterized film grain synthesis.

NOTE 1 – An encoder may use the alternative film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques.

apply_grain equal to 1 specifies that the film grain shall be added to the reconstructed picture. apply_grain equal to 0 specifies that the film grain shall not be added.

film_grain_seed specifies the starting value for the pseudo-random number generator used in film grain synthesis.

update_film_grain_model_flag equal to 1 specifies that the film grain SEI contains a new set of parameters for the film grain synthesis algorithm. This signaled new set of parameters replaces the  previously signaled set of parameters and applies to the current picture and the pictures following the current picture in the display order. update_film_grain_model_flag equal to 0 specifies that the previously signalled set of the film grain parameters should be used for this picture. It is a requirement of the bitstream conformance that in a coded video sequence, the first picture in the display order with an associated alternative film grain SEI message has the update_film_grain_model_flag equal to 1.

num_y_points specifies the number of points for the piece-wise linear scaling function of the luma component. It is a requirement of bitstream conformance that num_y_points is less than or equal to 14.

point_y_value[ i ] specifies the luma value for the i-th point of the piecewise linear scaling function for the luma component.

If i is greater than 0, it is a requirement of bitstream conformance that point_y_value[ i ] is greater than point_y_value[ i − 1], which ensures x coordinates are signaled in the ascending order.

NOTE: – The values of point_y_value[ i ] are from 0 to 255. In 10-bit content, these values correspond to luma values divided by 4. In 12-bit content, these values correspond to luma values divided by 16.

point_y_scaling[ i ] specifies the scaling value for the i-th point of the piecewise linear scaling function for luma component.

chroma_scaling_from_luma specifies that the chroma scaling function is inferred from the luma scaling function.

If any of the following conditions is true, chromaGrainScalingSignaling is set to 0, otherwise chromaGrainScalingSignaling is set to 1:

- chroma_scaling_from_luma is equal to 1

- chroma_format_idc is equal to 0

- chroma_format_idc is equal to 1 and num_y_points is equal to 0

- chroma_format_idc is equal to 2 and num_y_points is equal to 0

num_cb_points specifies the number of points for the piece-wise linear scaling function of the Cb component. It is a requirement of bitstream conformance that num_cb_points is less than or equal to 10.

NOTE: – When chroma_scaling_from_luma is equal to 1, chroma scaling also needs to support up to 14 points since it is allowed for num_y_points to take values up to 14.

point_cb_value[ i ] specifies the x coordinate for the i-th point of the piece-wise linear scaling function for Cb component. The values are signaled on the scale of 0..255.

For values of i greater than 0, it is a requirement of bitstream conformance that point_cb_value[ i ] is greater than point_cb_value[ i − 1].

point_cb_scaling[ i ] specifies the function value for the i-th point of the piecewise linear scaling function for Cb component.

num_cr_points specifies the number of points for the piece-wise linear scaling function of the Cr component. It is a requirement of bitstream conformance that num_cr_points is less than or equal to 10.

When chroma_format_idc is equal to 1 and num_cb_points is equal to 0, it is a requirement of bitstream conformance that num_cr_points is equal to 0.

When chroma_format_idc is equal to 1 and num_cb_points is not equal to 0, it is a requirement of bitstream conformance that num_cr_points is not equal to 0.

NOTE: – This requirement ensures that for 4:2:0 chroma subsampling, film grain noise will be applied to both chroma components, or to neither. There is no restriction for 4:2:2 or 4:4:4 chroma subsampling.

point_cr_value[ i ] specifies the x coordinate for the i-th point of the piece-wise linear scaling function for Cr component. The values are signaled on the scale of 0..255.

For values of greater than 0, it is a requirement of bitstream conformance that point_cr_value[ i ] is greater than point_cr_value[ i − 1].

point_cr_scaling[ i ] specifies the scaling value for the i-th point of the piecewise linear scaling function for Cr component.

grain_scaling_minus8 plus 8 specifies the shift applied to the values of the grain and determines the range and quantization step of the standard deviation of film grain. The value of grain_scaling_minus8 shall be in the range of 0..3, inclusive.

ar_coeff_lag specifies the number of auto-regressive coefficients for luma and chroma. The variables numPosLuma and numPosChroma are derived as follows:

numPosLuma = 2 * ar_coeff_lag * ( ar_coeff_lag + 1 )

numPosChroma = numPosLuma + 1

ar_coeffs_y_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the luma component of the film grain.

ar_coeffs_cb_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the Cb component of the film grain.

ar_coeffs_cr_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the Cr component of the film grain.

ar_coeff_shift_minus6 specifies the range of auto-regressive coefficients.

NOTE: – Values of 0, 1, 2, and 3 correspond to the auto-regressive coefficients ranges of [−2, 2), [−1, 1), [−0.5, 0.5), and [−0.25, 0.25) respectively.

grain_scale_shift specifies how much the Gaussian random numbers should be scaled down during the grain synthesis process.

cb_mult specifies a multiplier for the Cb component used in derivation of the input index to the Cb component scaling function.

cb_luma_mult specifies a multiplier for the average luma component used in derivation of the input index to the Cb component scaling function.

cb_offset specifies an offset used in derivation of the input index to the Cb component scaling function.

cr_mult specifies a multiplier for the Cr component used in derivation of the input index to the Cr component scaling function.

cr_luma_mult specifies a multiplier for the average luma component used in derivation of the input index to the Cr component scaling function.

cr_offset specifies an offset used in derivation of the input index to the Cr component scaling function.

grain_blocks_overlap_flag equal to 1 specifies that the overlap between film grain blocks is applied. grain_blocks_overlap_flag equal to 0 specifies that the overlap between film grain blocks is not applied.

clip_to_restricted_range equal to 1 specifies that clipping to the restricted (studio) range is applied to the sample values after adding the film grain. clip_to_restricted_range equal to 0 specifies that clipping to the full range shall be applied to the sample values after adding the film grain.

# Alternative film grain synthesis process

TBD (in progress)

An example of the film grain synthesis algorithm in AV1 can be found in [1], Section 7.18.3. An example implementation of the proposed film grain synthesis algorithm can be found in [4].

# Conclusions

This document proposes an alternative film grain characteristics SEI message to be adopted in the first version of VVC. The proposed SEI is compatible with the AV1 film grain synthesis algorithm. It is argued that if the proposed SEI is adopted in VVC, products implementing both VVC and AV1 codecs can re-use the AV1 synthesis algorithm implementations, thus there is no implementation overhead while improving compression efficiency on movie and TV content. It is preferred that this SEI is mandatory to support in VVC since this guarantees all the decoders can reconstruct the content with film grain correctly.

# References

[1] “AV1 Bitstream & Decoding Process Specification”, Version 1.0.0 with Errata 1, Jan. 8, 2019, https://aomediacodec.github.io/av1-spec/av1-spec.pdf

[2] A. Norkin and N. Birkbeck, “Film Grain Synthesis for AV1 Video Codec”, in Proc. IEEE Data Compression Sonference (DCC), Snowbird, Utah, Mar. 2018.

[3] R. Sjöberg, D. Saffar, M. Pettersson, M. Damghanian, “Mandatory film grain”, JVET-Q0424, Brussels, Jan. 2020.

[4] https://aomedia.googlesource.com/aom

[5] A. Norkin, "Film Grain Synthesis Support", JVET-Q0533, Brussels, January 2020.

[6] A. M. Tourapis, K. Rapaka, D. Singer, K. Kolarov, "Film Grain Synthesis Support in VVC", JVET-Q0614, Brussels, January 2020.

# Patent rights declaration(s)

Netflix, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).