| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0384-v3 |
| --- | --- |

| Title: | AHG17: Alternative film grain characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin | Tel: Email: | anorkin@netflix.com |
| Source: | Netflix |  |  |

_____________________________

# Abstract

This document proposes an alternative film grain characteristics SEI message. The syntax for the proposed SEI message is compatible with that of the mandatory film grain post-processing algorithm in the AV1 video specification. Adopting the proposed SEI in VVC would enable re-use of the post-processing modules that may already be on the device, thus improving the compression efficiency on movie and TV content with little if any additional costs for the manufacturers. Applications such as transcoding between different codecs could also be better supported. The document proposes the alternative film grain characteristics SEI message syntax and semantics (including the film grain synthesis process) and explains the film grain synthesis algorithm. In version 2 of the document, specification of the film grain process and the persistence flag have been added, with some editorial text changes made in version 3.

# Introduction

At the Brussels meeting, several documents proposed to adopt the film grain synthesis algorithm that is used in the AV1 video specification. In particular, JVET-Q0424 [3] proposed mandatory film grain synthesis support in VVC with signalling the film grain parameters in the APS and including film grain synthesis algorithm in the normative scope of VVC. In JVET-Q0614 [6], the film grain synthesis from AV1 was proposed to be included in VVC as an SEI message, along with including it in HEVC and H.264/AVC standards as well. Finally, JVET-Q0533 argued that the exact set of the parameters from the AV1 film grain synthesis algorithm should be used to enable re-use of the post-processing hardware modules between two codecs. During the discussion at Brussels meeting, it was agreed that the SEI approach is more preferable.

The SEI message proposed in this document is compatible with the film grain synthesis SEI in AV1. Re-using AV1 film grain post-processing could improve the coding efficiency on the movie and TV content without additional burden for many implementations since VVC and AV1 implementations could then re-use the same post-processing block. This document explains the AV1 film grain synthesis algorithm and suggests the syntax and semantics necessary to invoke the AV1 film grain synthesis algorithm.

As mentioned earlier, AV1 [1] specifies a film grain synthesis algorithm, which is applied as post-processing before outputting a reconstructed picture. This film grain synthesis algorithm is mandatory for implementation in AV1 decoding systems. The film grain is usually implemented as part of the post-processing pipeline, and implementations of the AV1 film grain synthesis algorithm should therefore be readily available at many manufacturers of the video decoding systems. Re-using the AV1 film grain synthesis algorithm in VVC would therefore not incur significant implementation costs.

This contribution proposes for the VVC video codec to use the same parameters and the reference film grain synthesis algorithm as the AV1 video codec to enable re-use of this post-processing block among the codecs implementations. A software patch with implementation of the example film grain signaling SEI in VTM8.0 is attached with the contribution.

# Motivation for film grain synthesis

The film grain is present in much of the movie and TV content. Often, the film grain is part of the creative intent and as such needs to be preserved when encoding the video.

Since film grain is a realization of a random process (with some degree of correlation in the spatial domain) it is difficult to compress it efficiently. Randomness makes prediction difficult, motion estimation less precise, and the prediction residual noisy. It often requires prohibitively high bitrates to reconstruct film grain with sufficient quality. Moreover, some encoding tools, such as the de-ringing filter may suppress the film grain.

The film grain SEI messages exist in H.264 and HEVC. These SEI messages, however, being optional in H.264/AVC and HEVC, are not supported by most decoders. The encoders cannot rely on support of these SEI messages to provide more efficient compression since if the film grain synthesis algorithm is not implemented by the decoding system, the reconstructed video may have sub-par quality.

# Film grain modeling/synthesis framework

The framework for film grain estimation and synthesis is shown in the figure below.

Figure 1. Film grain estimation and synthesis.

As one could see from Figure 1, film grain is removed from the video before encoding, its parameters are estimated based on the noisy and de-noised versions of the video sequence, and sent along with the compressed video bitstream. After decoding, film grain is synthesized and added to the reconstructed video frames.

The details of the algorithm are as follows. Synthesized noise is added separately to Y, Cb, and Cr color components. The noise is estimated in smooth areas of the picture. Then, the noise standard deviation is modeled using piece-wise linear function for each of Y, Cb, and Cr components.

In luma, grain strength is modulated with a function of luminance as shown below:

Y’ = Y + f (Y) * GrainL,																									(1)

where Y’ is the resulting signal after grain application, Y is the reconstructed signal (without the film grain), and GrainL is the grain sample. f(Y) is the piece-wise linear function that scales with the noise intensity depending on the value of the luma component.

Chroma modeling for noise is done as follows. For chroma component (e.g. Cb), the noise is modulated using the following formula:

Cb’ = Cb + f( z ) * GrainCb, 							(2)

z =  Cb * mCb + Y  * mY,Cb + OffsetCb,  						 (3)

Parameters mCb , mY,Cb, and OffsetCb are used to model the chroma grain dependency on luma values.

Cr is calculated similarly. Here, the value of Y is the average value corresponding to values of Y at co-located positions as the chroma sample.

# Modeling of film grain pattern

The film grain pattern is modeled with an auto-regression process. This auto-regression model is used to synthesize the template with film grain at the decoder. Let G(x, y) be a zero-mean film grain sample at the current position. For lag L = 2, the grain sample G is calculated as follows (see Fig. 3):

G(x, y) = a0 G(x − 2, y − 2)  + a1 G(x − 1, y − 2) + a2 G(x, y − 2) +… + z,	(4)

where a0… an are AR-coefficients, G(x + k, y + m) are previous film grain sample values in the causal neighborhood, and z is the unit-variance Gaussian noise. The Gaussian variable z can be obtained from a predefined set stored at the decoder side.  In the case of chroma components, there is one more coefficient to capture correlation with luma grain.

In Fig. 2, the value of lag (ar_coeff_lag) is equal to 2. The value of lag is signaled in the bitstream and taking values from 0 to 3.

Figure 2. Sample of film grain (green square with cross) with the neighborhood used for noise modeling.

# Film grain reconstruction

Using AR coefficients and a Gaussian sequence, AR process is applied to a 64x64 template, called the film grain template. With a pseudo-random generator, the decoder generates offsets for a 32 x 32 block (+ 2 additional lines and rows when optional overlap is used) inside the 64x64 template. The 32x32 block of the film grain is scaled with intensities of reconstructed samples, added to the reconstructed component (luma or chroma), and the clipping is applied. The block sizes for chroma components are twice smaller for 4:2:0.

The pseudo-random generator is a shift-back linear-feedback shift register (LFSR) based on XOR of the length of 16 bits. The XOR-ed values are at positions 16, 14, 13 and 11, which corresponds to the feedback polynomial of x16 + x15 + x13 + x4 + 1. The offset is generated using 8 least significant bits on the register, 4 bits for y offset and 4 bits for x offset (generating offsets from 0 to 15, with luma offsets being in multiples of 2 of these numbers).

The pseudo-random generator is initialized in the beginning of each row of 32x32 blocks to allow parallel row processing. The generator is initialized based on a random_seed element.

Film grain application process can use optional overlap between the noise blocks, which helps to reduce possible block artifacts in case of larger grain sizes. In this case, luma block sizes are 34 x 34 with the two last rows and two last columns overlapping with the blocks to the left and to the right. The size of chroma blocks is 16x16 when no overlap is used and 17 x 17 with 1 sample overlap when 4:2:0 subsampling is used.

The scaling for a color component can use a look-up table (LUT) with the 256 8-bit values for 8-bit video and 1024 8-bit values for the 10-bit video. The table is filled based on the points of the piece-wise linear scaling function in the beginning of noise signaling for each frame.

Then the sample-wise operations for adding noise to the samples of 32 x 32 block are applied as follows:

Y´(x, y) = clip3( Y(x, y) + ((GL(x + sx, y + sy) * f (Y) + 2shift−1 ) >> shift ), a, b), 	(5)

where a and b define the range, x and y are coordinates inside the block, sx and sy are shift values inside the block, and parameter shift controls scaling of the film grain.

## Overlap between grain blocks

The operation used for overlapped samples over horizontal block boundaries in luma is:

Gcur(x, 0) = (27 * Gup(x, 32) + 17 * Gcur(x, 0) + 16) >> 5,	(6)

Gcur(x, 1) = (17 * Gup(x, 33) + 27 * Gcur(x, 1) + 16) >> 5,	(7)

where Gcur(x, 0) are samples of row 0 of the current block and Gup(x, 32) denotes samples of row 32 of the upper block.

The overlap operation between chroma blocks is applied as follows:

Gcur(x, 0) = (23* Gtop(x, 16) + 22 * Gcur(x, 0) + 16) >> 5		         (8)

More details on AV1 film grain synthesis algorithm can be found in [2].

# Alternative film grain characteristics SEI message syntax

The alternative film grain characteristics SEI message uses the following parameters, that can be used to synthesize film grain, as described in the semantics section of the proposal. The syntax for the proposed SEI message is provided below.

| alternative_film_grain_characteristics_SEI( ) { | Descriptor |
| --- | --- |
| apply_film_grain_flag | u(1) |
| if (apply_film_grain_flag) { |  |
| film_grain_seed | u(16) |
| update_film_grain_model_flag | u(1) |
| if (update_film_grain_model_flag) { |  |
| num_y_points | u(4) |
| for ( i = 0; i < num_y_points; i++ ) { |  |
| point_y_value[ i ] | u(8) |
| point_y_scaling[ i ] | u(8) |
| } |  |
| chroma_scaling_from_luma | u(1) |
| if ( chromaGrainScalingSignaling ) { |  |
| num_cb_points | u(4) |
| for ( i = 0; i < num_cb_points; i++ ) { |  |
| point_cb_value[ i ] | u(8) |
| point_cb_scaling[ i ] | u(8) |
| } |  |
| num_cr_points | u(4) |
| for ( i = 0; i < num_cr_points; i++ ) { |  |
| point_cr_value[ i ] | u(8) |
| point_cr_scaling[ i ] | u(8) |
| } |  |
| } |  |
| grain_scaling_minus8 | u(2) |
| ar_coeff_lag | u(2) |
| if ( num_y_points ) { |  |
| for ( i = 0; i < numPosLuma; i++ ) |  |
| ar_coeffs_y_plus128[ i ] | u(8) |
| } |  |
| if ( chroma_scaling_from_luma ¦¦ num_cb_points ) { |  |
| for ( i = 0; i < numPosChroma; i++ ) |  |
| ar_coeffs_cb_plus128[ i ] | u(8) |
| } |  |
| if ( chroma_scaling_from_luma ¦¦ num_cr_points ) { |  |
| for ( i = 0; i < numPosChroma; i++ ) |  |
| ar_coeffs_cr_plus128[ i ] | u(8) |
| } |  |
| ar_coeff_shift_minus6 | u(2) |
| grain_scale_shift | u(2) |
| if ( num_cb_points ) { |  |
| cb_mult | u(8) |
| cb_luma_mult | u(8) |
| cb_offset | u(9) |
| } |  |
| if ( num_cr_points ) { |  |
| cr_mult | u(8) |
| cr_luma_mult | u(8) |
| cr_offset | u(9) |
| } |  |
| grain_blocks_overlap_flag | u(1) |
| clip_to_restricted_range | u(1) |
| film_grain_model_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

# Alternative film grain characteristics SEI message semantics

This SEI message provides the decoder with an alternative model for parameterized film grain synthesis. The film grain process is applied to output pictures and not to reference pictures.

NOTE 1 – An encoder may use the alternative film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques.

apply_film_grain_flag equal to 1 specifies that the film grain should be added to the reconstructed picture. apply_film_grain_flag equal to 0 specifies that the film grain should not be added. If apply_film_grain_flag is equal to 1, the film grain synthesis process is applied to the resonstructed picture samples as follows.

First, the arrays scalingLut[cIdx] are initialized (one for each component) and the grain templates grainTemplate[cIdx] are generated. It follows by generating the film grain planes grPlanes[cIdx], which are added to the reconstructed output picture recPicture as shown below.

NOTE 1 – Although the specification defines the generation process of grPlanes[cIdx] before adding grain samples to the reconstructed picture samples, the actual implementation does not need to do that. Instead, grPlanes[cIdx] blocks can be generated just in time, before adding them to the reconstructed picture.

First, film grain is added to the chroma components recPictureCb and recPictureCr of the reconstructed picture if indicated by num_cb_points, num_cr_points and chroma_scaling_from_luma syntax elements.

for ( y = 0; y < picHeightInLumaSamples >> subsY; y++ ) {
	for ( x = 0; x <  picWidthInLumaSamples >> subsX; x++ ) {
		lumaX = x << subsX
		lumaY = y << subsY
		lumaNextX = min( lumaX + 1, picWidthInLumaSamples − 1 )
		if ( subsX )
			avLuma = (recPictureL[ lumaY ][ lumaX ] + recPictureL[ lumaY ][ lumaNextX ]) >> 1
		else
			avLuma = recPictureL[ lumaY ][ lumaX ]
		if ( num_cb_points > 0 || chroma_scaling_from_luma ) {
			if ( chroma_scaling_from_luma ) {
				scInp = avLuma
			} else {
				scInp = Clip1(( (avLuma * ( cb_luma_mult − 128 ) + recPictureCb[ y ][ x ] *
						( cb_mult − 128 ) ) >> 6 ) + ( (cb_offset − 256 ) << (BitDepth − 8)))
			}
			grSamp = (scaleLut(1, grPlanes[ 1 ][ y ][ x ]) * grSamp + 2scalingShift−1) >>scalingShift
			recPictureCb[ y ][ x ] = Clip3(maxVal[1], maxVal[1], recPictureCb[ y ][ x ] + grSamp)
		}
		if ( num_cr_points > 0 || chroma_scaling_from_luma) {
			if ( chroma_scaling_from_luma ) {
				scInp = avLuma
			} else {
				scInp=Clip1(((avLuma * (cr_luma_mult − 128) +recPictureCr[y][x]*
				 			(cr_mult−128 ))>>6) + ( (cr_offset − 256 ) << (BitDepth − 8)))
			}
			grSamp = (scaleLut ( 2, scInp) * grPlanes[ 2 ][ y ][ x ] + 2scalingShift−1) >>scalingShift
			recPictureCr[ y ][ x ] = Clip3(maxVal[2], maxVal[2], recPictureCr [ y ][ x ]+ grSamp)
		}
	}
}
Then, if num_y_points is equal to 1, the film grain is added to the luma component recPictureL of the reconstructed picture.

for ( y = 0; y < h ; y++ ) {
	for ( x = 0; x < w ; x++ ) {
		grSamp = ( scaleLut( 0, recPictureL[y][x]) * grPlanes[ 0 ][ y ][ x ] + 2scalingShift−1) >> scalingShift
		if ( num_y_points > 0 ) 
			recPictureL[ y ][ x ] = Clip3(maxVal[0], maxVal[0], recPictureL[y][x]+ grSamp)
	}
}

film_grain_seed specifies the starting value for the pseudo-random number generator used in film grain synthesis. In particular, film_grain_seed contains the value used to initialize a 16-bit randomGenerator variable. randomGenerator is used to generate pseudo random numbers that are used in the film grain synthesis process with the function getRandNum.

getRandNum() has an input value nBits and the output value randResult as follows:

rVal = randRegister
	newBit = ((rVal >> 0) ^ (rVal >> 1) ^ (rVal >> 3) ^ (rVal >> 12)) & 1
	rVal = (rVal >> 1) | (newBit << 15)
	randResult = (rVal >> (16 − nBits)) & ((1 << nBits) − 1)
	randRegister = rVal

gaussSample is a look-up table that contains 2048 Gaussian distributed samples with zero mean and standard deviation of 512. The sample values are in the range [-2048, 2047].

update_film_grain_model_flag equal to 1 specifies that the alternative film grain SEI message contains a new set of parameters for the film grain synthesis algorithm. update_film_grain_model_flag equal to 0 specifies that the previously signalled film grain model for the current layer should be used for the picture associated with the SEI message. It is a requirement of the bitstream conformance that if apply_film_grain_flag is equal to 1 and update_film_grain_model_flag is equal to 0, there should exist a persisting film grain model previously signaled for this layer in the current coded video sequence.

num_y_points specifies the number of points for the piece-wise linear scaling function of the luma component. It is a requirement of bitstream conformance that num_y_points is less than or equal to 14.

point_y_value[ i ] specifies the luma value for the i-th point of the piecewise linear scaling function for the luma component.

If i is greater than 0, it is a requirement of bitstream conformance that point_y_value[ i ] is greater than point_y_value[ i − 1], which ensures x coordinates are signaled in the ascending order.

NOTE: – The values of point_y_value[ i ] are from 0 to 255. In 10-bit content, these values correspond to luma values divided by 4. In 12-bit content, these values correspond to luma values divided by 16.

point_y_scaling[ i ] specifies the scaling value for the i-th point of the piecewise linear scaling function for luma component.

For a color plane cIdx, the following initialization procedure is envoked to initialize the scalingLut[cIdx].

If cIdx is equal to 0, numPoints is equal to num_y_points, pointValue[ i ] is equal to point_y_value [ i ] and pointScaling[ i ] is equal to point_y_scaling [ i ].

If cIdx is equal to 1, numPoints is equal to num_cb_points, pointValue[ i ] is equal to point_cb_value [ i ] and pointScaling[ i ] is equal to point_cb_scaling [ i ].

If cIdx is equal to 2, numPoints is equal to num_cr_points, pointValue[ i ] is equal to point_cr_value [ i ] and pointScaling[ i ] is equal to point_cr_scaling [ i ].

The scalingLut[cIdx] are initialized as follows.

if ( numPoints == 0 ) {
	for ( x = 0; x < 256; x++ ) 
		scalingLut[cIdx][ x ] = 0
} else {
	for ( x = 0; x < pointValue[ 0 ]; x++ ) 
		scalingLut[cIdx][ x ] = pointScaling[ 0 ]
	for ( i = 0; i < numPoints − 1; i++ ) {
		dY = pointScaling[ i + 1 ] − pointScaling[ i ]
		dX = pointValue[ i + 1 ] − pointValue[ i ]
		stepY = dY * ( ( 65536 + (dX >> 1) ) / dX )
		for ( x = 0; x < dX; x++ )
			scalingLut[cIdx][ pointValue[ i ] + x ] = pointScaling[ i ] + ( ( x * stepY + 32768 ) >> 16 )
	}
	for ( x = pointValue[ numPoints − 1 ]; x < 256; x++ ) 
		scalingLut[cIdx][ x ] = pointScaling[numPoints − 1]
}

To obtain values of the scaling function, the following procedure is invoked with the color plane cIdx and the input value pointVal as inputs. The output is the value of the scaling function pointScal.

scaleLut(cIdx, pointVal) {
	x = pointVal >> (BitDepth – 8)
	rem = pointVal % x
	if ( BitDepth == 8 || x == 255)
		pointScal =  ScalingLut[cIdx][ x ]
	else
		pointScal = scalingLut[cIdx][ x ] + 
		 		(scalingLut[cIdx][ x ] − scalingLut[cIdx][ x + 1 ]) * rem + 2BitDepth–8-1) >> (BitDepth – 8)
}

chroma_scaling_from_luma specifies that the chroma scaling function is inferred from the luma scaling function.

If any of the following conditions is true, chromaGrainScalingSignaling is set to 0, otherwise chromaGrainScalingSignaling is set to 1:

- chroma_scaling_from_luma is equal to 1

- chroma_format_idc is equal to 0

- chroma_format_idc is equal to 1 and num_y_points is equal to 0

- chroma_format_idc is equal to 2 and num_y_points is equal to 0

To obtain tables scalingLut[1] and scalingLut[2], when chroma_scaling_from_luma is equal to 1, their values are copied from scalingLut[0].

num_cb_points specifies the number of points for the piece-wise linear scaling function of the Cb component. It is a requirement of bitstream conformance that num_cb_points is less than or equal to 10.

NOTE: – When chroma_scaling_from_luma is equal to 1, chroma scaling also needs to support up to 14 points since it is allowed for num_y_points to take values up to 14.

point_cb_value[ i ] specifies the x coordinate for the i-th point of the piece-wise linear scaling function for Cb component. The values are signaled on the scale of 0..255.

For values of i greater than 0, it is a requirement of bitstream conformance that point_cb_value[ i ] is greater than point_cb_value[ i − 1].

point_cb_scaling[ i ] specifies the function value for the i-th point of the piecewise linear scaling function for Cb component.

num_cr_points specifies the number of points for the piece-wise linear scaling function of the Cr component. It is a requirement of bitstream conformance that num_cr_points is less than or equal to 10.

When chroma_format_idc is equal to 1 and num_cb_points is equal to 0, it is a requirement of bitstream conformance that num_cr_points is equal to 0.

When chroma_format_idc is equal to 1 and num_cb_points is not equal to 0, it is a requirement of bitstream conformance that num_cr_points is not equal to 0.

NOTE: – This requirement ensures that for 4:2:0 chroma subsampling, film grain noise will be applied to both chroma components, or to neither. There is no restriction for 4:2:2 or 4:4:4 chroma subsampling.

point_cr_value[ i ] specifies the x coordinate for the i-th point of the piece-wise linear scaling function for Cr component. The values are signaled on the scale of 0..255.

For values of greater than 0, it is a requirement of bitstream conformance that point_cr_value[ i ] is greater than point_cr_value[ i − 1].

point_cr_scaling[ i ] specifies the scaling value for the i-th point of the piecewise linear scaling function for Cr component.

grain_scaling_minus8 plus 8 specifies the shift applied to the values of the grain and determines the range and quantization step of the standard deviation of film grain. The value of grain_scaling_minus8 shall be in the range of 0..3, inclusive. scalingShift is set to grain_scaling_minus_8 + 8

ar_coeff_lag specifies the number of auto-regressive coefficients for luma and chroma. The variables numPosLuma and numPosChroma are derived as follows:

numPosLuma = 2 * ar_coeff_lag * ( ar_coeff_lag + 1 )

numPosChroma = numPosLuma + 1

ar_coeffs_y_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the luma component of the film grain. aRCoeffsY [ i ] = ar_coeffs_y_plus128[ i ] −128

ar_coeffs_cb_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the Cb component of the film grain. aRCoeffsCb [ i ] = ar_coeffs_cb_plus128[ i ] −128

ar_coeffs_cr_plus128[ i ] minus 128 specifies the value of the i-th auto-regressive coefficient used for modeling the Cr component of the film grain. aRCoeffsCr [ i ] = ar_coeffs_cr_plus128[ i ] −128

ar_coeff_shift_minus6 specifies the range of auto-regressive coefficients. aRCoeffShift is set equal to ar_coeff_shift_minus_6 + 6

NOTE: – Values of 0, 1, 2, and 3 correspond to the auto-regressive coefficients ranges of [−2, 2), [−1, 1), [−0.5, 0.5), and [−0.25, 0.25) respectively.

grain_scale_shift specifies how much the Gaussian random numbers should be scaled down during the grain synthesis process. scaleShift is set to 12 - BitDepth + grain_scale_shift.

The film grain templates are generated as follows. The variable cIdx is the color component index.

If cIdx is equal to 0 (luma component), nCurrSw = 82, nCurrSh = 73, subsY=0, subsX = 0

If cIdx is equal to 1 or cIdx is equal to 2

- if chroma_format_idc is equal to 1,  nCurrSw = 44, nCurrSh = 38, subsY=1, subsX = 1

- if chroma_format_idc is equal to 2,  nCurrSw = 44, nCurrSh = 73, subsY=1, subsX = 0

- if chroma_format_idc is equal to 2,  nCurrSw = 82, nCurrSh = 73, subsY=0, subsX = 0

If cIdx is equal to 1, randRegister is set to film_grain_seed ^ 0xb524

If cIdx is equal to 2, randRegister is set to film_grain_seed ^ 0x49d8

for ( y = 0; y < nCurrSh; y++ ) {
	for ( x = 0; x < nCurrSw; x++ ) {
		if ( num_y_points > 0 ) 
			grainTemplate[cIdx][y][x]=(gaussSample[getRandNum(11)]+2scaleShift-1)>> scaleShift

The luma grain template is generated by the process below.

for ( y = 3; y < nCurrSh; y++ ) {
	for ( x = 3; x < nCurrSw − 3; x++ ) {
		pos = 0, s = 0
		for ( idxY = −ar_coeff_lag; idxY <= 0; idxY ++ ) 
			for (idxX = −ar_coeff_lag; idxX <= ar_coeff_lag && (idxY!=0 || idxX != 0); idxX ++ ) 
				s += grainTemplate[cIdx][ [ y + idxY][ x + idxX] * aRCoeffsY [ pos++ ]
		grainTemplate [0][y][x] = Clip3(−2BitDepth-1, 2BitDepth-1−1, grainTemplate[0][y][x] + (( s + 2aRCoeffShift-1) >> aRCoeffShift )))
	}
}

After generating the luma template, the chroma grain templates are generated by the process below. cIdx is set equal to 1 and 2. aRCoeffsCh [ i ] is equal to aRCoeffsCb [ i ] when cIdx is equal to 1 and to aRCoeffsCr [ i ] when cIdx is equal to 2.

for ( y = 3; y < nCurrSh; y++ ) {
	for ( x = 3; x < nCurrSw − 3; x++ ) {
		s = 0,  pos = 0
		for (idxY = −ar_coeff_lag; idxY <= 0; idxY ++ ) {
			for (idxX = −ar_coeff_lag; idxX <= ar_coeff_lag && (idxY!=0 || idxX != 0); idxX ++ ) {
				s = s + grainTemplate[cIdx][ y + idxY][ x + idxX] * aRCoeffsCh [ pos++ ]
			}
		}
		if ( num_y_points > 0 ) {
			lumaAv = 0
			lumaX = ( (x − 3) << subsX ) + 3
			lumaY = ( (y − 3) << subsY ) + 3
			for ( i = 0; i <= subsY; i++ )
				for ( j = 0; j <= subsX; j++ )
					lumaAv += grainTemplate [0] [ lumaY + i ][ lumaX + j ]
			s = s + (( lumaAv + 2 subsX + subsY -1) >> (subsX + subsY) ) * aRCoeffsCh [ pos++ ]
		}
		grainTemplate[cIdx][y][x]  = Clip3((−2BitDepth-1, 2BitDepth-1−1, grainTemplate [cIdx][y][x]  +  (( s + 2aRCoeffShift-1) >> aRCoeffShift )))	
	}
}

cb_mult specifies a multiplier for the Cb component used in derivation of the input index to the Cb component scaling function.

cb_luma_mult specifies a multiplier for the average luma component used in derivation of the input index to the Cb component scaling function.

cb_offset specifies an offset used in derivation of the input index to the Cb component scaling function.

cr_mult specifies a multiplier for the Cr component used in derivation of the input index to the Cr component scaling function.

cr_luma_mult specifies a multiplier for the average luma component used in derivation of the input index to the Cr component scaling function.

cr_offset specifies an offset used in derivation of the input index to the Cr component scaling function.

grain_blocks_overlap_flag equal to 1 specifies that the blending between film grain blocks is applied. grain_blocks_overlap_flag equal to 0 specifies that the blending between film grain blocks is not applied.

First, the blending is performed over the vertical grain block boundaries and then over the horizontal grain block boundaries. For each of the color component, if subsX is equal to 0, a two-sample wide blending is applied to the vertical grain block boundaries on the 66 samples height. If subsX is equal to 1, a one-sample blending is applied to the horizontal grain boundaries of the 33 sample height.

The result of the blending is grPlanes[cIdx] array that contains the grain samples before scaling. The blending process is as follows.

rowNum = 0
for ( y = 0; y < (picHeightInLumaSamples + 1) >> 1; y += 16 ) {
	randRegister = film_grain_seed
	randRegister ^= ((rowNum * 37 + 178) & 255) << 8
	randRegister ^= ((rowNum * 173 + 105) & 255)
	for ( x = 0; x < (picWidthInLumaSamples + 1) >> 1; x += 16 ) {
		rand = getRandNum( 8 )
		offsetX = rand >> 4
		offsetY = rand & 15
		for ( cIdx = 0; cIdx < 3; cIdx ++ ) {
			cSubsX = (cIdx > 0) ? subsX : 0
			cSubsY = (cIdx > 0) ? subsY : 0
			planeOffsetX = cSubsX? 6 + offsetX : 9 + offsetX * 2
			planeOffsetY = cSubsY? 6 + offsetY : 9 + offsetY * 2
			for ( i = 0; i < 34 >> cSubsY; i++ ) 
				for ( j = 0; j < 34 >> cSubsX; j++ ) {
					grSCur[cIdx][ i ][ j ] = grainTemplate [cIdx] [ lumaY + i ][ lumaX + j ]
					if (grain_blocks_overlap_flag)
						blendingOperation(grSCur[cIdx])
					grPlanes[cIdx][y+i][x+j] = grSCur[cIdx][ i ][ j ]
				}
		}
	}
	rowNum++
}

If grain_blocks_overlap_flag is equal to 0, a blending operation blendingOperation(grSCur[cIdx]) is applied to the block boundaries as follows.

blendingOperation(grSCur[cIdx]):

If subsX is equal to 0, a two-sample blending operation is applied to the vertical grain block boundaries. If subsX is equal to 1, a one-sample blending operation is applied to the vertical grain block boundaries.

If subsY is equal to 0, a two-sample blending operation is applied to the horizontal grain boundaries. If subsY is equal to 1, a one-sample blending operation is applied to the horizontal grain boundaries.

The blending operation for samples of the first two columns of grain samples in the block is as follows, where grSCur is the array of grain samples corresponding to the current block, and grSLeft is the array of grain samples corresponding to the grain block on the left. yCurr and xCurr are the coordinates of the first (0, 0) sample of the current grain block. i = 0,..,33.

grSCur[cIdx][yCurr+i][0] = (27*grSLeft[cIdx][yCurr+i][32] + 17*grSCur[cIdx][yCurr+i][0]+16) >> 5

grSCur[cIdx][yCurr+i][1] = (17*grSLeft [cIdx][yCurr+i][33] + 27*grSCur[cIdx][yCurr+i][1]+16) >> 5

The blending operation for the first of rows of a grain block below the horizontal grain block boundary is perfomed as follows, i = 0,..,33. grSAbove is the array of grain samples corresponding to the grain block above the current block.

grSCur[cIdx][0][xCurr+i] = (27*grSAbove[cIdx][32][xCurr+i] + 17*grSCur[cIdx][0][xCurr+i]+16) >> 5

grSCur[cIdx][1][xCurr+i] = (17*grSAbove[cIdx][33][xCurr+i] + 27*grSCur[cIdx][1][xCurr+i]+16) >> 5

The one-sample blending operation is applied to the first row of the grain block for horizontal boundaries and the first colums on the block for vertical boundaries. The blending operation is as follows, where grSCur is the array of grain samples corresponding to the current block, and grSPrev is the array of grain samples corresponding to the previous grain block. yCurr and xCurr are the coordinates of the first (0, 0) sample of the current grain block. In the following, i = 0,...16.

grSCur[cIdx][yCurr+i][0] = (23*grSLeft[cIdx][yCurr+i][16] + 22*grSCur[cIdx][yCurr+i][0]+16) >> 5

The one-sample blending operation for the horizontal grain block boundary is as follows, i = 0,...16.

grSCur[cIdx][0][yCurr+i] = (23*grSAbove[cIdx][16][yCurr+i] + 22*grSCur[cIdx][0][yCurr+i] +16) >> 5

clip_to_restricted_range equal to 1 specifies that clipping to the restricted (studio) range is applied to the sample values after adding the film grain. clip_to_restricted_range equal to 0 specifies that clipping to the full range shall be applied to the sample values after adding the film grain.

After adding film grain to the reconstructed picture, a clipping operation is applied to the output samples. The minimum and maximum values minVal[cIdx] and maxVal[cIdx] for the clipping operation are derived as follows. If clip_to_restricted_range is equal to zero, minVal[cIdx] is equal to 0, and maxVal[cIdx] is equal to 2BitDepth – 1.

If clip_to_restricted_range is equal to zero, minVal[cIdx] is equal to 16*2BitDepth–8 and maxVal[0] is equal to 235*2BitDepth–8.

- If matrix_coeffs is equal to 0, maxVal[1] and maxVal[2] are equal to 235*2BitDepth–8

- otherwise, maxVal[1] and maxVal[2] are equal to 240*2BitDepth–8

film_grain_model_persistence_flag specifies the persistence of the alternative film grain characteristics SEI message model for the current layer.

film_grain_model_persistence_flag equal to 0 specifies that the model specified in alternative film grain characteristics SEI message applies to the current decoded picture only.

Let picA be the current picture. film_grain_model_persistence_flag equal to 1 specifies that the alternative film grain characteristics SEI message model persists for the current layer in output order until any of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture picB in the current layer in an access unit containing an alternative film grain characteristics SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

# Alternative film grain synthesis implementation

The new film grain synthesis algorithm is described in Section 7, in the semantics section.

An example implementation of the proposed alternative film grain characteristics SEI message has been done in VTM8.0 (it does not include film grain estimation in the encoder).  A VTM software patch that supports an example of signalling of the proposed Alternative film grain characteristics SEI algorithm has been attached with this contribution.

# Conclusions

This document proposes an alternative film grain characteristics SEI message to be adopted in the first version of VVC. The proposed SEI is compatible with the AV1 film grain synthesis algorithm. It is argued that if the proposed SEI is adopted in VVC, products implementing both VVC and AV1 specifications can re-use the AV1 synthesis algorithm implementations, thus there is no implementation overhead while improving compression efficiency on movie and TV content. It is preferred that this SEI is mandatory for implementation in VVC since this guarantees all the decoders can reconstruct the content with film grain correctly. Optional support of the alternative film grain characteristics SEI message would also provide advantage for movie and TV content on platforms that implement both VVC and AV1 specifications.

# References

[1] “AV1 Bitstream & Decoding Process Specification”, Version 1.0.0 with Errata 1, Jan. 8, 2019, https://aomediacodec.github.io/av1-spec/av1-spec.pdf

[2] A. Norkin and N. Birkbeck, “Film Grain Synthesis for AV1 Video Codec”, in Proc. IEEE Data Compression Sonference (DCC), Snowbird, Utah, Mar. 2018.

[3] R. Sjöberg, D. Saffar, M. Pettersson, M. Damghanian, “Mandatory film grain”, JVET-Q0424, Brussels, Jan. 2020.

[4] https://aomedia.googlesource.com/aom

[5] A. Norkin, "Film Grain Synthesis Support", JVET-Q0533, Brussels, January 2020.

[6] A. M. Tourapis, K. Rapaka, D. Singer, K. Kolarov, "Film Grain Synthesis Support in VVC", JVET-Q0614, Brussels, January 2020.

# Patent rights declaration(s)

Netflix, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).