| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0388-v1 |
| --- | --- |

| Title: | AHG9: Cleanups on deblocking signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Ye-Kui Wang2, Li Zhang2 Kai Zhang2, Jizheng Xu2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhipin.deng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

After the categroy 1 AHG pre-meeting review of the contributions on deblocking signalling control, the proponents further studied the topic taking into account the AHG recommendation. It is assertred that 1) there are still some issues with the semantics that should be fixed, 2) there is a bug in the inference of the slice_deblocking_filter_override_flag, and 3) the feature of indicating an overriding operation in a picture header or a slice header and then immedaitely send the next bit in the same header to indicate a change of mind is a bit too weird and should be removed.

To address the above issues, this contribution proposes the following changes:

- Update the semantics of the deblocking signalling control syntax elements to avoid incorrect and incomplete statements in the semantics.

- Infer slice_deblocking_filter_override_flag to be equal to 0 (instead of to be equal to ph_deblocking_filter_override_flag) when not present.

- Skip the signalling of ph/slice_deblocking_filter_disabled_flag when pps_deblocking_filter_disabled_flag and ph/slice_deblocking_filter_override_flag are both equal to 1 and infer the value of ph/slice_deblocking_filter_disabled_flag to be equal to 0 under this condition.

# Introduction

After the categroy 1 AHG pre-meeting review of the contributions on deblocking signalling control, we further studied the topic taking into account the AHG recommendation on the adoption of the semantics changes proposed in JVET-R0159, and obverved a few issues that we would like to discuss.

The updated pps_deblocking_filter_disabled_flag semantics in JVET-R0159-v2, excluding the SPS flag part, is as follows:

Option 1:

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS of in which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are is not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS of in which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are is not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

Option 2:

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0 in which slice_deblocking_filter_disabled_flag is not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0 in which slice_deblocking_filter_disabled_flag is not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

Note that the changes are already a good improvement. However, there are still some issues.

Regarding the part of pps_deblocking_filter_disabled_flag equal to 1 of the above option 1 text, the follow two issues were observed:

- The condition ("slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are not present ") does not capture all cases when deblocking is disabled for slices under the value 1 of this flag, as shown in case #8 in R0072 (the three PPS flags and the two PH/SH flag are all equal to 1). In this case, ph/slice_deblocking_filter_disabled_flag is present and the value is 1, thus deblocking is still be disabled.

- When pps_deblocking_filter_disabled_flag is equal to 1, deblocking may also be enabled for slices referring to this PPS under certain condition, e.g., in case #9 in R0072 (the three PPS flags are all equal to 1, ph/sh_deblocking_filter_override_flag is equal to 1, and ph/slice_deblocking_filter_disabled_flag is equal to 0).

Thus, to make the semantics of pps_deblocking_filter_disabled_flag equal to 1 to be complete, it needs to add a condition on the 1st asepct above and add the mentioning the 2nd asepct above.

However, we think that it'd probably be sufficient to just specify the semantics of this value 1 part as follows:

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is disabled for slices referring to the PPS unless indicated otherwise at the picture or slice level.

Regarding the part of pps_deblocking_filter_disabled_flag equal to 0 of the above option 1 text, the follow two issues were observed:

- The condition does not capture all cases when deblocking is enabled for slices under the value 0 of pps_deblocking_filter_disabled_flag, as shown in case #5 in R0072 (the other two PPS flags are both equal to 1, ph/slice_deblocking_filter_override_flag is equal to 1, and ph/slice _deblocking_filter_disabled_flag is present and equal to 0).

- When pps_deblocking_filter_disabled_flag 0, deblocking may also be disabled for slices referring to this PPS under certain condition, e.g., in case #6 in R0072 (the other two PPS flags are both equal to 1, ph/slice_deblocking_filter_override_flag is equal to 1, and ph/slice _deblocking_filter_disabled_flag is present and equal to 1).

Thus, similarly as for the value 1 semantics, to make the semantics of pps_deblocking_filter_disabled_flag equal to 0 to be complete, it needs to add a correction on the 1st asepct above and add the mentioning of the 2nd asepct above.

Similarly as for the value 1 semantics, we think that it'd probably be sufficient to just specify the sematnics of this value 0 part as follows:

pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of deblocking filter is enabled for slices referring to the PPS unless indicated otherwise at the picture or slice level.

We think that the semantics of pps_deblocking_filter_disabled_flag of the above option 2 text is better than the option 1 text in the sense what is said regarding the first aspect is complete, but still missing the mentioning of the second aspect (under a few conditions when pps_deblocking_filter_disabled_flag is equal to 1, deblocking may also be enabled for slices referring to this PPS, but those conditions are not mentioned in the semantics, and similarly for the part of the semantics for pps_deblocking_filter_disabled_flag equal to 0). Again, we think that the semantics should be either made complete or make it in a simple fashion as suggested above.

In addition, according to the current text, when the SH syntax element slice_deblocking_filter_override_flag is not present, it is inferred to be equal to ph_deblocking_filter_override_flag. When the PPS flags deblocking_filter_control_present_flag, deblocking_filter_override_enabled_flag, and dbf_info_in_ph_flag are all equal to 1, the intention is to allow the deblocking overriding in the PH, other than in the SH. However, when the PH override flag is equal to 1 and the deblocking overriding is applied in the PH, and meanwhile, the SH override flag is not signalled but inferred to be equal to the PH override flag, additional deblocking parameters will also be also signalled in the SH, which is conflicting with the design intention.

Furthermore, when the PPS syntax element deblocking_filter_override_enabled_flag and pps_deblocking_filter_disabled_flag are equal to 1, and meanwhile the PH/SH syntax element ph/slice_deblocking_filter_override_flag is present and equal to 1, the ph/slice_deblocking_filter_disabled_flag is still allowed to be explicitly signalled to be equal to 1. What this combiantion of the values of the flags does is that deblocking is said at the PPS level to be disabled and allowed to be overriden at the picture or slice, then it is indicated at the PH/SH level that it is going to be overriden, and then a bit is immediately signalled in the same header (PH/SH) to finally determine that it is actually not overriden and deblocking remains disabled at the picture/slice level. This is asserted to have a double undesriable effects: not only a bit is spent unncessarily, it is spent just for causing some confusion (that the two continuous bits tell different stories).

We therefore propose to further improve the semantics of deblocking controlling syntax elements, and remove the weired behavior of allowing indicating overriding and then immedaitely send the next bit in the same PH or SH to indicate a change of mind.

# Proposal

The following changes are proposed to address the above issues:

- Update the semantics of the deblocking signalling control syntax elements to avoid incorrect and incomplete statements in the semantics.

- Infer slice_deblocking_filter_override_flag to be equal to 0 (instead of to be equal to ph_deblocking_filter_override_flag) when not present.

- Skip the signalling of ph/slice_deblocking_filter_disabled_flag when pps_deblocking_filter_disabled_flag and ph/slice_deblocking_filter_override_flag are both equal to 1, and infer the value of ph/slice_deblocking_filter_disabled_flag to be equal to 0 under this condition.

# Proposed spec text changes

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in turquoise, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts.

PPS semantics (based on the text in JVET-R0159-v2, excluding the SPS flag):

...

deblocking_filter_control_present_flag equal to 1 specifies the presence of deblocking filter control syntax elements in the PPS. deblocking_filter_control_present_flag equal to 0 specifies the absence of deblocking filter control syntax elements in the PPS and that the deblocking filter is applied for all slices referring to the PPS, using 0-valued deblocking parameter offsets for all β and tC.

deblocking_filter_override_enabled_flag equal to 1 specifies the presence of ph_deblocking_filter_override_flag in the PHs referring to the PPS or slice_deblocking_filter_override_flag in the slice headers referring to the PPS. deblocking_filter_override_enabled_flag equal to 0 specifies the absence of ph_deblocking_filter_override_flag in PHs referring to the PPS or slice_deblocking_filter_override_flag in slice headers referring to the PPS. When not present, the value of deblocking_filter_override_enabled_flag is inferred to be equal to 0.

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS of which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS of which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

Or

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is disabled for slices referring to the PPS unless indicated otherwise at the picture or slice level. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of deblocking filter is enabled for slices referring to the PPS unless indicated otherwise at the picture or slice level.

...

The syntax structure picture_header_structure( ) is changed as follows:

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

…

ph_deblocking_filter_override_flag equal to 1 specifies that deblocking parameters are present in the PH. ph_deblocking_filter_override_flag equal to 0 specifies that deblocking parameters are not present in the PH. When not present, the value of ph_deblocking_filter_override_flag is inferred to be equal to 0.

ph_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the slices associated with the PH in which slice_deblocking_filter_disabled_flag is not present [Ed.: When ph_deblocking_filter_disabled_flag is present, slice_deblocking_filter_disabled_flag won't be present in the SH of any slice of the picture, therefore the removal.]. ph_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the slices associated with the PH in which slice_deblocking_filter_disabled_flag is not present [Ed.: When ph_deblocking_filter_disabled_flag is present, slice_deblocking_filter_disabled_flag won't be present in the SH of any slice of the picture, therefore the removal.].

When ph_deblocking_filter_disabled_flag is not present, it is inferred as follows:

- If pps_deblocking_filter_disabled_flag and ph_deblocking_filter_override_flag are both equal to 1, the value of ph_deblocking_filter_disabled_flag is inferred to be equal to 0.

- Otherwise, the value of ph_deblocking_filter_disabled_flag is inferred to be equal to pps_deblocking_filter_disabled_flag.

...

And the syntax structure slice_header ( ) is changed as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |

…

slice_deblocking_filter_override_flag equal to 1 specifies that deblocking parameters are present in the slice header. slice_deblocking_filter_override_flag equal to 0 specifies that deblocking parameters are not present in the slice header. When not present, the value of slice_deblocking_filter_override_flag is inferred to be equal to ph_deblocking_filter_override_flag0.

slice_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the current slice. slice_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the current slice.

When slice_deblocking_filter_disabled_flag is not present, it is inferred as follows:

- If pps_deblocking_filter_disabled_flag and slice_deblocking_filter_override_flag are both equal to 1, the value of slice_deblocking_filter_disabled_flag is inferred to be equal to 0.

- Otherwise, the value of slice_deblocking_filter_disabled_flag is inferred to be equal to ph_deblocking_filter_disabled_flag.

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).