| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0390 |
| --- | --- |

| Title: | [AHG16] VVC multi-thread decoder and performance analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Srinivas Gudumasu Tangi Poirier Fabrice Urban Franck Hiron Philippe de Lagrange | Tel: Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution introduces a software based VVC decoder implementation using parallel processing. The design explores both task and data parallelization to distribute the decoding modules on general-purpose multiprocessor platform. The implementation is based on VTM8.0 without compromising on the coding efficiency or memory bandwidth; an average 75% decoding time reduction is reported for Class A and Class B sequences with random access CTC test conditions on a 10-core processor.

# Introduction

The current VVC draft supports tiles and wave-front parallel processing (WPP) to make the coding process parallelizable. Tile may cause coding efficiency degradation when each tile is independently coded, while WPP divides slices into CTU rows and processes rows in parallel and preserves coding dependencies. An early implementation of VVC software decoder was reported in JVET-Q0211, this contribution introduces the software based VVC decoder using parallel processing on general-purpose multiprocessor platform on VTM 8.0.

Figure 1 illustrates a general block diagram of VTM8.0 decoder. VVC decoding process can be divided into the following tasks.

- CABAC decoding

- Motion Vector Component Derivation

- Inter CU & CIIP inter reconstruction

- Intra CU & CIIP intra reconstruction

- Inverse reshaping

- In-loop filtering

  - De-blocking Filter

  - Sample Adaptive Offset

  - Adaptive Loop Filter

Figure 1 VVC Decoding architecture

The following 2 parallelization approaches are applied to VVC decoder.

- Load balanced task parallelization

- CTU based data parallelization.

VTM decoder pipeline consists of CABAC decoding and slice decoding tasks. CABAC decoding and slice decoding are running simultaneously on different slices or pictures due to data dependency. CABAC decoding is usually faster than slice decoding, a single CABAC thread for a frame is generally adequate. In case of very low QP or more complicated content, single thread CABAC decoding may be lagging behind the slice decoding. Multiple threads may be allocated to CABAC decoding to balance the workload between CABAC decoding and slice decoding and minimize the effect of synchronization overhead.

Approach II is applied on slice decoding tasks, including Inter and CIIP inter reconstruction, Intra and CIIP intra reconstruction, motion vector derivation, re-shaping, de-blocking filter, Sample Adaptive Offset (SAO) and Adaptive Loop Filtering (ALF). Approach II provides fine granularity parallelism on top of existing tile-based parallelization.

# Proposed Method

## Overall Parallelization Architecture

For current VVC decoding pipeline design, one thread is reserved for CABAC decoding and the rest threads are allocated for the slice decoding. The CABAC decoded output is stored in a first in first out (FIFO) queue. This thread continues CABAC decoding of the sub-sequent NAL units. Slice decoding executes on the CABAC decoded output as shown in Figure 2.

Figure 2 Overview parallelization architecture

## Parallelization of slice decoding tasks

### Thread scheduling

In this section, we illustrate the thread scheduling mechanism in Figure 3. A thread pool and a FIFO activity queue are created for processing the slice decoding tasks activities. A thread pool is created based on the number of available CPU cores in the multi-core system. For example, a thread pool of 20 threads is created for Intel Core™ i9 processor with 10 cores. Each slice decoding task shown in Figure 2 is further divided into multiple sub-tasks. Each sub-task may consist of a CTU or a group of CTUs or one row of CTUs. A scheduling thread pushes the sub-tasks into activity queue. Any free thread from the thread pool fetches the first available sub-task from the FIFO activity queue and executes the task. As a result, the sub-tasks of a decoding task can be executed in parallel without synchronization overhead. All the sub-tasks of one decoding task shall be completed before kicking off the sub-tasks of the next decoding task.

Figure 3 Thread scheduling mechanism for slice/picture reconstruction tasks

### Motion Vector Component Derivation (MVCD)

Multiple threads are assigned to motion vector component derivation task, as in Figure 4, each thread handles one CTU row with the data dependency on its left, top and/or top right neighbor CU motion vector, similar as WPP parallelization.

The scheduling thread pushes the MVCD sub-task to the activity queue in the order of increasing CTU row number. An available thread from the thread pool will fetch the first available CTU row from the activity queue and derive the motion vector of each CTUs in sequential order, the thread is released when the entire CTU row is processed. The synchronization overhead is required between threads to guarantee the data dependency. The maximum of threads can be used for MVCD parallel processing is equal to the number of CTU rows within a slice. An average motion vector component derivation time reduction is 70.99%.

Figure 4 Thread scheduling mechanism for MVCD task

### Inter CU reconstruction

After motion vector component derivation, inter CU reconstruction along with the CIIP CU inter reconstruction is processed. The inter CU reconstruction process is divided into multiple sub-tasks. Each inter reconstruction sub-task may consist of one or a group of CTUs.

The scheduling thread pushes the inter CU reconstruction process of a CTU or a group of CTUs to the activity queue in the increasing CTU number order. An available free thread will fetch the first available CTU or CTU group from the activity queue and performs the inter CU reconstruction process. All threads perform the inter CU reconstruction process of the CTU groups simultaneously as shown in Figure 5.

Inter CUs residual reconstruction for chroma samples is separated from inter CU reconstruction process as chroma residual calculation is based on the nearest VPDU reconstructed luma samples when LMCS flag is enabled. The nearest VPDU reconstructed luma samples may not be available as intra CUs reconstruction process is not yet completed for those VPDU samples. So, inter CUs residual reconstruction for chroma samples is postponed and executed with other intra CUs reconstruction process which occurs at a later stage.

The inter prediction of the CIIP mode CU can be combined with other inter CU prediction process. CIIP intra CU reconstruction process is combined with other intra CUs reconstruction process. An average inter CU reconstruction processing time reduction is 89.09%.

Figure 5 Thread scheduling mechanism for Inter CU reconstruction task

### Intra CU reconstruction

A traditional WPP parallelization scheme is applied for the intra CU prediction. In this approach, each CTU row is treated as a sub-task, and each sub-task is processed by a free thread as in Figure 6. Multiple CTU rows are executed in parallel with synchronization overhead. Each CTU cannot be proceeded until its left and top-right neighboring CTUs is available. CIIP CU intra prediction is also processed along with normal intra CUs prediction. An average intra CU reconstruction time reduction is 71.13%.

Figure 6 Thread scheduling mechanism for Intra reconstruction task

Note: any CTU containing only CIIP and inter CUs can be processed with different scheduling. This is applicable when the CTU’s neighboring (left, top left, top and top right) CTUs are coded in inter mode only. The scheduling thread creates a sub-task for this CTU’s CIIP intra reconstruction task and pushes it into the activity queue. A free thread can then execute the CIIP intra reconstruction independently. As a result, all the above mentioned CTUs CIIP intra reconstruction sub-tasks can be executed in parallel as shown in Figure 7. This approach has not been implemented but can be useful to improve the decoding performance for CIIP CUs.

Figure 7 Thread scheduling mechanism for CIIP CU intra reconstruction task

### Inverse Re-shaper

After the CU reconstruction task, inverse reshaping for the entire picture is executed in parallel using multiple threads. The inverse reshaping process for the entire picture is equally distributed among the available threads as illustrated in Figure 8. For example, when 10 free slice decoding threads are available to process 2160 rows of a 4K picture (3840x2160), each slice decoding thread can process inverse reshaping task for 216 rows. An average inverse re-shaping time reduction is 86.91%.

Figure 8 Thread scheduling mechanism for Inverse Re-shaping task

### De-blocking filter

The de-blocking module in the VVC loop-filtering task is applied immediately after the inverse re-shaper. In VVC, the vertical de-blocking filter is applied on the horizontal de-blocking filter output. Since there is no data dependency between CTUs for either the horizontal or vertical filtering tasks, all CTUs for the whole frame can be processed in parallel. The first available thread fetches the next available CTU or CTU group (e.g. 4 CTUs) from the activity queue for horizontal de-blocking filtering. Vertical de-blocking filtering checks the completion of the horizontal filtering for the current and CTU below as shown in Figure 9. An average decoding time reduction of 90.35% is observed on the de-blocking filtering task.

Figure 9 Thread Scheduling mechanism for De-blocking filter task

### Sample Adaptive Offset & Adaptive Loop-Filter

All CTUs of the entire frame are processed in parallel for ALF and SAO. ALF process is divided into multiple sub-task with each task containing one or a group of CTUs. The scheduling thread pushes one or a group of CTUs sub-task to the activity queue. Free thread fetches first available sub-task and executes ALF process on those CTU(s). The same thread scheduling mechanism is applied for SAO processing. An average ALF and SAO time reduction is 73.51% and 73.13% respectively.

## Video Player

A VVC video player application is developed using OpenGL framework. The player application fetches the decoded output from the decoded picture buffer (DPB) and renders to output device in real-time.  OpenGL based pixel shader is used to convert the 10 bit 4:2:0 YUV format video frame to RGB32 format in real-time for display purpose.

# Experimental Results

The following experiments were conducted using the VTM-8.0 reference software and the JVET common test conditions [1]. An Intel Core™ i9-9900 X-series processor with 10 cores running at 3.5 GHz is used to generate experimental results for both the anchor i.e. VTM-8.0 and proposed parallelization.

The test results of the proposed implementation are compared against the VTM-8.0 anchor. The rest of the section is sub-divided into Sections 3.1 and 3.2. Section 3.1 includes the percentage contribution from parallelizing each slice decoding task explained in Section 2.2. Section 3.2 provides experimental results combining the methods proposed in Section 2. The decoding time reduction is calculated as the percentage reduction from the proposed method(s) over VTM-8.0. Mathematically, it can be represented as

| Decoding Time Reduction %=TVTM8.0-TProposedTVTM8.0×100 | (1) |
| --- | --- |

where, TVTM8.0 is the time taken by the individual modules of default VTM-8.0 and  TProposed is the corresponding module time from the proposed method.

## Slice decoding performance

The percentage decoding time reduction is calculated based on (1) for each task of slice decoding. Five 4K sequences and five 1080P sequences from CTC sequences are used for our experiments. Bitstreams are generated from Class A: 3840x2160 10 bit 4:2:0 YUV format and Class B: 1920x1080 8 bit 4:2:0 YUV format sequences at QPs: 22, 27, 32, 37. The test results of all the proposed methods using 20 decoding threads are compared against the VTM-8.0 anchor in this section.

Figure 10 shows the processing time percentage of each slice decoding tasks in VTM-8.0. Figure 11 shows the processing time percentage of each decoding task from the proposed parallel solution. The percentage of each slice decoding task time is almost equal for both VTM-8.0 and the proposed solution.

The decoding time percentage for parallelized intra CU reconstruction and MVCD tasks are slightly higher comparing to VTM-8.0 since both tasks are limited by the WPP dependency. All other tasks can be executed in parallel at CTU or pixel level and can use as many threads as possible.

Figure 10 VTM-8.0 complexity analysis at QP: 37

Figure 11 Proposed solution complexity analysis at QP: 37

Table 1 to Table 7 show the processing time comparison details between the parallel solution and VTM-8.0 for Class A sequences.

Table 1. MVCD decoding time

| Sequences | QP | Decoding time, % improvement for MVCD |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 5.564 | 1.337 | 75.97 | 73.43 |
|  | 27 | 4.809 | 1.273 | 73.53 |  |
|  | 32 | 4.503 | 1.269 | 71.82 |  |
|  | 37 | 5.042 | 1.392 | 72.39 |  |
| DaylightRoad2 | 22 | 5.702 | 1.372 | 75.94 | 69.86 |
|  | 27 | 3.417 | 0.999 | 70.76 |  |
|  | 32 | 2.905 | 0.95 | 67.30 |  |
|  | 37 | 3.138 | 1.085 | 65.42 |  |
| CampFireParty | 22 | 5.117 | 1.593 | 68.87 | 69.48 |
|  | 27 | 3.199 | 0.984 | 69.24 |  |
|  | 32 | 3.254 | 0.987 | 69.67 |  |
|  | 37 | 3.891 | 1.162 | 70.14 |  |
| Tango2 | 22 | 3.828 | 0.966 | 74.76 | 72.26 |
|  | 27 | 3.649 | 1.028 | 71.83 |  |
|  | 32 | 3.788 | 1.114 | 70.59 |  |
|  | 37 | 4.662 | 1.312 | 71.86 |  |
| FoodMarket4 | 22 | 4.24 | 1.131 | 73.33 | 69.93 |
|  | 27 | 3.506 | 1.022 | 70.85 |  |
|  | 32 | 3.34 | 1.047 | 68.65 |  |
|  | 37 | 3.587 | 1.188 | 66.88 |  |

Table 2. Inter CU reconstruction decoding time

| Sequences | QP | Decoding time, % improvement for Inter CU reconstruction |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 46.519 | 4.527 | 90.27 | 89.62 |
|  | 27 | 45.675 | 4.528 | 90.09 |  |
|  | 32 | 43.991 | 4.396 | 90.01 |  |
|  | 37 | 42.276 | 5.017 | 88.13 |  |
| DaylightRoad2 | 22 | 45.569 | 4.581 | 89.95 | 88.43 |
|  | 27 | 47.257 | 5.464 | 88.44 |  |
|  | 32 | 45.997 | 5.55 | 87.93 |  |
|  | 37 | 44.241 | 5.566 | 87.42 |  |
| CampFireParty | 22 | 29.833 | 2.882 | 90.34 | 90.32 |
|  | 27 | 26.331 | 2.542 | 90.35 |  |
|  | 32 | 25.826 | 2.489 | 90.36 |  |
|  | 37 | 25.075 | 2.446 | 90.25 |  |
| Tango2 | 22 | 38.671 | 3.776 | 90.24 | 88.83 |
|  | 27 | 41.305 | 4.58 | 88.91 |  |
|  | 32 | 39.718 | 4.666 | 88.25 |  |
|  | 37 | 37.822 | 4.574 | 87.91 |  |
| FoodMarket4 | 22 | 48.617 | 5.4 | 88.89 | 88.25 |
|  | 27 | 48.96 | 5.611 | 88.54 |  |
|  | 32 | 47.06 | 5.594 | 88.11 |  |
|  | 37 | 44.136 | 5.543 | 87.44 |  |

Table 3. Intra CU reconstruction decoding time

| Sequences | QP | Decoding time, % improvement for Intra CU reconstruction |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 8.956 | 2.211 | 75.31 | 70.50 |
|  | 27 | 4.494 | 1.311 | 70.83 |  |
|  | 32 | 3.056 | 0.945 | 69.08 |  |
|  | 37 | 2.168 | 0.72 | 66.79 |  |
| DaylightRoad2 | 22 | 12.708 | 3.187 | 74.92 | 70.07 |
|  | 27 | 4.941 | 1.412 | 71.42 |  |
|  | 32 | 2.903 | 0.916 | 68.45 |  |
|  | 37 | 1.965 | 0.678 | 65.50 |  |
| CampFireParty | 22 | 33.83 | 8.569 | 74.67 | 71.44 |
|  | 27 | 20.679 | 5.883 | 71.55 |  |
|  | 32 | 15.295 | 4.513 | 70.49 |  |
|  | 37 | 11.565 | 3.581 | 69.04 |  |
| Tango2 | 22 | 11.643 | 2.756 | 76.33 | 72.84 |
|  | 27 | 6.156 | 1.656 | 73.10 |  |
|  | 32 | 4.274 | 1.214 | 71.60 |  |
|  | 37 | 3.173 | 0.941 | 70.34 |  |
| FoodMarket4 | 22 | 7.739 | 2.062 | 73.36 | 70.78 |
|  | 27 | 5.533 | 1.555 | 71.90 |  |
|  | 32 | 4.278 | 1.274 | 70.22 |  |
|  | 37 | 3.267 | 1.057 | 67.65 |  |

Table 4. Inverse Re-shaper decoding time

| Sequences | QP | Decoding time, % improvement for Inverse Re-Shaper |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 1.499 | 0.213 | 85.79 | 86.82 |
|  | 27 | 1.5 | 0.203 | 86.47 |  |
|  | 32 | 1.494 | 0.193 | 87.08 |  |
|  | 37 | 1.491 | 0.18 | 87.93 |  |
| DaylightRoad2 | 22 | 1.497 | 0.201 | 86.57 | 87.37 |
|  | 27 | 1.496 | 0.169 | 88.70 |  |
|  | 32 | 1.488 | 0.186 | 87.50 |  |
|  | 37 | 1.495 | 0.199 | 86.69 |  |
| CampFireParty | 22 | 1.469 | 0.205 | 86.04 | 86.25 |
|  | 27 | 1.472 | 0.2 | 86.41 |  |
|  | 32 | 1.464 | 0.201 | 86.27 |  |
|  | 37 | 1.465 | 0.201 | 86.28 |  |
| Tango2 | 22 | 1.466 | 0.197 | 86.56 | 86.73 |
|  | 27 | 1.461 | 0.194 | 86.72 |  |
|  | 32 | 1.46 | 0.195 | 86.64 |  |
|  | 37 | 1.461 | 0.19 | 87.00 |  |
| FoodMarket4 | 22 | 1.488 | 0.197 | 86.76 | 87.37 |
|  | 27 | 1.491 | 0.18 | 87.93 |  |
|  | 32 | 1.497 | 0.185 | 87.64 |  |
|  | 37 | 1.493 | 0.192 | 87.14 |  |

Table 5. Adaptive Loop Filter (ALF) decoding time

| Sequences | QP | Decoding time, % improvement for Adaptive Loop Filter |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 22.277 | 4.93 | 77.87 | 73.37 |
|  | 27 | 15.693 | 4.278 | 72.74 |  |
|  | 32 | 10.59 | 3.037 | 71.32 |  |
|  | 37 | 6.999 | 1.991 | 71.55 |  |
| DaylightRoad2 | 22 | 23.656 | 5.151 | 78.23 | 73.69 |
|  | 27 | 19.455 | 4.855 | 75.04 |  |
|  | 32 | 15.971 | 4.475 | 71.98 |  |
|  | 37 | 11.102 | 3.387 | 69.49 |  |
| CampFireParty | 22 | 23.285 | 5.088 | 78.15 | 74.61 |
|  | 27 | 20.474 | 5.046 | 75.35 |  |
|  | 32 | 17.379 | 4.574 | 73.68 |  |
|  | 37 | 15.183 | 4.367 | 71.24 |  |
| Tango2 | 22 | 21.679 | 5.008 | 76.90 | 73.14 |
|  | 27 | 18.714 | 4.856 | 74.05 |  |
|  | 32 | 17.063 | 4.806 | 71.83 |  |
|  | 37 | 11.675 | 3.53 | 69.76 |  |
| FoodMarket4 | 22 | 22.101 | 5.005 | 77.35 | 72.74 |
|  | 27 | 17.587 | 4.863 | 72.35 |  |
|  | 32 | 14.161 | 4.034 | 71.51 |  |
|  | 37 | 9.727 | 2.942 | 69.75 |  |

Table 6. Sample Adaptive Offset (SAO) decoding time

| Sequences | QP | Decoding time, % improvement for Sample Adaptive Offset |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 7.155 | 1.392 | 80.55 | 73.92 |
|  | 27 | 1.376 | 0.323 | 76.53 |  |
|  | 32 | 0.502 | 0.134 | 73.31 |  |
|  | 37 | 0.34 | 0.118 | 65.29 |  |
| DaylightRoad2 | 22 | 7.724 | 1.512 | 80.42 | 74.43 |
|  | 27 | 2.142 | 0.487 | 77.26 |  |
|  | 32 | 0.669 | 0.183 | 72.65 |  |
|  | 37 | 0.325 | 0.106 | 67.38 |  |
| CampFireParty | 22 | 7.769 | 1.449 | 81.35 | 76.32 |
|  | 27 | 3.099 | 0.721 | 76.73 |  |
|  | 32 | 0.949 | 0.231 | 75.66 |  |
|  | 37 | 0.355 | 0.101 | 71.55 |  |
| Tango2 | 22 | 4.572 | 0.964 | 78.92 | 69.08 |
|  | 27 | 0.731 | 0.181 | 75.24 |  |
|  | 32 | 0.282 | 0.095 | 66.31 |  |
|  | 37 | 0.145 | 0.064 | 55.86 |  |
| FoodMarket4 | 22 | 2.038 | 0.444 | 78.21 | 71.88 |
|  | 27 | 0.685 | 0.168 | 75.47 |  |
|  | 32 | 0.296 | 0.095 | 67.91 |  |
|  | 37 | 0.179 | 0.061 | 65.92 |  |

Table 7. De-blocking Filter (DBF) decoding time

| Sequences | QP | Decoding time, % improvement for De-blocking Filter |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 (sec) | Proposed (sec) | Time reduction (%) | Average reduction (%) |
| CatRobot1 | 22 | 44.793 | 4.183 | 90.66 | 90.76 |
|  | 27 | 30.45 | 2.809 | 90.78 |  |
|  | 32 | 25.829 | 2.373 | 90.81 |  |
|  | 37 | 23.744 | 2.19 | 90.78 |  |
| DaylightRoad2 | 22 | 54.709 | 5.324 | 90.27 | 90.19 |
|  | 27 | 32.161 | 3.169 | 90.15 |  |
|  | 32 | 25.029 | 2.456 | 90.19 |  |
|  | 37 | 21.645 | 2.131 | 90.15 |  |
| CampFireParty | 22 | 74.135 | 7.443 | 89.96 | 90.24 |
|  | 27 | 47.757 | 4.585 | 90.40 |  |
|  | 32 | 37.408 | 3.616 | 90.33 |  |
|  | 37 | 32.058 | 3.115 | 90.28 |  |
| Tango2 | 22 | 42.891 | 3.995 | 90.69 | 90.39 |
|  | 27 | 30.171 | 2.887 | 90.43 |  |
|  | 32 | 26.538 | 2.552 | 90.38 |  |
|  | 37 | 25.068 | 2.487 | 90.08 |  |
| FoodMarket4 | 22 | 33.589 | 3.259 | 90.30 | 90.16 |
|  | 27 | 26.849 | 2.636 | 90.18 |  |
|  | 32 | 23.459 | 2.303 | 90.18 |  |
|  | 37 | 21.146 | 2.117 | 89.99 |  |

## Performance improvement combining all decoding tasks

The total percentage decoding time reduction using 20 decoding threads calculated based on (1) by combining Approaches I and II discussed in Section 2 is reported in Table 8, Table 9 and Figure 12. Five Class A and five Class B sequences from CTC sequences are used for our experiments. Bitstreams are generated from Class A: 3840x2160 10 bit 4:2:0 YUV and Class B: 1920x1080 8 bit 4:2:0 YUV format sequences at QPs: 22, 27, 32, 37.

Table 8. FPS improvement due to CABAC and slice decoding parallelization for Class A sequences

| Sequences | QP | FPS |  |
| --- | --- | --- | --- |
|  |  | VTM-8.0 | Proposed |
| CatRobot1 | 22 | 1.91 | 8.57 |
|  | 27 | 2.61 | 11.82 |
|  | 32 | 3.05 | 15.61 |
|  | 37 | 3.36 | 19.03 |
| DaylightRoad2 | 22 | 1.65 | 7.36 |
|  | 27 | 2.44 | 10.59 |
|  | 32 | 2.89 | 12.03 |
|  | 37 | 3.29 | 14.42 |
| CampFireParty | 22 | 1.34 | 5.56 |
|  | 27 | 2.10 | 9.68 |
|  | 32 | 2.62 | 11.42 |
|  | 37 | 3.02 | 12.89 |
| Tango2 | 22 | 2.06 | 9.25 |
|  | 27 | 2.62 | 11.05 |
|  | 32 | 2.90 | 11.64 |
|  | 37 | 3.23 | 14.64 |
| FoodMarket4 | 22 | 2.26 | 10.12 |
|  | 27 | 2.63 | 11.31 |
|  | 32 | 2.94 | 12.35 |
|  | 37 | 3.31 | 15.91 |

Figure 12 Performance improvement combining all decoding tasks

Table 9. FPS improvement due to CABAC and slice decoding parallelization for Class B sequences

| Sequences | QP | FPS |  |
| --- | --- | --- | --- |
|  |  | VTM-8.0 | Proposed |
| BQTerrace | 22 | 4.71 | 19.45 |
|  | 27 | 10.63 | 35.89 |
|  | 32 | 13.78 | 44.63 |
|  | 37 | 15.58 | 50.23 |
| RitualDance | 22 | 7.64 | 29.87 |
|  | 27 | 9.58 | 36.83 |
|  | 32 | 11.38 | 42.38 |
|  | 37 | 13.16 | 47.98 |
| MarketPlace | 22 | 7.25 | 28.44 |
|  | 27 | 9.45 | 36.34 |
|  | 32 | 11.51 | 43.81 |
|  | 37 | 13.39 | 51.33 |
| BasketBallDrive | 22 | 6.51 | 26.31 |
|  | 27 | 8.95 | 34.42 |
|  | 32 | 10.73 | 37.59 |
|  | 37 | 12.28 | 41.60 |
| Cactus | 22 | 6.85 | 28.49 |
|  | 27 | 10.21 | 41.09 |
|  | 32 | 12.24 | 46.57 |
|  | 37 | 13.89 | 60.09 |

## Performance improvement w.r.t. number of threads

The decoding time reduction depends on the total number of threads. An Intel Core I-9-9900 X-series processor with 10 physical cores (hyper-threading enabled) was used for our experimentation. Table 10 shows the decoded frames per second (FPS) vs. number of threads. It provides details for five Class A and five Class B sequences at QPs: 22, 27, 32, 37.

Table 10. Performance improvement w.r.t. number of threads

| Sequences | QP | FPS w.r.t. number of threads |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VTM-8.0 | 4 | 8 | 10 | 20 |
| CatRobot1 | 22 | 1.91 | 7.31 | 9.58 | 9.50 | 8.57 |
|  | 27 | 2.61 | 9.78 | 13.13 | 13.02 | 11.82 |
|  | 32 | 3.05 | 11.70 | 16.28 | 16.54 | 15.61 |
|  | 37 | 3.36 | 13.37 | 19.62 | 19.86 | 19.03 |
| DaylightRoad2 | 22 | 1.65 | 6.64 | 8.59 | 8.40 | 7.36 |
|  | 27 | 2.44 | 9.02 | 12.04 | 11.87 | 10.59 |
|  | 32 | 2.89 | 10.65 | 14.23 | 13.82 | 12.03 |
|  | 37 | 3.29 | 12.25 | 16.76 | 16.42 | 14.42 |
| CampFireParty | 22 | 1.34 | 6.05 | 6.38 | 6.02 | 5.56 |
|  | 27 | 2.10 | 8.24 | 10.25 | 10.24 | 9.68 |
|  | 32 | 2.62 | 9.92 | 12.43 | 12.44 | 11.42 |
|  | 37 | 3.02 | 11.06 | 13.81 | 13.89 | 12.89 |
| Tango2 | 22 | 2.06 | 7.92 | 10.35 | 10.21 | 9.25 |
|  | 27 | 2.62 | 9.74 | 12.77 | 12.63 | 11.05 |
|  | 32 | 2.90 | 10.58 | 13.87 | 13.32 | 11.64 |
|  | 37 | 3.23 | 12.23 | 16.49 | 16.19 | 14.64 |
| FoodMarket4 | 22 | 2.26 | 8.41 | 11.41 | 11.00 | 10.12 |
|  | 27 | 2.63 | 9.71 | 12.99 | 12.42 | 11.31 |
|  | 32 | 2.94 | 10.90 | 14.84 | 14.38 | 12.35 |
|  | 37 | 3.31 | 12.48 | 17.65 | 17.47 | 15.91 |
| BQTerrace | 22 | 4.71 | 20.97 | 21.98 | 21.62 | 19.45 |
|  | 27 | 10.63 | 36.93 | 42.24 | 40.40 | 35.89 |
|  | 32 | 13.78 | 45.73 | 51.61 | 49.27 | 44.63 |
|  | 37 | 15.58 | 52.36 | 59.60 | 56.29 | 50.23 |
| RitualDance | 22 | 7.64 | 27.65 | 32.09 | 31.42 | 29.87 |
|  | 27 | 9.58 | 33.74 | 38.74 | 37.83 | 36.83 |
|  | 32 | 11.38 | 39.49 | 45.53 | 44.79 | 42.38 |
|  | 37 | 13.16 | 45.02 | 51.09 | 50.63 | 47.98 |
| MarketPlace | 22 | 7.25 | 27.68 | 31.29 | 30.60 | 28.44 |
|  | 27 | 9.45 | 33.99 | 39.37 | 38.50 | 36.34 |
|  | 32 | 11.51 | 40.97 | 47.74 | 46.20 | 43.81 |
|  | 37 | 13.39 | 48.36 | 56.49 | 56.13 | 51.33 |
| BasketBallDrive | 22 | 6.51 | 25.39 | 29.62 | 28.76 | 26.31 |
|  | 27 | 8.95 | 32.31 | 37.28 | 35.37 | 34.42 |
|  | 32 | 10.73 | 37.57 | 43.01 | 41.60 | 37.59 |
|  | 37 | 12.28 | 42.13 | 47.31 | 46.14 | 41.60 |
| Cactus | 22 | 6.85 | 26.41 | 30.63 | 30.28 | 28.49 |
|  | 27 | 10.21 | 36.31 | 42.26 | 42.71 | 41.09 |
|  | 32 | 12.24 | 43.75 | 52.19 | 51.87 | 46.57 |
|  | 37 | 13.89 | 51.33 | 62.01 | 61.47 | 60.09 |

# Conclusion

InterDigital has implemented software based VVC decoder and player using parallel processing on multi-core computing platform. The parallel solution is verified on VVC Class A and Class B test sequences under CTC condition, and achieved decoding speed between 9 to 20 FPS for 5 Class A sequences.

# Reference

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

Ittiam system private limited, “AHG16:Early Implementation of VVC software player and Demonstration on Mobile device”, JVET-P0307, Geneva, CH, October, 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).