| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0392-v6 |
| --- | --- |

| Title: | VVC Version 1 Profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan (Broadcom Inc.) Didier LeGall (Ambarella) Aaron Wells (Ambarella) Harold Edward (AMD) Gabor Sines (AMD) David Singer (Apple) Alexis Tourapis (Apple) Sassan Pejhan (ATEME) Mickael Raulet (ATEME) Peshala Pahalawatta (ATT Inc.) Eric Petajan (ATT Inc.) Scott Davis (Charter Communications) Dan Grois (Comcast Cable) Yasser Syed (Comcast Cable) Xavier Ducloux (Harmonic Inc.) Paul Haskell (Harmonic Inc.) Julien Le Tanou (MediaKind) Clarence Hau (NBCUniversal) Ajay Luthra (Picsel Labs) Teruhiko Suzuki (Sony) Eric Chai (Ubilinx) | Email: | wade.wan@broadcom.com |
| Source: | Broadcom Inc., Ambarella, AMD, Apple, ATEME, ATT Inc., Charter Communications, Comcast Cable, Harmonic Inc., MediaKind, NBCUniversal, Picsel Labs, Sony, Ubilinx |  |  |

_____________________________

v2: added AMD as an author

v3: added AT&T Inc. as an author

v4: added NBCUniversal as an author

v5: editorial changes in response to a request to make the abstract and introduction sections clearer by more directly stating the intent of the proposal in those sections

v6: added MediaKind and Picsel Labs as authors.  Editorial changes to use “Constrained Main” (for the proposed profile without subpictures and scalability) and “Main” (for the profile with subpictures and scalability) instead of “Main” and “Advanced”, respectively, that were proposed in earlier versions.  This is to address concerns that VVC may somehow be negatively affected by the Main Profile being less versatile than perceived.

# Abstract

This contribution claims that the current single profile for 10 bit 4:2:0 video support in version 1 of the VVC standard is problematic given the maturity and interoperability of the subpicture and scalability features.  It is also asserted that these features are not applicable for the video use cases of many products so their inclusion in the only v1 profile with 10 bit 4:2:0 support increases the cost for those products with no practical benefit.  It is proposed that two profiles for 10 bit 4:2:0 video support are defined in version 1 to enable the appropriate markets to choose the appropriate profile for their needs:

- A “Constrained Main 10” profile without subpictures and scalability

- A “Main 10” profile with subpictures and scalability

Note that the feature set of Main 10 Profile is unaffected by this proposal but this would enable products that neither utilize nor benefit from subpictures and scalability to minimize the cost and risk of their development while not affecting the profile definition for other products to utilize subpictures and scalability.

# Introduction

The current working draft text for the VVC standard [1] has a single profile (the Main 10 profile) defined for 10 bit 4:2:0 video support.  The authors of this contribution are concerned that a single “main” profile for version 1 will be detrimental to the commercial adoption of the VVC standard and propose defining two profiles for 10 bit 4:2:0 support in the first version of the VVC standard:

- A “Constrained Main 10” profile without subpictures and scalability

- A “Main 10” profile with subpictures and scalability

Note that the feature set of Main 10 Profile in the current draft is unaffected by this proposal.

Section 2 begins with the assertion that a single “main” v1 profile that include features not well matched to product requirements poses a risk to commercial adoption of VVC.  It also contends that there are several aspects of the subpicture and scalability features that make it desirable to separate its support into another v1 profile.

Sections 3 and 4 provide more background and discussion on subpictures and scalability, respectively.

Section 5 concludes by proposing not to remove these features from version 1 but to include them in another v1 profile, and to allow products and services to choose their appropriate level of support based on their commercial need rather than forcing support of these features on them by inclusion in a single profile.

# Background

The risk with defining a single “main” v1 profile is that the authors of this contribution foresee substantial interop problems with this initial profile when implementations only support the subset of features necessary to match actual product requirements.

A similar situation occurred with the Baseline Profile in the AVC standard, which included the Flexible Macroblock Ordering (FMO) and Arbitrary Slice Ordering (ASO) tools when first defined in 2003.  These tools were not widely adopted in the industry and many implementors chose to not implement these features.  This resulted in undesirable interop problems and market confusion until the Constrained Baseline Profile was defined in 2009 with these tools removed from the Baseline Profile.

Another previous example occurred with MPEG-4 Advanced Simple Profile, which encountered many compatibility problems with product development.  An unofficial subset (ASP*) was the subset widely implemented by vendors and did not include several features such as quarter-pel motion compensation (Qpel), Global Motion Compensation (GMC), Data Partitioning (DP), and Reversible VLC (RVLC).  Later DivX defined a subset that became a de facto industry profile but even that later version did not match the ASP definition in the MPEG-4 standard.

These examples illustrate the fact that industry will create their own profiles when faced with tools deemed burdensome or with minimal mainstream benefits and not necessarily be forced into supporting tools in a profile definition.  Unfortunately, that creates risk and compatibility issues that the authors would like to avoid with VVC.  Especially in the current video landscape, where there are other video codec alternatives available, it would be highly desirable to minimize potential adoption issues of VVC to maximize its chances of commercial success.

This contribution follows up on the JVET-Q0485 contribution [3] presented at the Brussels meeting which noted:

- Profile (and level) definitions are critical to commercial viability

- Raised the issue of whether one or more “main” v1 profiles will be defined

- Commercial adoption of a “main” profile should not be impeded by unnecessary cost (implementation, licensing, etc.) or immature features

Concerns about subpictures and scalability were expressed in that contribution and while there has been work and progress since the Brussels meeting to improve their technical aspects, the authors of this contribution assert that there are still questions and concerns about their maturity and stability in the specification as well as in their implementation in the VTM reference software.  So far, there has been limited discussion of profile definitions, but the assessment of the proponents of this contribution (which represent a broad range of companies across the video ecosystem) is that many services and products would neither utilize nor benefit from these two features.  Thus, their inclusion in the single/only “main” profile introduces unnecessary cost (for implementation as well as licensing) and delay deployment of these products with no practical benefit.  Note that the proposal is not to remove these features from the first VVC version but as a compromise to define another v1 profile with these features to enable other products who wish to utilize these tools.

# Subpictures

The main idea behind subpictures is to enable separate subpictures to be reassembled together for decoding into a single picture.  The following comments have been expressed and discussed in earlier meetings:

- Similar functionality can already be achieved in existing systems using separate streams with post-decode processing.

  - Such an approach appears to offer more flexible support for this feature (e.g., ability to combine subpicture of different formats like SDR & HDR, ability to separately scale subpictures, etc.) that raise questions of the utility of embedding the subpicture concept inside a video codec

- Some use cases, like OMAF viewport dependent streaming, are already implemented currently without subpictures using HEVC with encoder-only conventions and appropriate metadata/SEI

- Subpictures require highly coordinated encoders and bitstreams.  Thus, in general, these are not applicable to many video use cases and are targeted towards niche applications.

The intent of listing these comments is not necessarily to engage in debate and convince everyone that these are absolutely true but to explain what has shaped the opinion of the authors of this contribution that the subpicture feature is not going to be applicable to many video use cases and therefore it is not desirable to be implemented in every decoder, which would be the result of a single “main” v1 profile with this tool enabled.

The authors understand that there has been recent progress on implementing subpictures in the VTM but the same concerns remain that were first expressed in JVET-Q0485.  The actual SW development tying together the conceptual design to the actual low level tools started very late in the standardization process so it is not surprising that the maturity of this feature is still questionable despite the recent SW efforts.

As an example, some experts who have not spent a lot of time analyzing subpictures (because it is not a high priority feature for their markets) recently reviewed the subpicture framework in the VTM with the assumption that the feature is in a stable state.  A very cursory review found multiple bugs in the implementation as of April 6, 2020:

- Subpic motion clipping is missing completely for affine prediction.

- The rectangular slice initialization function adds CTUs in raster-scan order rather than tile-scan order for SingleSlicePerSubPic.

- The rectangular slice initialization function is never called for SingleSlicePerSubPic, so the slice map is invalid.

- The CTU to subpicture mapping is never initialized, so getCtuToSubPicIdx() always fails.

- Never possible to change subpic IDs (even at IRAP) because previousSubPicIds list is not cleared at the end of each picture (the list grows indefinitely).

- Subpic ID overflow – 8-bit variables used throughout software, but ID length can be 16-bits

Appropriate bug tickets will be filed for these issues and even if these bugs are fixed shortly after we have highlighted them, this finding raises the question of whether this feature is stable and mature?  The authors of this contribution do not believe it is stable or mature, partly due to the limited amount of time that it has been working in the VTM software, which then enables other experts to further analyze.

Therefore, this contribution proposes to disable multiple subpictures in a picture (require one subpicture per picture) in the Constrained Main 10 profile which can be accomplished by the constraint that the SPS syntax element subpic_info_present_flag shall be equal to 0.

# Scalability

Traditionally, scalability support has been specified using another profile that is separately defined from profiles for single layer coding as was done with the SVC extensions for AVC [2] and the Scalable Main and Scalable Main 10 profiles for HEVC [4].

The initial design goal was to provide some tools and support for scalability in the first version of VVC rather than wait for future versions as it was claimed that this impacted scalability adoption in previous standards.  While it is unclear whether the timing of a scalable profile definition is the primary reason that scalable coding has not been widely adopted in the marketplace, the authors do not object to a profile containing scalability support but question why scalability support is not mandated for every decoder to implement.

It should be noted that the Reference Picture Resampling (RPR) feature, when used in the same layer, is not affected by this proposal as, while there are some aspects that overlap with scalability (the ability to switch decoded resolutions over time), it is the layered aspects of scalability that are concerning to the authors of this proposal.  Therefore, this proposal places no constraints on RPR, and thus RPR is supported in both the “Constrained Main 10” and “Main 10” profiles.

There has been much progress toward enabling a simpler form of scalability (compared to previous standards) for inclusion in a version 1 profile.  However, there are still many permutation/combinations needed for a decoder to support to be fully compliant, e.g., different number and types of layers as well as the DPB management and relationship between different layers conveyed by the output layer sets.

Thus, compliant decoder support will require a significant amount of testing and verification for products to be confident they are fully compliant.  The issue goes beyond the conceptual design and state of the specification text but will be reflected on actual product development and product specifications.  This may result in undesirable scenarios such as delayed product launches or risk of initial/early deployments (that may not even be interested in scalability) being discovered to be nonconformant at a later point in time.

Therefore, this contribution proposes to disable the following aspects of scalable/layered coding in Constrained Main 10 profile:

- The SPS syntax element inter_layer_ref_pics_present_flag shall be equal to 0.

  - This disables inter-layer prediction used for SNR and spatial scalability

- The VPS syntax element vps_max_layers_minus1 shall be equal to 0

  - This disables multiple output layer sets so decoders do not have to manage different output layer sets

- A single value of the NAL unit header syntax element nuh_layer_id shall be used across the whole CVS (coded video sequence)

  - This disables more than one picture in each AU (note that the current draft does not mandate that nuh_layer_id has to start from 0)

# Summary

This contribution asserts that ratifying the first version of VVC with a single “main” profile as in the current draft text is problematic.  It discusses the concerns over the maturity and interoperability implications of the subpicture and scalability features, as well as questions the requirement for these tools to be implemented in every decoder supporting a v1 profile due to the observation that these tools are not applicable for many anticipated products.  The proposed solution is not to remove these features from version 1 but to define another v1 profile without these features, to allow products and services to choose their appropriate level of support based on their commercial need rather than forcing support of these features on them by inclusion in a single profile.

# References

[1] B. Bross et al., “Versatile Video Coding (Draft 8)”, document JVET-Q2001, JVET, Brussels, Belgium, January 2020.

[2] ISO/IEC 14496-10 | ITU-T Rec. H.264, “Advanced video coding for generic audiovisual services”, ISO/IEC | ITU-T standard, 2019.

[3] W. Wan et. al., “Profile and Level Definitions:, document JVET-Q0485, JVET, Brussels, Belgium, January 2020.

[4] ISO/IEC 23008-2 | ITU-T Rec. H.265, “High efficiency video coding”, ISO/IEC | ITU-T standard, 2019.

# Patent rights declaration(s)

Broadcom Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ambarella may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

AMD (Advanced Micro Devices Inc.) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ATT Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Charter Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Comcast Cable may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harmonic Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaKind may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NBCUniversal Media, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Picsel Labs  has no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ubilinx may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).