| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0393 |
| --- | --- |

| Title: | AHG9: On LMCS for GDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong Krit Panusopone Miska M. Hannuksela | Tel: Email: | +1 858-335-9196 limin.2.wang@nokia.com seungwook.hong@nokia.com krit.panusopone@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

____________________________

# Abstract

According to VVC Draft 8, it is asserted that the use of LMCS will cause leaks for GDR, so that exact-match at recovery point becomes unguaranteed for GDR. To fix the asserted problem associated with using LMCS with GDR, this contribution proposes to add a constraint to disable chroma residual scaling of LMCS for pictures within a GDR period.

Encoders can satisfy the proposed constraint by disabling either LMCS entirely or just the chroma residual scaling part of LMCS for the pictures within a GDR period. In the performed simulations, the impact on overall coding performance by the proposal is xx% for disabling LMCS within the GDR periods and yy% for disabling chroma residual scaling of LMCS within the GDR periods.

# Introduction

Gradual Decoding Refresh (GDR) is mainly for (ultra) low delay applications, where it is desirable that all the coded pictures have similar number of bits so that the encoder to decoder delay can be reduced to around one picture interval. Instead of coding an intra picture at a random access point, GDR progressively refreshes pictures by spreading intra coded areas over several pictures.

Fig. 1 illustrates the basic concept of (vertical) GDR, where a GDR period starts with picture of POC (n) and ends with picture of POC(n+N-1), including N pictures in total. The first picture of POC(n) within the GDR period is called GDR picture. Forced intra coded areas (green) gradually spread over the N pictures of the GDR period from the left to the right. The picture of POC(n+N) at the recovery point is called the recovery point picture.

Fig. 1. A GDR period starts with a GDR picture of POC(n) and ends with picture of POC(n+N-1). A picture within GDR period consists of a clean area and a dirty area separated by a virtual boundary.

Typically, a current picture within a GDR period consists of a clean area and a dirty area, where the clean area may contain a forced intra area next to the dirty area for progressive intra refresh (PIR), as shown in picture of POC(n+1) of Fig. 1. The boundary between clean area and dirty area is signaled by virtual boundary syntax in Picture Header.

For exact-match at recovery point, CUs in clean area cannot use any coding information (e.g. reconstructed pixels, coding mode, MVs, refIdx, etc.) from dirty area. However, under the current VVC spec [1], LMCS for CUs in clean area may use the coding information in dirty area. So, exact-match may not be guaranteed for GDR with LMCS.

Fig. 2 shows an example about how LMCS for CUs in clean area could use the reconstructed pixels in dirty area, where CTU size is set to 128x128, a virtual boundary passes through a current CTU, and a current CU (in green) of 32x32 within the current CTU is in clean area.

In the current VVC spec [1], the syntax element sps_log2_ctu_size_minus5 is signaled, and the assocaited variables are calculated as follows

CtbSizeY = 1  <<  (sps_log2_ctu_size_minus5 + 5),

sizeY = Min( CtbSizeY, 64 ).

For a current CU in a current CTU, an array recLuma[i], i=0, 1,…,2×sizeY-1, is defined as a collection of sizeY left reconstructed neighbouring pixels and sizeY above reconstructed neighbouring pixels, as shown in Fig. 2 where sizeY=64.

The average of array recLuma[i], i=0, 1,…,2×sizeY-1, is calculated as

invAvgLuma=((i=02×sizeY-1recLumai)+sizeY)≫log2(2×sizeY)

This variable invAvgLuma calculated above is then used to determine a scaling factor, varScale, for chroma residuals,

invAvgLuma→varSacle .

With the scaling factor varScale, the chroma components (Cb and Cr) of the current CU are finally reconstructed as follows.

Cbreci[j]=Cbpreij+Cbresij×varScaleCrreci[j]=Crpreij+Crresij×varScale

where the subscripts rec, pre and res mean reconstructed, prediction and residual, respectively.

Fig. 2 A virtual boundary passes through a CTU of 128x128 and a current CU of 32x32 is in the clean area.

It is possible that some elements of array recLuma[i], i=0, 1,…,2×sizeY-1, may be from dirty area, which means the final reconstruction of chroma components of a current CU in clean area may use the reconstructed neighbouring luma pixels in dirty area, which will cause the leaks and result in exact-match not be achievable. For example, in Fig. 2, for the current CU (in green), its above reconstructed neighbouring luma pixels (in orange) of 96 to 127 are in dirty area.

# Proposal

To fix the asserted problem associated with LMCS for GDR, it is proposed to add a constraint on chroma residual scaling of LMCS for pictures within a GDR period as follows. It is remarked that encoders can satisfy the proposed constraint by disabling either LMCS entirely (by setting sps_lmcs_enabled_flag equal to 0 or ph_lmcs_enabled_flag equal to 0) or just the chroma residual scaling part of LMCS for the pictures within a GDR period.

A new variable WithinGdrPeriodFlag is first introduced in Picture Header semantics as follows, where the changes on top of the current VVC spec [1] are in red.

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If the current picture is a GDR picture that is associated with the PH, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The value of recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When the current picture is a GDR picture, the variable GdrPicOrderCntVal, RpPicOrderCntVal are derived as follows: GdrPicOrderCntVal = PicOrderCntVal RpPicOrderCntVal = GdrPicOrderCntVal + recovery_poc_cnt (81) For the current picture, the variable WithinGdrPeriodFlag is derived to be equal to 1 if there is a GDRpicture that has GdrPicOrderCntVal less than or equal to PicOrderCntVal and RpPicOrderCntVal is greater than PicOrderCntVal, and derived to be equal to 0 otherwise.

Constraint on chroma residual scaling of LCMS for pictures with a GDR period: When a current picture is within a GDR period, chroma residual scaling of LMCS is required to be disabled at the picture header, i.e. ph_chroma_residual_scale_flag is required to be equal to 0 (or absent and thus inferred to be equal to 0). The text in red is proposed:

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the all slices associated with the PH. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be disabled for one, or more, or all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0. When WithinGdrPeriodFlag is equal to 1, ph_chroma_residual_scale_flag shall be equal to 0.

# Experimental Results

The proposals were evaluated using VTM 8.0 over JVET test sequences under low-delay B CTC (i.e., encoder_lowdelay_vtm.cfg). Within a second of pictures, each picture n=0,1, …,frame_rate-1 is assigned one unique (vertical) virtual boundary at the position of

x=picture_width8×frame_rate×8×n

relative to the left boundary (x=0) of the picture, where picture_width is the picture width and frame_rate is the frame rate of test sequence.

The test results are compared with an anchor. The anchor bitstreams are generated using the same (modified) low-delay B CTC.

The performance results are shown in Table 1 for disabling LMCS for pictures within a GDR period and in Table 2 for disabling chroma residual scaling of LMCS for pictures within a GDR period.

As seen, the impact on coding performance by the proposals is very minor; xx% for disabling LMCS for pictures within a GDR period and yy% for disabling chroma residual scaling of LMCS for pictures within a GDR period.

Table 1

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal 1 over VTM 8.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class D | 0.57% | -0.74% | -0.26% | #NUM! | #NUM! | 0.39% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |

Table 2

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal 2 over VTM 8.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class D | -0.19% | 1.60% | 2.02% | #NUM! | #NUM! | 0.18% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |

# Conclusion

This contribution reported that under the current VVC specification, LMCS will cause leaks for GDR, resulting in exact-match at recovery point is impossible. To fix the reported problem associated with LMCS for GDR, it was proposed to add constraint to disable chroma residual scaling of LMCS for pictures within a GDR period. With the proposal, LCMS cause no leaks for GDR. The impact on overall coding performance by the proposal was xx% with disabling LMCS and yy% with disabling chroma residual scaling only.

# References

- B. Bross, J. Chen, S. Lin, and Y-K. Wang, “Versatile Video Coding”, JVET-Q2001-v13, Jan 2020.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).