| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0401-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0401 : A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck of a proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the results of the crosscheck of the JVET-R0401 (A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST).

# Experimental results

The crosscheck has been performed for an intra-only (AI) and a random-access (RA) configuration with the VTM-8.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. Simulations have been run for the main test defined by the proponents of JVET-R0358.

Table 1: Results for an AI configuration on top of VTM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.58% | 2.00% | 93% | 101% |
| Class A2 | -0.04% | 1.61% | 1.64% | 93% | 100% |
| Class B | 0.12% | 1.05% | 1.42% | 95% | 100% |
| Class C | 0.25% | 0.62% | 0.97% | 91% | 98% |
| Class E | 0.22% | 1.10% | 0.90% | 93% | 98% |
| Overall | 0.12% | 1.14% | 1.37% | 93% | 100% |
| Class D | 0.23% | 0.95% | 1.10% | 94% | 100% |
| Class F | 0.22% | 0.55% | 0.55% | 95% | 101% |
| Class TGM | 0.09% | 0.03% | 0.03% | 96% | 101% |

Table 2: Results for a RA configuration on top of VTM-8.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.42% | 0.51% | 102% | 99% |
| Class A2 | 0.00% | 0.52% | 0.66% | 101% | 99% |
| Class B | 0.05% | 0.55% | 0.78% | 99% | 99% |
| Class C | 0.15% | -0.39% | -0.34% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.27% | 0.40% | 100% | 99% |
| Class D | 0.18% | 0.70% | 0.81% | 100% | 98% |
| Class F | 0.25% | -0.19% | 0.29% | 98% | 99% |
| Class TGM | 0.08% | -0.02% | -0.02% | 99% | 100% |

# Conclusion

Results perfectly match the ones presented by the proponents in JVET-R0358 in AI. In RA, however, there are various tiny differences in the values of the last PSNR decimal digit and some small differences in decimal digit rounding in the rate of RaceHorses. This is a very minor issue probably due to differences in the methods used for the measurements of the rate and the PSNR. Encoder and decoder run-times are very similar.