| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0402-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0330: AHG16: On clipping average luma value for chroma residual scaling factor derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jie Chen | Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of removal of clipping operation on averaged luma value for chroma residual scaling actor calculation proposed in JVET-R0330[1]. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

In VVC draft 8, the average of neighboring reconstructed luma samples above and left to one 64×64 region is used to calculate the chroma residual scaling factor for the coding units (CUs) inside the region. Additionally, one clipping operation is applied to clip the luma average to the full range of the internal bit-depth, which is asserted to be unnecessary. JVET-R0330 proposed to remove such unnecessary clipping operation from the derivation of chroma residual scaling factor.

The test is conducted in this contribution. It is reportedly shown the coding results are identical with those reported in JVET-R0330.

Test Result

The test results are shown in Table 1 to Table 3.

Table 1. Simulation results of R0330 in AI

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 94% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 94% |

Table 2. Simulation results of R0330 in RA

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 96% |

Table 3. Simulation results of R0330 in LDB

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% |

References

[1] X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, AHG16: On clipping average luma value for chroma residual scaling factor derivation, JVET-R0330, Teleconference, April, 2020.