| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0404-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on high level control of LMCS, Scaling list, ALF and SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of the 20 proposals on high level control of LMCS, scaling list, ALF and SAO submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# High level control and semantics changes of LMCS

The following aspects are proposed:

- Controlling of presence of the SH LMCS enabled flag slice_lmcs_enabled_flag

  - Conditionally add a new SPS SE to indicate whether slice_lmcs_enabled_flag is present, and when not present, infer the value to be equal to the PH LMCS enabled flag. (JVET-R0051)

  - Replace the PH flag ph_lmcs_enabled_flag with a 2-bit ph_lmcs_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2); and only signal slice_lmcs_enabled_flag for mode 2. (JVET-R0063)

  - Skip the signalling of the SH LMCS enabled flag for the one slice per picture case. (JVET-R0089, JVET-R0098, JVET-R0210, JVET-R0200, JVET-R0202)

- Slice-level control of chroma residual scaling

  - Remove the PH control flag (ph_chroma_residual_scale_flag) and add one flag in SH. (method 1 of JVET-R0096, method 2 of JVET-R0171)

  - Add a control flag in slice level under certain condition (proposal 2 of JVET-R0089, method 2 of JVET-R0096, method 1 of JVET-R0171)

    - if( slice_lmcs_enabled_flag  &&  ph_chroma_residual_scale_flag ) (JVET-R0089, method 1 of JVET-R0171)

    - if( slice_lmcs_enabled_flag  &&  ChromaArrayType ! =  0  &&  ph_chroma_residual_scale_flag ) (method 2 of JVET-R0096)

- Revised semantics

- Update the semantics of sps_lmcs_enabled_flag equal to 1 to use the wording of "may be used" instead of "is used". (JVET-R0051, JVET-R0160)

sps_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be used in the CLVS. sps_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not used in the CLVS.

- Reivse the current semantics of ph_lmcs_enabled_flag to the following: (JVET-R0051, JVET-R0160)

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be enabled for one, or more, or all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be is disabled for one, or more, or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

- Reivse the current semantics of ph_chroma_residual_scale_flag to the following:

  - ph_chroma_residual_scale_flag equal to 1

    - specifies that chroma residual scaling is may be enabled for all slices associated with the PH. (JVET-R0051, JVET-R0160)

    - specifies that chroma residual scaling is enabled for all slices associated with the PH and whether it is applied for each slice is further controlled by the slice_lmcs_used_flag signalled in the slice header. (JVET-R0063)

  - ph_chroma_residual_scale_flag equal to 0

    - ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be disabled for one, or more, or is disabled for all slices associated with the PH. (JVET-R0051, JVET-R0063, JVET-R0160)

- Update the semantics of slice_lmcs_enabled_flag equal to 1 to use the wording of "luma mapping is enabled for the current slice and chroma scaling may be enabled for the current slice" instead of "luma mapping with chroma scaling is enabled for the current slice" (JVET-R0160):

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice and chroma scaling may be enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

# Design questions on high level control of scaling list

The following aspects are proposed:

- Controlling of presence of the SH explicit scaling list enabled flag slice_explicit_scaling_list_used_flag

- Conditionally add a new SPS SE to indicate whether slice_explicit_scaling_list_used_flag is present, and when not present, infer the value to be equal to the PH explicit scaling list enabled flag. (JVET-R0051)

- Replaced the PH one-bit flag by a 2-bit ph_explicit_scaling_list_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). and only signal slice_explicit_scaling_list_used_flag for mode 2. (JVET-R0064)

- Skip the signalling of the SH explicit scaling list enabled flag for the one slice per picture case. (JVET-R0089, JVET-R0098, JVET-R0202)

# High level control and semantics changes of ALF/SAO

The following aspects are proposed:

- Control ALF and SAO at SPS, PH (on/off control, ALF APS information for ALF) and SH level (on/off control) and remove the slice level ALF parameter adaptation. (JVET-R0160)

- Indication of chroma ALF

  - Use two separate SEs (one for Cb, one for Cr) to replace ph_alf_chroma_idc in PH, and use two separate SEs to replace slice_alf_chroma_idc in SH. (JVET-R0225)

- Indication of CC-ALF

  - Use two separate syntax (alf_ctb_cc_cb_flag and alf_ctb_cc_cr_idx) to replace alf_ctb_cc_cb_idc in CTU level. (JVET-R0225)

- In PH/SH, add a constraint such that if CCALF is disabled in SPS, an ALF_APS cannot contain any CCALF filters. (JVET-R0232)

- Revised semantics

- Update the semantics of ph_alf_enabled_flag equal to 0 to use the wording of "is disabled for all slices" instead of "may be disabled for one, or more, or all slices" (JVET-R0068, JVET-R0251)

ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter is may be disabled for one, or more, or all slices associated with the PH. (JVET-R0068)

ph_alf_enabled_flag being present and equal to 0 specifies that adaptive loop filter may beis disabled for all colour components in one, or more, or all slices associated with the PH. When not present, ph_alf_enabled_flag is inferred to be equal to 0. (JVET-R0251)

- Update the semantics of sps_alf_enabled_flag equal to 1 to use the wording of "may be enabled" instead of "is enabled" (JVET-R0160):

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is may be enabled.

- Update the semantics of sps_alf_enabled_flag equal to 1 to use the wording of "may be enabled" instead of "is enabled" (JVET-R0160):

- Update the semantics of sps_sao_enabled_flag equal to 1 to use the wording of "may be applied" instead of "is applied" (JVET-R0160):

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is may be applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

# APS related aspects

- Move the flag scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS. (JVET-R0064)

- Updating, cross-layer sharing, and decoding order of APSs

  - Update to the content of an ALF APS NAL unit within a PU is allowed. (JVET-R0070)

  - To change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture. (JVET-R0149)

  - Sharing of an APS NAL unit across layers is disallowed. (JVET-R0070)

  - Add the following constraints: When both one or more prefix APS NAL units and one or more suffix APS NAL units are present between two consecutive VCL NAL units in decoding order, the VCL NAL units shall belong to different subpictures and all the suffix APS NAL units shall precede, in decoding order, all the prefix APS NAL units. (JVET-R0149)

  - To constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU. (JVET-R0201)

  - To allow prefix and suffix APS NAL units with particular APS identifier and type to have different content. (JVET-R0201)

  - To constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU. (JVET-R0201)

- Signalling APS information in PH/SH

  - Add additional signalling of alf_data()/scaling_list_data()/lmcs_data() in SH, and add a constraint such that all presence flags of ALF/Scaling list/LMCS in SH shall be 1 when no_aps_constraint_flag is equal to 1. (JVET-R0180)

  - Add a mode of directly inclusion of the APS data structure inside a PH NAL. (JVET-R0273)

- Constratint for APS types:

  - Add a constraint for APS type based on the enabled tools in SPS. If a tool that uses an APS is disabled, then the APS with the corresponding APS type should not be present in a bitstream. (JVET-R0232)

- Constratint for alf_data in ALF APS:

  - Add a constraint to CC-ALF based on sps_ccalf_enabled_flag. When sps_ccalf_enabled_flag is equal to 0, an ALF_APS cannot contain any CCALF filters. (JVET-R0232)

- Remove BitDepth constratint for lmcs_data in LMCS APS (JVET-R0232):

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepth − 214, inclusive.

PH:

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

The value of lmcs_delta_cw_prec_minus1 of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be in the range of 0 to BitDepth – 2, inclusive.

# List of proposals on high level control of LMCS, scaling list, and ALF

JVET-R0051 AHG9: HLS on LMCS and scaling list [S.-T. Hsiang, Z.-Y. Lin, C.-Y. Lai, O. Chubach, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes three high-level syntax modifications related to luma mapping with chroma scaling (LMCS) and scaling list, summarized as follows:

- When sps_lmcs_enabled_flag is equal to 1, a new sequence parameter set (SPS) syntax element sps_lmcs_enabled_slice_present_flag is further signalled to indicate whether slice_lmcs_enabled_flag is present in the slice header.

- When sps_explicit_scaling_list_enabled_flag is equal to 1, a new SPS syntax element sps_explicit_scaling_list_used_slice_present_flag is further signalled to indicate whether slice_explicit_scaling_list_used_flag is present in the slice header.

- Semantic corrections are proposed for the syntax elements sps_lmcs_enabled_flag, ph_lmcs_enabled_flag, and ph_chroma_residual_scale_flag.

JVET-R0063 AHG9: Signalling of LMCS control [L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

This contribution proposes to replace the PH flag ph_lmcs_enabled_flag with a 2-bit ph_lmcs_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the LMCS is used for all slices of a picture, and no signalling of LMCS control flag is needed in the SH. The semantics of slice_lmcs_enabled_flag is revised accordingly. In addition, fixes of the semantics of ph_chroma_residual_scale_flag are proposed to reflect the intention of enabling/disabling chroma residual scaling for a picture or slice.

JVET-R0064 AHG9: Signalling of scaling list control [Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

This contribution proposes the following changes on signalling of scaling list control:

- The PH flag ph_explicit_scaling_list_enabled_flag is replaced with a 2-bit ph_explicit_scaling_list_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the explicit scaling list is used for all slices of a picture, and no scaling list signalling is needed in the SH.

- Move the flag scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS.

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 4 of this contribution belongs to this category.

- Update the semantics of ph_alf_enabled_flag equal to 0 to use the wording of "is disabled for all slices" instead of "may be disabled for one, or more, or all slices".

JVET-R0089 AHG9: On slice level control of LMCS and explicit scaling list [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)]

This contribution proposes two aspects regarding slice level control of LMCS and explicit scaling list:

- Proposal 1: If the current picture consists of only one slice, slice level control flags of LMCS and explicit scaling list (slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag) are not signalled and each flag is inferred to be equal to the picture level flag.

- Proposal 2: Add slice_chroma_residual_scale_flag to slice header to indicate whether chroma residual scaling is enabled for the current slice.

JVET-R0096 AHG9: On signalling of chroma residual scaling [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

In the current VVC draft, if a picture contains multiple slices, the chroma residual scaling cannot be independently controlled for individual slices. In order to allow slice level control of chroma residual scaling, this contribution proposes a slice level chroma residual scaling flag slice_chroma_residual_scale_flag. Two methods are proposed.

- In the first method, picture level ph_chroma_residual_scale_flag is removed and replaced by slice level slice_chroma_residual_scale_flag.

- In the second method, slice level slice_chroma_residual_scale_flag is conditionally signalled based on the value of picture level ph_chroma_residual_scale_flag.

JVET-R0098 AHG9: On Slice Header Signalling of LMCS and Scaling Lists Information [	S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

This contribution proposes modifications to slice header signalling related to LMCS and scaling list. Following is proposed:

- Proposal 1: Not signalling and inferring slice_lmcs_enabled_flag when picture header is included in slice header.

- Proposal 2: Not signalling and inferring slice_explicit_scaling_list_used_flag when picture header is included in slice header.

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Items 4 and 5 of this contribution belong to this category.

This contributions proposes the following aspects regarding repetition and update of non-VCL data units and sharing of APSs across layers:

- VPS, SPS, and DCI NAL units are disallowed to be present in an AU that has no nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive.

- A PU is disallowed to have more than one VPS, SPS, or PPS NAL unit with a particular VPS, SPS, or PPS ID, and is disallowed to have more than one DCI NAL unit.

- A slice unit (SU) is defined as a set of NAL units that are consecutive in decoding order and contain exactly one coded slice and all its associated non-VCL NAL units.

  - Within an SU it is disallowed to have more than one APS NAL unit with a particular APS ID and of particular APS type.

  - Within an SU it is disallowed to have more than one SEI payload with particular type and a particular content.

- Update to the content of an ALF APS NAL unit within a PU is allowed.

- Sharing of an APS NAL unit across layers is disallowed.

- The same tpes of APSs share the same value space for the APS ID, regardelss of whether the APSs are prefix or suffix APS NAL units.

JVET-R0149 AHG9/AHG12: Relaxing an APS constraint [M. M. Hannuksela, M. Homayouni, A. Hallapuro, A. Aminlou (Nokia)]

VVC Draft 8 includes a constraint that the content of all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a picture unit shall have the same content. This contribution asserts that, because of this constraint, ALF needs to be turned off in many applications using adaptive subpicture-based delivery or decoding. To enable the use of ALF in such applications, the contribution proposes:

- To change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture.

- To disallow interleaving of APS NAL units of different subpictures.

JVET-R0201 AHG9: On prefix and suffix APSs [N. Ouedraogo, G. Laroche, P. Onno (Canon)]

In this contribution, it is reported that the random access or the bitstream extraction operations starting at an IRAP picture may lead to the generation of non-conformant bitstream due to the obligation of copying all the needed APS before the IRAP. This is particularly the case when a PU with slices refers to APSs signalled in a previous PU. To solve this issue, it is proposed:

- To constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU.

- To allow prefix and suffix APS NAL units with particular APS identifier and type to have different content.

Second, it is reported that the VVC Draft 8 does not constrain two slices of a PU to refer to two APSs with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type but with different contents when one of the APSs is within a previous PU. In addition to the previous constraints, it is proposed:

- To constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU.

It is reported that it would solve the current random access issue and ease the parallel decoding based on slices.

JVET-R0160 AHG9: High level syntax cleanup for LMCS, ALF and SAO [J. Chen, J. Luo, M. G. Sarwer, Y. Ye, R.-L. Liao (Alibaba)]

This contribution includes two aspects.

- The first aspect of this contribution is to align the high-level control scheme of Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) with that of Luma Mapping with Chroma Scaling (LMCS) by controlling ALF and SAO at SPS, PH and SH level and removing the slice level ALF parameter adaptation.

- The second aspect of this contribution is to fix the semantics of ALF, SAO and LMCS high level control related syntax elements according to the current control scheme.

JVET-R0171 AHG9: Slice-Level Chroma Residual Scaling Flag [K. Naser, E. François, F. Hiron, C. Chevance (InterDigital)]

LMCS can be controlled at both picture header and slice header level. At picture header level, either luma mapping or chroma residual scaling part of LMCS can be deactivated. However, the slice level control can only deactivate LMCS completely, which leads to less control at this level. This contribution proposes adding a slice header flag to deactivate the chroma residual scaling of LMCS.

- Add an SH flag to control chroma residual scaling in slice level.

- Remove the current PH flag and add a slice level flag to control chroma residual scaling in slice level.

JVET-R0180 AHG9: On ALF, LMCS and Scaling List Parameters Signalling [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

In the VVC draft 8 specification, ALF, LMCS and Scaling List parameters are signaled using the adaptation parameters set. This is to have the maximum flexibility where the encoder can signal any of these tools’ parameters at any moment. However, for certain profiles, APS is not signaled. For these profiles, ALF, LMCS and Scaling List shall not be employed. This contribution proposes additional coding mechanism for these parameters such that the tools can still be used with restricted decoders.

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

This contribution proposes 2 modifications:

- Move the LMCS information at an early stage of the slice header as in the picture header

- When picture header signaled inside slice header, avoid APS related information inside slice header.

  - Skip the signalling of the SH LMCS enabled flag for the one slice per picture case

  - Add "When slice headers referring to the PPS contain the PH syntax structure, it is a requirement of bitstream conformance that alf_info_in_ph_flag shall be equal to 1." to the semantics of alf_info_in_ph_flag.

The first aspect (on slice_lmcs_enabled_flag) of item 2 of this contribution belongs to this category.

JVET-R0202 AHG9: Syntax cleanups when Picture Header is in the Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

In order to remove some redundant signaling in the slice header, this contribution proposes some modifications of the VVC specifications when the picture header structure is inside the slice header (e.g. the picture_header_in_slice_header_flag is equal to 1) as only one slice is enabled under such condition.

- In a first aspect, when PH structure is in the SH, the syntax elements slice_lmcs_enabled_flag, slice_explicit_scaling_list_used_flag, num_tiles_in_slice_minus1 and slice_address are not signaled in the slice header and their values are inferred.

- In a second aspect, it is proposed to add an explicit constraint to set separate_colour_plane_flag and subpic_info_present_flag to 0 when PH structure is in the SH.

- In a third aspect, it is proposed to clarify the value of the flags qp_delta_info_in_ph_flag, rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, wp_info_in_ph_flag, alf_info_in_ph_flag when PH structure is in the SH by adding semantics.

- In a fourth aspect, it is proposed to avoid the overriding of the reference picture list in the SH when picture_header_in_slice_header_flag is equal to 1.

The 1st aspect of this contribution on slice_lmcs_enabled_flag, slice_explicit_scaling_list_used_flag belongs to this category.

JVET-R0210 AHG9: Cleanup of Picture Header Syntax Structure in Slice Header [S. Esenlik, B. Wang, A. Kotra, E. Alshina (Huawei)]

In the 17th JVET meeting in Brussels it was decided to adopt the possibility of including the picture header syntax structure in the slice header. The present contribution proposes refinements to the slice header, especially for the case when PH is included in the slice header. Namely it is proposed to condition the inclusion of following three syntax elements in slice header on the presence of picture header syntax structure in slice header:

- num_tiles_in_slice_minus1

- slice_address

- slice_lmcs_enabled_flag.

The 3rd aspect of this contribution belongs to this category.

JVET-R0225 AHG9: On ALF/CC-ALF high level syntax [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

This contribution proposes the following two aspects,

- Use two separate syntax (ph_alf_cb_enabled_flag and ph_alf_cr_enabled_flag) to replace ph_alf_chroma_idc in picture header. And use slice_alf_cb_enabled_flag and slice_alf_cr_enabled_flag to replace slice_alf_chroma_idc in slice header. The separate design of Cb and Cr can make the ALF spec more clear.

- Use two separate syntax (alf_ctb_cc_cb_flag and alf_ctb_cc_cr_idx) to replace alf_ctb_cc_cb_idc in CTU level.

JVET-R0232 AHG9: APS, LMCS, deblocking and PPS constraints [N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Four aspects are proposed in this contribution:

- This contribution proposes to add a constraint for APS type based on the enabled tools in SPS. If a tool, which uses an APS, is disabled then the APS with the corresponding APS type should not be present in a bitstream.

- Secondly, the contribution proposes to add a constraint for CCALF filters in an ALF APS based on CCALF SPS enable flag. If CCALF is disabled in SPS, an ALF_APS cannot contain any CCALF filters.

- Thirdly, the contribution proposes to add constraints for chroma deblocking and chroma LMCS scaling parameter for monochrome content, similar constraints are already in place for other chroma related tools and syntax elements.

- At last, as another option, to decouple constraints in both PPS and APS from SPS, ChromaArrayType and BitDepth constraints are removed from both PPS and APS semantics.

All aspects excluding the 3rd aspect on deblocking filtering belong this this category.

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

This contribution proposes the following changes to the picture header in the VVC specification:

- Rename the syntax elements pic_sign_data_hiding_enabled_flag, sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to ph_sign_data_hiding_enabled_flag, sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag, respectively. (editorial)

- Change the semantics for ph_alf_enabled_flag such that when ph_alf_enabled_flag is present and equal to 0, adaptive loop filter is disabled for all color components in all slices associated with the PH. In the current version of VVC it is stated that when ph_alf_enabled_flag is equal to 0, the adaptive loop filter may be disabled for one, or more, or all slices associated with the PH. (expressed intent)

Item 2 of this contribution belongs to this category.

JVET-R0273 AHG9: APS signalled in picture header [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

This contribution proposes to add a mode of APS signalling inside a PH NAL in a similar manner as picture header signalled inside a VCL NAL. This new mode is indicated by a flag in a picture header followed by the number of APSs included into a PH NAL.