| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0405 |
| --- | --- |

| Title: | Editors input on VVC conformance testing |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Elena Alshina Kei Kawamura Iole Moccagatta Sean McCarthy Karsten Suehring Wade Wan Xiaozhong Xu | Email: | jill.boyce@intel.com elena.alshina@huawei.com ki-kawamura@kddi.com iole.moccagatta@intel.com Sean.McCarthy@dolby.com karsten.suehring@hhi.fraunhofer.de wade.wan@broadcom.com xiaozhongxu@tencent.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is an editor’s input draft of the conformance testing specification for Versatile Video Coding (VVC). Changes are marked with respect to JVET-Q2008. The first section of the document provides information for conformance bitstream submitters, and the second section provides the draft conformance specification.

# General

This document provides a preliminary description of a conformance testing bitstream set and testing process for VVC. See clause 3 for a preliminary list of planned bitstreams and volunteered contributors. It is encouraged for participants to contribute additional bitstreams that test other features and combinations.

# Procedure

## ftp site

An ftp site at ITU-T is used to exchange bitstreams. The site for ftp download of bitstreams is:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

(user id: avguest, password: Avguest201007)

ftp encryption should be set to “Require implicit ftp over TLS”. If using FileZilla, set user id and ftp encryption by configuring site manager as follows:

The site is also accessible by HTTP (without a password) at:

http://wftp3.itu.int/av-arch/jvet-site/bitstream_exchange/VVC

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation will be available at this directory. (See clause 3 for a preliminary list of planned bitstreams.) It includes the list of bitstreams, configuration features and settings, and status of verification.

The ftp site for uploading bitstream files is as follows:

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, password: Avguest201007)

Once a bitstream is uploaded to the dropbox, please send email to jvet-conformance@lists.rwth-aachen.de  to notify the coordinators of its availability.

## Bitstream and associated files

Volunteers should upload the bitstream and associated files in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

*.bit – bitstream as described in section 2.3 (mandatory)

*.txt – description (mandatory)

*.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

*.md5 – MD5sum of the bitstream file (mandatory)

*.opl – output picture log as described in section 2.4 (mandatory)

*.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

All bitstreams are required to use the decoded picture hash SEI message.

All files in the zip archive should be in the top level, without a subfolder.

The .md5 file should contain only the MD5sum value and no additional characters.

The following naming convention is to be used.

Feature_BistreamID_Source_Version

- Feature is summarized in Sec. 3.

- BitstreamID is a single capital letter, to distinguish among multiple bitstreams provided for a particular feature.

- Source is the name of the company who provided the bitstream.

- Version is the version number of the file (starting from 1), in case a bitstream is replaced because of an identified problem or updated for compatibility with a new version of the VTM.

For example, for the feature “feature”, bitstream “A”, from source “company”, in its version “1”, the  archive is named "feature_A_companyA_1.zip", and the files would be named

feature_A_company_1.bit

feature_A_company_1.txt

feature_A_company_1.yuv.md5

feature_A_company_1.md5

feature_A_company_1.opl

feature_A_company_1.cfg

## Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information is to be provided:

- Bitstream file name

- Explanation of bitstream features

- Profile, tier, and level for the bitstream

- Max picture width and height, and any additional picture width and height values used (i.e., for reference picture resample or scalability)

- Picture rate

- VTM decoder release version number able to correctly decode the bitstream with identical output, or description of VTM decoder incompatibility (preferably including a corresponding error tracking issue number.)

- VTM release version number used to generate the bitstream, if applicable

- Contact name and email of bitstream submitter

## Output picture log file for each output layer set (name.opl)

Each output picture log file contains one row for each output picture in the bitstream, in output order.

Each row contains the following information, as CSV.

- PicOrderCntVal

- pic_width_max_in_luma_samples

- pic_height_max_in_luma_samples

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

VTM 8.0 and later versions provide support in the decoder to output an .opl file using a command line argument, -opl. For example: DecoderApp –b feature_A_company_1.bit -opl feature_A_company_1

## Acceptance of bitstreams

The uploaded bitstreams should be verified by at least two organizations. The following aspects should be verified.

- Confirm if bitstream is correctly decoded with perfect match of the .yuv.md5 and .opl files associated with the bitstream

- Confirm if all intended features are included in the bitstream

## Organization of ftp site file directory structure

The ftp site has 2 high level directories, named draft_conformance and under_test.

In the under_test directory, bitstream zip files are placed in a sub-directory corresponding to a VTM decoder release number which can correctly decode the bitstream, or in the VTM-incompatible directory if there is no VTM version that can correctly decode the bitstream.

## Recommendations

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases. Multiple bitstreams should be provided for each feature.

It is recommended to include both low resolution (512x512) and short duration (10 frame) test bitstreams as well as typical resolutions and longer duration test bitstreams for each test category.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

The test bitstreams shall not violate the profile, tier, and level indicated in the bitstream. The minimum level that the bitstream conforms to should be used.

A VTM config file should be included in the .zip file if an unmodified VTM version is used, and the command line used should be included in the .txt file. If a modified version of the VTM is used, the VTM config file should not be included.

# Conformance bitstream list

The conformance bitstreams are listed in the following categories, described below.

- Streams which exercise the range of capabilities of individual coding tools for 10-bit 4:2:0

Table 2: Coding tools for 4:4:4 10b

- Streams which exercise the range of capabilities of individual coding tools for 10-bit 4:4:4

Table 3: High-level syntax features for 4:2:0 10b

- Streams which exercise the range of capabilities of individual high-level syntax features for 10-bit 4:2:0

Table 4: Additional chroma formats and bit depths

- Streams which exercise all the chroma formats and bit depths allowed by the profiles

Table 5: Coding tools sets

- Streams with coding tools incrementally enabled in sets, as indicated by the allowable syntax element values

Table 1: Coding tools for 4:2:0 10b

| Categories | Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- | --- |
| CTU partition | Chroma separate tree | CST | Test CST on and off | Tzu-Der (Peter) Chuang (MediaTek) |
| Transform and quantization | Dependent quantization | DQ |  | Heiko Schwarz (HHI) |
| Intra coding | Cross-component linear model | CCLM | Exercise different variants of down-sample filters Corner cases for liner model parameters | Alexey Filippov (Huawei) |
|  |  |  | Corner cases for enabling conditions of coding structures | Kei Kawamura (KDDI) |
| Transform and quantization | Multiple transform set | MTS | include MTS combinations of (intra, inter): (implicit, none), (explicit, none), (implicit, explicit), (explicit, explicit) | Moonmo Koo (LGE) Mehdi Salehifar (LGE) |
| In-loop filter | Adaptive loop filter | ALF | ALF virtual boundary processing for line buffer reduction | Anand Meher Kotra (Huawei) |
|  |  |  | Exercise clipping values of non-linear ALF | Kyohei Unno (KDDI) |
|  | Adaptive loop filter | ALF | Use multiple APSes | Vadim Seregin (Qualcomm) |
| Inter coding | Affine motion model | AFF | include Affine AMVP and | Chen Huanbang (Huawei) |
|  |  |  | Affine Merge; Control flags |  |
| Inter coding | Subblock-based temporal merging candidates | SbTMVP |  | Na Zhang (Bytedance) |
| Inter coding | Adaptive motion vector resolution | AMVR | include SIF | M. Winken (HHI) |
| Inter coding | Triangular partition mode | TPM | include triangle part. pred. | C.-W Kuo (Panasonic) |
| Inter coding | Bi-directional optical flow | BDOF | Test BDOF with different implicit on/off decision and subblock usages | Ching-Yeh Chen (MediaTek) |
| Inter coding | Combined intra/inter prediction | CIIP | Test CIIP for different sizes and different combining weights | Man-Shu Chiang (MediaTek) |
| Inter coding | Merge with MVD | MMVD |  | Kiho Choi (Samsung) |
| Inter coding | Bi-predictive with CU weights | BCW | Test BCW with different BCW weights | Ching-Yeh Chen (MediaTek) |
| Intra coding | Multi-reference line prediction | MRLP |  | B. Bross (HHI) |
| SCC coding | Intra block copy mode | IBC |  | Xiaozhong Xu (Tencent) |
| Intra coding | Intra sub-partitioning | ISP |  | S. de Luxan Hernandez (HHI) |
| Inter coding | Decoder motion vector refinement | DMVR | Exercise enabling conditions, MV wraparound and MV clip at picture boundary | Semih Esenlik (Huawei) |
|  |  |  | Exercise corner cases of SAD variations | Kyohei Unno (KDDI) |
| Transform and quantization | Sub-block transform | SBT | Different block sizes; control flag | Zhao Yin (Huawei) |
| In-loop filter | Luma mapping with chroma scaling | LMCS | Exercise multiple APSes, slice level LMCS on/off | Taoran Lu (Dolby) |
| Entropy coding | Sign data hiding | SDH |  | Taoran Lu (Dolby) |
| Inter coding | Symmetric motion vector difference | SMVD | Long-term reference handling mvd_l1_zero_flag handling | Chen Huanbang (Huawei) |
| Intra coding | Quantized residual DPCM | DPCM |  | Gordon Clare, Felix Henry (Orange) |
| Intra coding | Matrix based intra prediction | MIP |  | J. Pfaff (HHI) |
| Transform and quantization | Low frequency non-separable transform | LFNST | Various block sizes /shapes Combinations with intra coding tools | Moonmo Koo (LGE) Mehdi Salehifar (LGE) M. Siekmann (HHI) |
| Transform and quantization | Transform tool set | MTS, LFNST | Tool on or off of MTS and LFNST together with implicit or explicit MTS | Moonmo Koo (LGE) Mehdi Salehifar (LGE) |
| Transform and quantization | Joint coding of chrominance residuals | JCCR |  | H. Schwarz (HHI) J. Lainema (Nokia) |
| Inter coding | Temporal motion vector predictor | TMVP |  | D.-Y. Kim (Chips&Media) |
| Inter coding | Motion vector compression | MVCOMP | MV compression for temporal storage (including corner cases) | F. Bossen (Sharp Labs) |
| In-loop filter | Sampled adaptive offset | SAO |  | Kiho Choi (Samsung) |
| Inter coding | Prediction refinement using optical flow | PROF | Various non-translational motion parameters for PROF | Yuwen He (InterDigital) |
| In-loop filter | Deblocking | DBF | Exercise luma adaptive deblock filter and long tap filter | Kenneth Andersson (Ericsson) K. Misra (Sharp Labs) |
| In-loop filter | Chroma deblocking | CDBF | Perform chroma deblocking that includes use of filters of lengths (3,3), (3,1) and (1,3) | K. Misra (Sharp Labs) |
|  |  |  | Deblocking at 4 x 4 grid | Anand Meher Kotra (Huawei) |
| Inter coding | Weighted prediction | WP | Various combinations with other inter tools | Philippe Bordes (InterDigital) |
| Intra coding | Intra prediction | IP | Enable all modes, especially the wide-angle modes. | Alexey Filippov (Huawei) J. Lainema (Nokia) |
| Intra coding | Luma intra prediction mode | MPM | Enable all conditions to generate MPM candidate | Jin Heo(LGE) |
| CTU partition | CTU sizes | CTU | Exercise range of CTU, CU sizes | Chih-Wei Hsu (MediaTek) |
|  |  | CU |  |  |
| CTU partition | Trees and partitioning | BT | Exercise range of sizes and depths of BT, TT, QT | A. Wieckowski (HHI) |
|  |  | TT |  | Chih-Wei Hsu (MediaTek) |
|  |  | QT |  |  |
| CTU partition | Boundary partition | BOUNDARY | Boundary are sizes 8…120 samples, all combinations of QT and BT | Han Gao (Huawei) |
| Transform and quantization | Transform | TRANS | min and max transform | D.-Y. Kim (Chips&Media) |
|  |  |  | min number of entropy coded coeff. |  |
|  |  |  | max number of coeff. |  |
| Transform and quantization | Quantization | QUANT | CU level delta QP CU level Chroma delta QP transform-quant bypass with DB |  |
|  | Scaling list | SCALING | Exercise multiple APSes Quantization Matrices | Philippe de Lagrange (InterDigital) |
| Entropy coding | Entropy coding | ENTROPY | max bins and bits | D.-Y. Kim (Chips&Media) Karl Sharman (Sony) |
|  |  |  | min bits |  |
| Inter coding | All merge modes | MERGE | max number of merge candidates | Wei-Jung Chien (Qualcomm) |
| Intra coding | Position dependent prediction combination (PDPC) | PDPC | Force clipping. Different PU sizes and shapes. | Geert van der Auwera (Qualcomm) |
| Entropy coding | Wavefronts | WPP |  | Ikai Tomohiro (Sharp) |
| Transform and quantization | Lossless and near-lossless, include transform skip | LOSSLESS |  | Tung Nguyen (HHI) |
| Inter coding | Reference picture resizing | RPR | Multiple RPR scale factors RPR cropping window and scaling window offsets | Jiancong Luo (Alibaba) J. Samuelsson (Sharp Labs) |
| In-loop filter | CC-ALF | CCALF | Enable CC-ALF for all CTUs in the bit-stream Slice-by-slice adaptation CTU-by-CTU adaptation Dynamic range exercise (including clip) | Kiran Misra (Sharp) |
| Inter coding | Geometric Partitioning Mode | GPM |  | Semih Esenlik (Huawei), Ru-Ling Liao (Alibaba) |

Table 2: Coding tools for 4:4:4 10b

| Categories | Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- | --- |
| Basic 4:4:4 | 10-bit 4:4:4 | 10b444 | No 4:4:4-specific coding tools enabled |  |
| SCC coding | Adaptive Color Transform | ACT |  | (Xiaoyu Xiu) Kwai |
| SCC coding | Palette mode | PALETTE |  | M. G. Sarwer (Alibaba) |

Table 3: High-level syntax features for 4:2:0 10b

| Feature | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- |
| Filler | FILLER | Filler data NAL unit present. | Gary Sullivan (Microsoft) |
| Decoding Parameter Set | DPS | DPS present | B. Choi (Tencent) |
| Video parameter set | VPS | Single and multiple VPSs | J. Boyce (Intel) |
| Sequence parameter set | SPS | Multiple SPSs | Y.-K. Wang (Bytedance) |
| Video usability information | VUI | Transfer characteristics of PQ and HLG | PQ: T. Lu (Dolby) HLG: S. Nemoto (NHK) |
| Picture parameter set | PPS | Multiple PPSs | Y.-K. Wang (Bytedance) |
| Mixed NUT | MNUT | Mixed NAL unit type within a picture | Biao Wang (Huawei) |
| Extension of parameter set | PSEXT | Extension data in DPS, VPS, SPS, PPS, PH, SH | M. M. Hannuksela (Nokia) |
| Hypothetical reference decoder | HRD | HRD signalling, including HSS, DU, rept., Field(?) | Sachin Deshpande (Sharp Labs) |
| Adaptation parameter set | APSALF | Multiple APSs of each type (ALF) | Vadim Seregin (Qualcomm) |
| Adaptation parameter set | APSLMCS | Multiple APSs of each type (LMCS) | Taoran Lu (Dolby) |
| Adaptation parameter set | APSMULT | Multiple APSs of each type (Scaling list) | Chen-Yen (Jenny) Lai (MediaTek) |
| Random access point | RAP | IRAP functionality | R. Skupin (HHI) |
| Picture output | POUT | Pic output flag, no output pic | Sachin Deshpande (Sharp Labs) |
| Gradual decoder refresh | GDR | GDR | R. Sjöberg (Ericsson) |
| Step-wise temporal sub-layer access | STSA | STSA | R. Sjöberg (Ericsson) |
| Access unit delimiter | AUD | Access unit delimiter | W. Wan (Broadcom) |
| Picture order count | POC | POC derivation, POC reset, POC MSB present | Miska. Hannuksela (Nokia) |
| Tiles | TILE | Pictures partitions in tiles with same and different tile size, slices in tiles, tiles in slices | Miska Hannuksela (Nokia) |
| Slices | SLICE | Raster-scan and rectangular slices | Xiang Ma and Biao Wang (Huawei) |
| Subpictures | SUBPIC | Various number of sub-pictures, sub-pictures of varying sizes, multiple tiles in some sub-pictures, mix of independent and non-independent subpictures, use of subpicture ID in PPS and in PH | Xiang Ma and Biao Wang (Huawei) |
| Picture header and slice header | PHSH | Signalling control of syntax elements in PH and SH Bitstream with both picture header in slice header as well as picture header in its own NALU Collocated picture for TMVP signaled in picture header | Hendry (LG Electronics) J. Samuelson (Sharp Labs) |
| Temporal scalability | TEMPSCAL | Maximum number of temporal layers, varying number of temporal sub-layers | V. Drugeon (Panasonic) |
| Spatial scalability | ILRPL | Inter-layer ref pic list | Xiang Ma and Biao Wang (Huawei) |
| Spatial scalability | SPALSCAL | Different resolution layers, including unusual scaling ratios | Vadim Seregin (Qualcomm) |
| Layered coding with OLS | OLS | Layered coding w/ and w/o inter-layer prediction, with 2 and 3 layers | B. Choi (Tencent) |
| Reference picture lists | RPL | Reference list modification | R. Sjöberg (Ericsson) |
| Long term ref picture | LTRP | LTRP handling | R. Sjöberg (Ericsson) |
| Number of active ref pics | ACTPIC | Using default, using different num of active ref pic in slices | Xiang Ma and Biao Wang (Huawei) |
| Virtual boundaries | VIRTUAL | Loop filters on/off at non-tile/slice boundaries | Jian-Liang Lin (MediaTek) |
| Reference wraparound | WRAP |  | Yong He, Edouard Francois (InterDigital) |
| 360 Video | CUBEMAP | Cube map layout and SEI | Ya-Hsuan . Lee (MediaTek) |
| 360 Video | ERP | Equirectangular projection layout and SEI | Ya-Hsuan Lee (MediaTek) |
| Conformance cropping window | CROP | Use of conformance cropping parameters, varying parameters, large offsets, odd offset values for 4:4:4 Cropping window signaling in both SPS and PPS | Virginie Drugeon (Panasonic) J. Samuelsson (Sharp Labs) |
| Bumping | BUMP | Bumping process | Hendry (LGE) |
| Decoded picture buffer | DPB | Sublayer DPB size signalling | Sachin Deshpande (Sharp Labs) |
| Field pictures | FIELD | frame-field information (SPS) | Kei Kawamura (KDDI) |

Table 4: Additional chroma formats and bit depths

| Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- |
| 8b 4:0:0 | 8b400 | 8-bit 4:0:0 in Main 10 profile |  |
| 8b 4:2:0 | 8b420 | 8-bit 4:2:0 in Main 10 profile |  |
| 8b 4:2:2 | 8b422 | 8-bit 4:2:2 in Main 4:4:4 10 profile |  |
| 8b 4:4:4 | 8b444 | 8-bit 4:4:4 in Main 4:4:4 10 profile |  |
| 10b 4:0:0 | 10b400 | 10-bit 4:0:0 in Main 10 profile |  |
| 10b 4:2:2 | 10b422 | 10-bit 4:2:0 in Main 4:4:4 10 profile |  |

Table 5: Coding tools sets

| Submitter | Xiaozhong Xu (Tencent) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature Name | CodingToolsSets |  |  |  |  |
| Syntax element | SET0 | SET1 | SET2 | SET3 | SET4 |
| slice_type | I | I, P | I | I, P | I, P, B |
| bit_depth_minus8 | 0 | 0 | 0 | 0 | 0 |
| separate_colour_plane_flag | 0 | 0 | 0 | 0 | 0 |
| ref_pic_resampling_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_log2_ctu_size_minus5 | 0 | 0 | 1 | 1 | 1 |
| subpics_present_flag | 0 | 0 | 0 | 0 | 1 |
| bit_depth_minus8 | 0 | 0 | 2 | 2 | 2 |
| sps_weighted_pred_flag | 0 | 0 | 0 | 0 | 1 |
| sps_weighted_bipred_flag | 0 | 0 | 0 | 0 | 1 |
| sps_max_luma_transform_size_64_flag | 0 | 0 | 0 | 0 | 1 |
| sps_sao_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_alf_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_transform_skip_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_bdpcm_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_temporal_mvp_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_sbtmvp_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_amvr_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_bdof_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_bdof_pic_present_flag | 0 | 0 | 0 | 0 | 1 |
| sps_smvd_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_dmvr_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_dmvr_pic_present_flag | 0 | 0 | 0 | 0 | 1 |
| sps_mmvd_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_isp_enabled_flag | 0 | 0 | 1 | 1 | 1 |
| sps_mrl_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_mip_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_mts_enabled_flag | 0 | 0 | 1 | 1 | 1 |
| sps_sbt_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_affine_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_bcw_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_ibc_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_ciip_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_fpel_mmvd_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_geo_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_lmcs_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_lfnst_enabled_flag | 0 | 0 | 0 | 1 | 1 |
| sps_ladf_enabled_flag | 0 | 0 | 0 | 0 | 1 |
| sps_scaling_list_enabled_flag | 0 | 0 | 0 | 0 | 1 |

Conformance specification for ITU-T H.266 | versatile video coding

# 1	Scope

This Recommendation | International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in Rec. ITU-T H.266 | ISO/IEC 23090-3.

# 2	Normative references

## 2.1	General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## 2.2	Identical Recommendations | International Standards

–	None.

## 2.3	Paired Recommendations | International Standards equivalent in technical content

–	Recommendation ITU-T H.266 (in force), Versatile video coding.

–	ISO/IEC 23090-3: in force, Information technology – Coded representation of immersive media– Part 3: Versatile video coding.

–	Recommendation ITU-T H.266.2 (in force), Reference software for ITU-T H.266 Versatile video coding.

–	ISO/IEC 23090-X: in force, Information technology – Coded representation of immersive media – Part X: Reference software for versatile video coding.

# 3	Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 (particularly in clause 3) apply. The following terms are further clarified for purposes herein as follows.

3.1	bitstream: A Rec. ITU-T H.266 | ISO/IEC 23090-3 video bitstream.

3.2	decoder: A Rec. ITU-T H.266 | ISO/IEC 23090-3 video decoder, i.e., an embodiment of the decoding process specified by Rec. ITU-T H.266 | ISO/IEC 23090-3. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	encoder: An embodiment of a process, not specified in this Recommendation | International Standard (except in regard to identification of the reference software encoder), that produces a bitstream.

3.4	reference software decoder: The software decoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-X.

3.5	reference software encoder: The software encoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-X.

# 4	Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms are specified in clause 4 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

# 5	Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

# 6	Conformance testing for ITU-T H.266 | ISO/IEC 23090-3

## 6.1	Introduction

The following clauses specify normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard and the reference software decoder specified in Rec. ITU-T H.266.2 | ISO/IEC 23008-5.

## 6.2	Bitstream conformance

Bitstream conformance for Rec. ITU-T H.266 | ISO/IEC 23090-3 is specified by clause C.4 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

## 6.3	Decoder conformance

Decoder conformance for Rec. ITU-T H.266 | ISO/IEC 23090-3 is specified by clause C.5 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

## 6.4	Procedure to test bitstreams

A bitstream that claims conformance with Rec. ITU-T H.266 | ISO/IEC 23090-3 shall pass the following normative test.

The bitstream shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming to the video layer, i.e., that it does indeed meet all the requirements for the video layer (except Annexes C, D and E) specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 including the hypothetical reference decoder (HRD) conformance (based on Annexes C, D and E). These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

Rec. ITU-T H.266 | ISO/IEC 23090-3 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in Rec. ITU-T H.266 | ISO/IEC 23090-3.

A verifier may not necessarily perform all stages of the decoding process specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## 6.5	Procedure to test decoder conformance

### 6.5.1	Conformance bitstreams

A bitstream has values of general_profile_idc, general_tier_flag, and general_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile, tier, and level is required in Annex A of Rec. ITU-T H.266 | ISO/IEC 23090-3 to properly perform the decoding process.

### 6.5.2	Contents of the bitstream file

The conformance bitstreams are included in this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

- *.bit – bitstream as described in section 2.3 (mandatory)

- *.txt – description (mandatory)

- *.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

- *.md5 – MD5sum of the bitstream file (mandatory)

- *.opl – output picture log as described in section 2.4 (mandatory)

- *.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

### 6.5.3	Requirements on output of the decoding process and timing

Two classes of decoder conformance are specified:

–	output order conformance; and

–	output timing conformance.

The output of the decoding process is specified in clause 8 and Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3.

For output order conformance, it is a requirement that all of the decoded pictures specified for output in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### 6.5.4	Recommendations (informative)

This clause does not form an integral part of this Recommendation | International Standard.

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations specified in Rec. ITU-T H.266 | ISO/IEC 23090-3 that are not an integral part of that Recommendation | International Standard. This clause discusses some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases it is possible to resume decoding at the next start code or slice header. It is recommended that a conforming decoder be able to perform concealment for the coding tree blocks or video packets for which all the coded data has not been received.

### 6.5.5	Static tests for output order conformance

Static tests of a video decoder require testing of the decoded samples. This clause will explain how this test can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process. The test will check that the values of the samples decoded by the decoder under test shall be identical to the values of the samples decoded by the reference decoder. When a hash of the values of the samples of the decoded pictures is attached to the bitstream file, a corresponding hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results.

### 6.5.6	Dynamic tests for output timing conformance

Dynamic tests are applied to check that all the decoded samples are output and that the timing of the output of the decoder's decoded samples conforms to the specification of clause 8 and Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3, and to verify that the HRD models (as specified by the CPB and DPB specification in Annex C of Rec. ITU-T H.266 | ISO/IEC 23090-3) are not violated when the bits of the bitstream are delivered at the proper rate.

The dynamic test is often easier to perform on a complete decoding system, which may include a systems decoder, a video decoder and a display process. It may be possible to record the output of the display process and to check that display order and timing of decoded pictures are correct at the output of the display process. However, since the display process is not within the normative scope of Rec. ITU-T H.266 | ISO/IEC 23090-3, there may be cases where the output of the display process differs in timing or value even though the video decoder is conforming. In this case, the output of the video decoder itself (before the display process) would need to be captured in order to perform the dynamic tests on the video decoder. In particular the output order and timing of the decoded pictures shall be correct.

If buffering period and picture timing SEI messages are included in the test bitstream, HRD conformance shall be verified using the values of nal_initial_cpb_removal_delay, nal_initial_cpb_removal_offset, au_cpb_removal_delay_minus1 and pic_dpb_output_delay that are included in the bitstream.

If buffering period and picture timing SEI messages are not included in the bitstream, the following inferences shall be made to generate the missing parameters:

–	fixed_pic_rate_within_cvs_flag shall be inferred to be equal to 1.

–	low_delay_hrd_flag shall be inferred to be equal to 0.

–	cbr_flag shall be inferred to be equal to 0.

–	The frame rate of the bitstream shall be inferred to be equal to the frame rate value specified in the corresponding table of clause 6.7, where the bitstream is listed. If this is missing, then a frame rate of either 25 or 30000 ÷ 1001 can be inferred.

–	vui_time_scale shall be set equal to 90 000 and the value of vui_num_units_in_tick shall be computed based on frame rate.

–	The bit rate of the bitstream shall be inferred to be equal to the maximum value for the level specified in Table A.1 in Rec. ITU-T H.266 | ISO/IEC 23090-3.

–	CPB and DPB sizes shall be inferred to be equal to the maximum value for the level specified in Table A.1 in Rec. ITU-T H.266 | ISO/IEC 23090-3.

With the above inferences, the HRD shall be operated as follows.

–	The CPB is filled starting at time t = 0, until it is full, before removal of the first access unit. This means that the nal_initial_cpb_removal_delay shall be inferred to be equal to the total CPB buffer size divided by the bit rate divided by 90000 (rounded downwards) and nal_initial_cpb_removal_offset shall be inferred to be equal to zero.

–	The first access unit is removed at time t = nal_initial_cpb_removal_delay ÷ 90000 and subsequent access units are removed at intervals based on the frame distance, i.e., (90000 ÷ vui_num_units_in_tick) or the field distance, i.e., (90000 ÷ num_units_in_tick).

–	Using these inferences, the CPB will not overflow or underflow and the DPB will not overflow.

### 6.5.7	Decoder conformance test of a particular profile, tier, and level

In order for a decoder of a particular profile, tier, and level to claim output order conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3 as specified by this Recommendation | International Standard, the decoder shall successfully pass the static test specified in clause 6.5.5 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile, tier, and level combination.

In order for a decoder of a particular profile, tier, and level to claim output timing conformance to Rec. ITU-T H.266 | ISO/IEC 23090-3 as specified by this Recommendation | International Standard, the decoder shall successfully pass both the static test specified in clause 6.5.5 and the dynamic test specified in clause 6.5.6 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile, tier, and level. Tables 1 through 6 specify the normative test suites for each profile, tier, and level combination. The test suite for a particular profile, tier, and level combination is the list of bitstreams that are marked with an 'X' in the column corresponding to that profile, tier, and level combination. In the column 'Main tier', 'X' indicates that the bitstream is for the Main tier. A decoder conforming to the Main tier shall be capable of decoding the specified bitstreams, among the testing profile-level combination, indicated by 'X' in the 'Main tier' column in the associated table. A decoder conforming to the High tier shall be capable of decoding all the specified bitstreams, among the testing profile-level combinations, in the associated table.

'X' indicates that the bitstream is designed to test both the dynamic and static conformance of the decoder.

The bitstream column specifies the bitstream used for each test.

A decoder that conforms to the Main 10 profile at a specific level shall be capable of decoding the specified bitstreams in Table 1. A decoder that conforms to the Main 4:4:4 10 profile at specific level shall be capable of decoding the specified bitstreams in Table 2. In addition to the bitstreams defined in Table 2, a decoder that conforms to the Main 4:4:4 10 profile shall be capable of decoding the Main 10 profile bitstreams specified in Table 1.

## 6.6	Specification of the test bitstreams

### 6.6.1	General

Some characteristics of each bitstream listed in Table 1 are specified in this clause. In Table 1, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001 and the value "59.94" shall be interpreted as an approximation of an exact value of 60000 ÷ 1001.

### 6.6.2	Test bitstreams – Block structure

#### 6.6.2.1	Test bitstream STRUCT_A

Specification: All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and maximum CU sizes are used.

Functional stage: Test the reconstruction process of slices.

Purpose: Check that the decoder can properly decode I, P and B slices with various CTU and maximum CU sizes.

#### 6.6.2.2	Test bitstream STRUCT_B

Specification: All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and minimum CU sizes are used.

Functional stage: Test the reconstruction process of slices.

Purpose: Check that the decoder can properly decode I, P and B slices with various CTU and minimum CU sizes.

This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The conformance bitstreams needed for this Recommendation are available at the following link: http://handle.itu.int/11.1002/2000/12297. The bitstreams can also be downloaded from the ITU-T Test Signal Database.