| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0410-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on PH and SH syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 11 proposals on PH and SH syntax (the agenda item 4.1.8 in JVET-R0339).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of design questions on PH and SH syntax

- On allowed slice types in a picture

  - Add a new PH flag ph_multiple_slice_types_in_pic_flag (this new flag is skipped when the PPS indicates that there is only one slice), and when this new flag is equal to 0, signal the slice type in the PH (by adding a new PH SE ph_slice_type) and remove the signalling of slice type in the SH. (R0052 methods 1 and 2)

    - In addition, remove the existing PH flag ph_inter_slice_allowed_flag. (R0052 methods 1 and 2)

      - Derive the variables InterSliceAllowed and IntraSliceAllowed based on the values of the PH SEs ph_multiple_slice_types_in_pic_flag (new), ph_intra_slice_allowed_flag (existing), and ph_slice_type (new), and use these variables (instead of using the existing flags) for skipping intra-coding-specific PH SEs and inter-coding-specific PH SEs. (R0052 method 1)

    - In addition, replace the existing PH flag ph_inter_slice_allowed_flag with a 2-bit indicator ph_allowed_slice_types_idc that specifies allowed slice types within a picture as follows (R0052 method 2):

| ph_allowed_slice_types_idc | allowed values of slice types |
| --- | --- |
| 0 | 1, 2 (P, I) |
| 1 | 0, 2 (B, I) |
| 2 | 0, 1 (B, P) |
| 3 | 0, 1, 2 (B, P, I) |

      - Derive the variables BSliceAllowed, PSliceAllowed, and ISliceAllowed based on the values of the PH SEs ph_multiple_slice_types_in_pic_flag (new), ph_slice_type (new), and ph_allowed_slice_types_idc (new), and use these variables (instead of using the existing flags) for skipping intra-coding-specific PH SEs and inter-coding-specific PH SEs.

      - Use the BSliceAllowed and PSliceAllowed for skipping the WP table in the PH.

    - When ph_multiple_slice_types_in_pic_flag (new) is equal to 1:

      - The slice type is signalled in the SH with ue(v) coding (as existing) if IntraSliceAllowed is equal to 1 and with u(1) coding otherwise. (R0052 method 1)

      - If ph_allowed_slice_types_idc is equal to 3, the slice type is signalled in the SH with ue(v) coding (as existing). Otherwise, signal a one-bit slice_type_modified SE. (R0052 method 2)

    - Add a new PPS flag pps_multiple_slice_types_in_pic_flag. (R0052)

  - Add a new PH flag ph_b_slice_allowed_flag (this new flag is skipped when no inter slice is allowed in a picture). (R0061 and R0250)

    - Add a new SPS flag sps_b_slice_allowed_flag. (R0061)

      - When it is known that a CLVS has no B slices (sps_b_slice_allowed_flag is equal to 0), skip the SPS flags sps_weighted_bipred_flag, sps_bdof_enabled_flag, sps_smvd_enabled_flag, sps_dmvr_enabled_flag, sps_bcw_enabled_flag, and sps_gpm_enabled_flag and infer the values. (R0061)

- Skip the signalling of some PH SEs when it is known that a picture has no B slices (R0052 method 2, R0061, R0250, R0324)

  - When it is known that a picture has no B slices, skip the PH SE ph_collocated_from_l0_flag and infer the value. (R0052 method 2, R0061, R0250)

    - Skip them based on a new variable BSliceAllowed that is derived based on the values of ph_multiple_slice_types_in_pic_flag (new), ph_slice_type (new), and ph_allowed_slice_types_idc (new). (R0052 method 2)

    - Skip them based on a new PH flag ph_b_slice_allowed_flag (new). (R0061, R0250)

  - When it is known that a picture has no B slices, skip the PH SEs mvd_l1_zero_flag, ph_disable_bdof_flag, and ph_disable_dmvr_flag and infer the values. (R0052 method 2, R0061, R0250, R0324)

    - Skip them based on a new variable BSliceAllowed that is derived based on the values of PH SEs ph_multiple_slice_types_in_pic_flag (new), ph_slice_type (new), and ph_allowed_slice_types_idc (new). (R0052 method 2)

    - Skip them based on a new PH flag ph_b_slice_allowed_flag (new). (R0061, R0250)

    - Skip them when the following condition (based on existing SEs) is false (R0324):

rpl_info_in_ph_flag  &&  num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1  &&
num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1

  - Use the information that a picture has no B slices to skip the WP table in the PH. (R0052 method 2, R0250)

    - The information is derived based on a new variable BSliceAllowed that is derived based on the values of ph_multiple_slice_types_in_pic_flag (new), ph_slice_type (new), and ph_allowed_slice_types_idc (new). (R0052 method 2)

    - The information is derived based on a new PH flag ph_b_slice_allowed_flag. (R0250)

  - When it is known that a picture has no B slices, skip the WP table SE num_l1_weights and infer the values. (R0052 method 2, R0061)

    - Skip it based on a new variable BSliceAllowed that is derived based on the values of ph_multiple_slice_types_in_pic_flag (new), ph_slice_type (new), and ph_allowed_slice_types_idc (new). (R0052 method 2)

    - Skip it based on a new PH flag ph_b_slice_allowed_flag. (R0061)

- Skip the SH SEI slice_type based on new PH SE(s). (R0052 methods 1 and 2, R0061, R0250)

  - When ph_multiple_slice_types_in_pic_flag (new) is equal to 0, skip the SH SE slice_type and infer it to be equal to ph_slice_type (new). (R0052 methods 1 and 2)

  - When ph_b_slice_allowed_flag (new) and ph_intra_slice_allowed_flag (existing) are both equal to 0, skip the SH SE slice_type and infer it to be equal to 1. (R0061, R0250)

    - Furthermore, skip the SH SEI slice_type when the SH flag picture_header_in_slice_header_flag (existing) is equal to 1. (R0250)

- When the PH flags gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), skip ph_inter_slice_allowed_flag and infer its value to be equal to 0. (R0112)

- Change the semantics of gdr_or_irap_pic_flag as follows (R0112):

gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be a GDR or an IRAP picture.

- Do either of the following two (R0192):

  - When GDR is enabled (i.e., gdr_enabled_flag is equal to 1), a non-zero value is signalled in SPS to be used as an offset to be added into the equation for deriving the POC of the recovery point picture.

  - Change the syntax element recovery_poc_cnt to become recovery_poc_cnt_minus1.

- When gdr_pic_flag is equal to 1, skip the PH SE ph_inter_slice_allowed_flag infer it to be equal to 1. (R0198)

- When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0 (i.e., the picture is an IRAP picture), add a new PH flag idr_pic_flag. (R0198)

  - When sps_idr_rpl_present_flag is equal to 0 and idr_pic_flag is equal to 1, RPL signalling is skipped in the PH, even when the value of rpl_info_in_ph_flag is equal to 1. (R0198)

- Rename the syntax elements pic_sign_data_hiding_enabled_flag, sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to ph_sign_data_hiding_enabled_flag, sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag, respectively. (editorial) (R0251)

- Byte align before entry point offset fields for easier updating during encoding, amd separate entry point offsets by one bit equal to one and divide into upper and lower bits to avoid start code emulation without the need for an emulation prevention byte. (R0165)

- Move the entry point syntax to the end of the slice header, i.e. behind the slice header extension. (R0298)

- Move the slice_lmcs_enabled_flag to an earlier position, immediately after the ALF SEs in the SH, similarly as in the PH (R0200).

# List of proposals on PH and SH syntax (11)

JVET-R0052 AHG9: Overhead reduction for picture header and slice header [S.-T. Hsiang, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to modify the high-level syntax in the VVC Draft 8 for a further overhead reduction in the picture header (PH) and the slice header. The proposed methods signal a new syntax element ph_multiple_slice_types_in_pic_flag in the PH for specifying whether more than one slice types may be present in the coded picture. When ph_multiple_slice_types_in_pic_flag is equal to 0, a new syntax element ph_slice_type is further signalled to specify the slice type for all slices of the picture. The syntax element slice_type in the slice header is not coded and is inferred to be equal to the value of ph_slice_type.

In Method 1, the possible presence of inter/intra slices is derived from related syntax elements signalled in the PH. It is more efficient in signalling the selected slice type for slices of the picture when only one slice type is allowed to be present in the picture.

In Method 2, the possible presence of each specific slice type is derived from related syntax elements signalled in the PH. It can further reduce overhead by skipping coding the PH syntax elements only applied to the B slice type when B slices are not allowed in the picture.

In addition, a new syntax element pps_multiple_slice_types_in_pic_flag can be further signalled in the PPS as the enabling flag for the methods.

JVET-R0061 AHG9: On allowed slice types in a picture [L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

This contribution proposes to add a SPS flag, sps_b_slice_allowed_flag, indicating permission of B slices, to better support applications with low-complexity encoders and reduce overhead cost. On top of that, a PH flag, ph_b_slice_allowed_flag, is further added to specify whether B slice is allowed in a picture. In addition, the following changes are proposed accordingly:

- When sps_b_slice_allowed_flag is equal to 0, the syntax elements sps_weighted_bipred_flag, sps_bdof_enabled_flag, sps_smvd_enabled_flag, sps_dmvr_enabled_flag, sps_bcw_enabled_flag, and sps_gpm_enabled_flag are not signalled and their values are inferred.

- When ph_b_slice_allowed_flag is equal to 0, the syntax elements ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and num_l1_weights are not signalled and their values are inferred, and, consequently, weighted prediction parameters for reference picture list 1 are not signalled in the PH or SHs of the picture.

- When both ph_b_slice_allowed_flag and ph_intra_slice_allowed_flag are equal to 0, the syntax element slice_type is not signalled and the value is inferred to be equal to 1.

JVET-R0250 AHG9: On B-slice signalling in the PH and derivation of slice_type [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

This contribution proposes the following two changes to the VVC specification:

- Signal a flag ph_inter_B_slice_allowed_flag in the PH when ph_inter_slice_allowed_flag is equal to 1 and condition the presence of ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and pred_weight_table( ) on ph_inter_B_slice_allowed_flag.

- Use the proposed ph_inter_B_slice_allowed_flag to increase the number of cases when the value of slice_type is inferred without being signalled.

It is claimed that the proposed ph_inter_B_slice_allowed_flag will control the presence of the syntax elements in the PH that are used only for bi-prediction in a similar way to how it is done in the slice header, and allows preventing signalling of syntax elements that are used only for bi-prediction when there are no B slices in the picture associated with the PH. It is claimed that signalling of syntax elements that has no meaning should in general be avoided.

It is further claimed that the ph_inter_B_slice_allowed_flag provides an easy way to check whether a picture is a B-picture or not. It is asserted that this would otherwise require checking whether there is any slice in the picture with slice_type equal to B.

It is also claimed that the ph_inter_B_slice_allowed_flag provides some bit savings.

JVET-R0112 AHG9: On picture header [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution includes a proposal for a modification in the picture header. It is asserted that the proposed modification improves clarity, removes redundancy and reduces the risk of incorrect (or confusing) values being signalled for IRAP pictures.

In summary, it is proposed that the flag ph_inter_slice_allowed_flag is not signalled for IRAP pictures. Instead, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0 for IRAP pictures.

JVET-R0192 AHG9: On signalling recovery point picture [Hendry (LGE)]

It is asserted that the signalling of recovery_poc_cnt is not optimal and can be made more efficient based on the following observations:

- The value of recovery_poc_cnt shall be positive and should be greater than 0 because the recovery point picture associated with a GDR picture cannot be the GDR picture itself. However, the value range of recovery_poc_cnt is specified to range from 0 to MaxPicOrderCntLsb – 1.

- In practice, the POC difference between any pair of recovery point picture and its associated GDR picture may be uniform.

Based on the observations, this contribution proposed two options as follows:

Option 1: When GDR is enabled (i.e., gdr_enabled_flag is equal to 1), a non-zero value is signalled in SPS to be used as an offset to be added into the equation for deriving the POC of the recovery point picture.

Option 2: Change the syntax element recovery_poc_cnt to become recovery_poc_cnt_minus1.

It is remarked that option 1 is preferred as this would give more bit saving in common use case scenario of the application of GDR.

JVET-R0198 AHG9: On signalling of IDR or GDR picture flag in picture header [J. Nam, H. Jang, J. Lim, Hendry, S. Kim (LGE)]

In this contribution, the following assertions were made:

- A GDR picture should have at least one slice that is not intra coded, otherwise, it should not be labelled as a GDR but an IRAP instead.

- In the current signalling mechanism for reference picture list (RPL) in picture header (PH), RPL still needs to be signalled in PH even when the picture associated with the picture header is IDR picture and the value of sps_idr_rpl_present_flag is equal to 0. This seems redundant and should be avoided if possible.

This contribution proposed the following changes related to the signalling of syntax elements in PH:

- When gdr_pic_flag is equal to 1, the syntax element ph_inter_slice_allowed_flag is not present, and its value is inferred to be equal to 1.

- When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0, a new flag called idr_pic_flag is proposed to specify whether the picture associated with the picture header is an IDR picture. When sps_idr_rpl_present_flag is equal to 0 and idr_pic_flag is equal to 1, RPL signalling is not present in the PH, even when the value of rpl_info_in_ph_flag is equal to 1.

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 1 of this contribution belongs to this category.

This contribution proposes the following changes to the picture header in the VVC specification:

- Rename the syntax elements pic_sign_data_hiding_enabled_flag, sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag and sps_prof_pic_present_flag to ph_sign_data_hiding_enabled_flag, sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag, respectively. (editorial)

JVET-R0324 AHG9: On syntax signalling conditions in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

In this proposal, it is proposed to signal mvd_l1_zero_flag, ph_disable_bdof_flag and ph_disable_dmvr_flag depending on the whether the coded slices associated with the PH are bi-predictive, to prevent redundant signaling due to non-applicable syntax values in the picture header.

JVET-R0165 AHG12: Entry point offsets avoiding start code emulation prevention byte [K. Abe, T. Toma, V. Drugeon (Panasonic)]

This contribution proposes to modify the syntax structure for signalling of entry point offsets. An encoder needs to know the number of bits in each subset of the current slice in order to write entry point offsets into the slice header. Some real time encoders achieve it by updating this syntax field after finishing encoding the slice.

This contribution proposes two items to simplify this encoding operation: (1) byte align before entry point offset field for easier updating during encoding; (2) separate entry point offsets by one bit equal to one and divide into upper and lower bits to avoid start code emulation without the need for an emulation prevention byte.

JVET-R0298 AHG9: On order of syntax elements for entry point offsets [K. Suehring (HHI), R. Foray (Allegro DVT)]

This contribution proposed to move the entry point syntax to the end of the slice header, i.e. behind the slice header extension. This ordering was suggested in bug #736 on the JVET bug tracker.

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

Item 1 of this contribution belongs to this category.

- Move the LMCS information at an early stage of the slice header as in the picture header.