| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0411-v3 |
| --- | --- |

| Title: | AHG9: A Summary of Proposals Related to Reference Picture Lists, Weighted Prediction, and Collocated Picture Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of proposals on reference picture lists, weighted prediction and collocated picture signalling.

Nine proposals from the category “4.1.10 RPL, WP, and Collocated Picture Signalling” listed in JVET-R0339 are included in this summary. In v2 some of the questions below are updated based on feedback received. In v3, questions are grouped and updated further and one document is removed due to its movement to another category in JVET-R0338-v9 revision.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner.

# List of design questions

Related to RPL signalling

- Modify the current RPL active entries override signalling/ derivation?

  - Allow signalling the RPL active entries override information in picture header? (JVET-R0102) or Derive the number of RPL active entries in picture header to be default active entries (JVET-R0059 item 1)

  - Modify the RPL override signaling condition in slice header?:

    - Update if conditioning in the slice header to be based on using !rpl_info_in_ph – consistent with other cases where information is either in slice header or picture header. Depends on having number of active entries in picture header (see 1.a above) (JVET-R0059, JVET-R0102)

    - Remove only rpl_info_in_ph_flag  | |  ( ( nal_unit_type  !=  IDR_W_RADL  &&  nal_unit_type  !=IDR_N_LP )  | |  sps_idr_rpl_present_flag ) )  && conditioning in the slice header to not signal override flag for I-slices (JVET-R0277 item 1).

    - Remove only  rpl_info_in_ph_flag  | |  conditioning in the slice header (JVET-R0255 item 2)

- Skip the signalling of the ltrp_in_header_flag[ listIdx ][ rplsIdx ] syntax element when the ref_pic_list_struct( listIdx, rplsIdx ) syntax strucure is directly included in the PH or SH instead of in the SPS? (JVET-R0059)

- Add and remove RPL constraints related to num_ref_entires and NumRefIdxActive as follows (strikeout means removal)? (JVET-R0138)

The value of num_ref_idx_active_minus1[ i ] shall be less than num_ref_entries[ i ][ RplsIdx[ i ] ], inclusive.

When the current slice is a P slice, the value of NumRefIdxActive[ 0 ] shall be greater than 0.

When the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0.

When slice_type is equal to P or B, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] shall be equal to greater than 0.

When slice_type is equal to B, num_ref_entries[ 1 ][ RplsIdx[1 ] ] shall be equal to greater than 0.

When nal_unit_type is equal to IDR_W_RADL or IDR_N_LP, num_ref_entries[ i ][ RplsIdx[ i ] ] shall be equal to 0 for each i equal to 0 or 1.

- Add the following RPL related constraints? (JVET-R0253 item 2, 3)

  - When ph_inter_slice_allowed_flag is equal to 1 and rpl_info_in_ph is equal to 1, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] shall be greater than 0.

  - In JVET-R0250 it is proposed to add a ph_inter_B_slice_allowed_flag. If such a flag is adopted, add following constraint?:When ph_inter_B_slice_allowed_flag is equal to 1 and rpl_info_in_ph is equal to 1, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] shall be greater than 0.

- Conditionally signal inter_layer_ref_pics_present_flag and infer it to be equal to 0 when not signalled? (JVET-R0156 item 2, JVET-R0205)

  - Condition is: sps_ptl_dpb_hrd_params_present_flag is equal to 0 (JVET-R0156 item 2)

  - Condition is: sps_video_parameter_set_id is greater than 0 (JVET-R0205)

Or modify existing constraint: When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag shall is inferred to be equal to 0.

- Change the reference picture list structure semantics by replacing the parameters ph_rpl_idx[ listIdx ] and slice_rpl_idx [ listIdx ] with rpl_idx[ listIdx ]? (JVET-R0255 item 1)

The ref_pic_list_struct( listIdx, rplsIdx ) syntax structure may be present in an SPS, in a PH syntax structure, or in a slice header. Depending on whether the syntax structure is included in an SPS, a PH syntax structure, or a slice header, the following applies:

–	If present in a PH syntax structure or a slice header, the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies reference picture list listIdx of the current picture (the picture containing the slice).

–	Otherwise (present in an SPS), the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies a candidate for reference picture list listIdx, and the term "the current picture" in the semantics specified in the remainder of this clause refers to each picture that 1) has a PH syntax structure containing ph_rpl_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS or one or more slices containing slice_rpl_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS, and 2) is in a CVS that refers to the SPS.

- Modify the inference of rpl_idx [ i ] when not present: if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0. (JVET-R0255 item 3)?

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( num_ref_pic_lists_in_sps[ i ] ) ) bits. When not present, the value of rpl_idx[ i ] is inferred to be equal to 0. The value of rpl_idx[ i ] shall be in the range of 0 to num_ref_pic_lists_in_sps[ i ] − 1, inclusive. When rpl_sps_flag[ i ] is equal to 1 and num_ref_pic_lists_in_sps[ i ] is equal to 1, the value of rpl_idx[ i ] is inferred to be equal to 0. When not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0.

- Repurpose rpl1_idx_present_flag to indicate the presence RefPicList1 related syntax elements and not only the presence of rpl_sps_flag[1] and rpl_idx[1]? (JVET-R0277 item 3)

- Add a constraint for reference pictures to be the same for all slices in a picture: Set of reference pictures consisting of RefPicList[ 0 ] and RefPicList[ 1 ] entries shall be the same for all VCL NALs of a picture.? (JVET-R0277 item 4)

Related to Weighted Prediction

- In the weighted prediction syntax pred_weight_table( ), remove the syntax elements num_l0_weights and num_l1_weights by instead deriving and using in its place number of active entries - Depends on having number of active entries in picture header (see 1.a above)? (JVET-R0059- item 1b, JVET-R0102)

Or: Condition the presence of num_l1_weights, and derivation of NumWeightsL1 by repurposing rpl1_present_flag when using it as proposed in #7 above. (JVET-R0277 item 3)

Related to collocated picture signalling

- In the syntax condition for signalling of the collocated picture in the PH, replace "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ]" and "num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]" with "NumRefIdxActive[ 0 ]" and " NumRefIdxActive[ 1 ]" respectively - Depends on having number of active entries in picture header (see 1.a above)? (JVET-R0059)

When ph_collocated_from_l0_flag is equal to 1, ph_collocated_ref_idx refers to an entry in reference picture list 0, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] NumRefIdxActive[ 0 ] − 1, inclusive.

When ph_collocated_from_l0_flag is equal to 0, ph_collocated_ref_idx refers to an entry in reference picture list 1, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] NumRefIdxActive[ 1 ] − 1, inclusive.

- Replace the existing constraint on the value of ph_temporal_mvp_enabled_flag with a NOTE, with the addition of the scaling window offsets to be also the same? (JVET-R0059)

ph_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If ph_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (ph_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0. When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

NOTE – The value of ph_temporal_mvp_enabled_flag has to be equal to 0 when no reference picture in the DPB has the same spatial resolution and scaling window offsets as the current picture.

- Modify the condition for signaling ph_collocated_ref_idx as follows - Depends on having number of active entries in picture header (see 1.a above)? (JVET-R0059)

if( ( ph_collocated_from_l0_flag  &&  NumRefIdxActive[ 0 ] > 1
	num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 )  | |
	 ( !ph_collocated_from_l0_flag  &&  NumRefIdxActive[ 1 ] > 1
	num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) )

- Modify the existing constraint in the slice header semantics on the collocated picture by only keeping the 0-valued-RprConstraintsActive[ ][ ] aspect as follows? (JVET-R0059)

It is a requirement of bitstream conformance that the values of pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture referred to by slice_collocated_ref_idx shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture, and RprConstraintsActive[ slice_collocated_from_l0_flag ? 0 : 1 ][ slice_collocated_ref_idx ] shall be equal to 0.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution and scaling window offsets as the current picture.

- Slice_collocated_from_l0_flag for p slices:

  - Add following constraint? (JVET-R0059, JVET-R0253 item 1)

When ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1 and there is at least one P slice in the picture, the value of ph_collocated_from_l0_flag shall be equal to 1? (JVET-R0059) OR

It is a requirement of bitstream conformance that slice_collocated_from_l0_flag shall be equal to 1 when slice_type is equal to P (JVET-R0253 item 1).

  - When not present, infer slice_collocated_from_l0_flag to be equal to 1 for P-slices? (JVET-R0277 item 2)

Related to ph_temporal_mvp_enabled_flag:

- Modify the existing constraint in the slice header semantics on the collocated picture by only keeping the 0-valued-RprConstraintsActive[ ][ ] aspect as follows? (JVET-R0059)

It is a requirement of bitstream conformance that the values of pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture referred to by slice_collocated_ref_idx shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture, and RprConstraintsActive[ slice_collocated_from_l0_flag ? 0 : 1 ][ slice_collocated_ref_idx ] shall be equal to 0.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution and scaling window offsets as the current picture.

- Constraint on ph_temporal_mvp_enabled_flag:

  - Replace the existing constraint on the value of ph_temporal_mvp_enabled_flag with a NOTE, with the addition of the scaling window offsets to be also the same? (JVET-R0059)

ph_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If ph_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (ph_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0. When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

NOTE – The value of ph_temporal_mvp_enabled_flag has to be equal to 0 when no reference picture in the DPB has the same spatial resolution and scaling window offsets as the current picture.

  - Add a constraint considering the offsets that are applied to the picture size for scaling ratio calculation in the bitstream conformance of ph_temporal_mvp_enabled_flag? (JVET-R0323 items 1)

When no reference picture in the DPB has the same spatial resolution and the same offsets that are applied to the picture size for scaling ratio calculation as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

- Add a constraint considering the offsets that are applied to the picture size for scaling ratio calculation in the bitstream conformance of ph_temporal_mvp_enabled_flag? (JVET-R0323 items 1)

- When no reference picture in the DPB has the same spatial resolution and the same offsets that are applied to the picture size for scaling ratio calculation as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

- Add a bitstream conformance on ph_temporal_mvp_enabled_flag to force its value to 0 when there is no common reference picture existing in the reference picture lists of the all the slices associated with the coded picture? (JVET-R0323 item 2)

When there is no common reference picture existing among all the slices associated with the PH, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

# List of proposals related to Reference Picture Lists, Weighted Prediction, and Collocated Picture Signalling

Nine proposals from the category “4.1.10 RPL, WP, and Collocated Picture Signalling” listed in JVET-R0339 are included in this summary. Complete list of covered items is below.

JVET-R0059 AHG9: Cleanups on RPL and related signalling [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang, J. Xu (Bytedance)]

This contribution proposes the following changes on RPL and related signalling:

- When the RPL information is signalled in the PH instead of the SH, derive the numbers of active entries to be equal to the default numbers of active entries signalled in the PPS, and update the SH syntax accordingly. Consequently, the following changes are also proposed:

  - In the syntax condition for signalling of the collocated picture in the PH, replace "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ]" and " num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]" with "NumRefIdxActive[ 0 ]" and " NumRefIdxActive[ 1 ]" respectively.

  - In the weighted prediction (WP) syntax pred_weight_table( ), remove the syntax elements num_l0_weights and num_l1_weights.

- In the reference picture list structure syntax, skip the signalling of the ltrp_in_header_flag[ listIdx ][ rplsIdx ] syntax element when the syntax strucure is directly included in the PH or SH instead of in the SPS.

- It is required that, when ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1 and there is at least one P slice in the picture, the value of ph_collocated_from_l0_flag shall be equal to 1.

- Replace the existing constraint on the value of ph_temporal_mvp_enabled_flag with a NOTE, with the addition of the scaling window offsets to be also the same.

- Simplify the existing constraint in the slice header semantics on the collocated picture by just keeping the 0-valued-RprConstraintsActive[ ][ ] aspect, which is asserted to cover all aspects needed.

JVET-R0102 AHG9: On Reference Picture List Override Signalling [S. Deshpande, T. Chujoh, T. Ikai, J. Samuelsson, A. Segall (Sharp)]

It is proposed to allow signalling of reference picture list override information in picture header. A corresponding modification is proposed to the signalling in slice header and weighted prediction table. It is asserted that since the reference picture list information can be signalled at the picture header, allowing signalling the reference picture list override information at the picture header makes a consistent design.

JVET-R0138 AHG9: Some constraints of num_ref_entries [T. Chujoh, T. Ikai (Sharp)]

The syntax element num_ref_entries[ listIdx ][ rplsIdx ] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. Multiple variables related to reference picture lists are derived from this syntax element. In current VVC Draft 8, there are some problems depending on the value of num_ref_entries. If the value of num_ref_entries is set to 0, the reference picture lists can not be derived when slice_type is equal to P or B. According to the discussion at the previous meetings, in order to generate a mixed NAL picture, it seems to be necessary to set num_ref_entries to zero when nal_unit_type is equal to IDR_W_RADL or IDR_N_LP. In this contribution, some constraints of num_ref_entries have been proposed.

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 2 of this contribution belongs to this category.

Item 2: When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not signaled but inferred to be equal to 0.

JVET-R0205 AHG9: On signalling of inter_layer_ref_pics_present_flag [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

Conditional signalling of inter_layer_ref_pics_present_flag depending on sps_video_parameter_set_id is proposed. When sps_video_parameter_set_id is equal to 0 (i.e., the CVS contains only one layer), inter_layer_ref_pics_present_flag is not signalled in the SPS and the value of the flag is inferred to be equal to 0.

JVET-R0253 AHG9: Three restrictions when RPL is present in PH [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

This contribution proposes the following changes to the VVC specification:

- Restrict that slice_collocated_from_l0_flag shall be equal to 1 when slice_type is equal to P.

- Restrict that when ph_inter_slice_allowed_flag is equal to 1 and rpl_info_in_ph is equal to 1, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] shall be greater than 0.

- In JVET-R0250 it is proposed to add a ph_inter_B_slice_allowed_flag. If such a flag is adopted, it is further proposed to add a restriction that when ph_inter_B_slice_allowed_flag is equal to 1 and rpl_info_in_ph is equal to 1, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] shall be greater than 0.

It is claimed that without the first restriction, the specification is broken since it is not defined how to derive the temporal motion vector prediction from reference picture list 1 for a P slice.

It is further claimed that without the second and third restrictions it would be possible to have a slice with slice_type equal to P without any entries in L0 or a slice with slice_type equal to B without any entries in L1. This is asserted to be an unwanted behaviour.

JVET-R0255 AHG9: Fixes related to RPL [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

This contribution proposes the following changes to RPL in the VVC specification:

- Change the reference picture list structure semantics by replacing the non-defined parameters ph_rpl_idx[ listIdx ] and slice_rpl_idx [ listIdx ] with rpl_idx[ listIdx ]. (claimed cleanup)

- Remove conditioning the presence of num_ref_idx_active_override_flag in the slice header on the value of the rpl_info_in_ph_flag syntax element. (claimed expressed intent)

- Fix the inference of rpl_idx [ i ] such when not present, if rpl_sps_flag[ i ] is equal to 1 and rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ i ] is inferred to be equal to 0. It is asserted that the inference of rpl_idx[ i ] is ambiguous in the current VVC specification. (claimed expressed intent)

JVET-R0277 AHG9: On reference picture list signalling [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

This contribution proposes to modify reference picture list signalling as follows:

- Remove reference index override flag signalling for I-slices

- When not present, infer slice_collocated_from_l0_flag to be equal to 1 for P-slices

- Repurpose rpl1_idx_present_flag to indicate the presence RefPicList1 related syntax elements and not only the presence of rpl_sps_flag[1] and rpl_idx[1].

- Add a constraint for reference pictures to be the same for all slices in a picture

JVET-R0323 AHG9: On TMVP enabling flag in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

There are two methods proposed in this proposal to modify the syntax element ph_temporal_mvp_enabled_flag in picture header. First, it is proposed to consider the offsets that are applied to the picture size for scaling ratio calculation in the bitstream conformance of ph_temporal_mvp_enabled_flag because the offsets also affect the enabling of TMVP.

Second, in current VVC, there is a requirement of bitstream conformance that the picture referred to by slice_collocated_ref_idx shall be the same for all slices of a coded picture. But, when the coded picture is coded in multiple slices and there is no common reference picture existing among all the slices, this bitstream conformance has no chance to be met. Therefore, it is proposed to have another bitstream conformance on ph_temporal_mvp_enabled_flag to force its value to 0 when there is no common reference picture existing in the reference picture lists of the all the slices associated with the coded picture.

----