| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0412-WD |
| --- | --- |

| Title: | On maximum palette size and palette predictor size under local dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Wei-Jung Chien, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

7.3.10.5	 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER && ( ( cbWidth * cbHeight ) > ( treeType != DUAL_TREE_CHROMA ? 16 : 16 * SubWidthC * SubHeightC ) ) ) |  |
| pred_mode_plt_flag | ae(v) |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType, modeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_transposed[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) && !cu_act_enabled_flag ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, treeType, modeType ) |  |
| else if( !pred_mode_plt_flag ) { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( cbWidth / SubWidthC <= MaxTsSize && cbHeight / SubHeightC <= MaxTsSize && sps_bdpcm_enabled_flag ) { |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| } else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# 7.3.10.6	Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, treeType, modeType) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE && modeType = = MODE_TYPE_ALL ) ? 31 : 15 |  |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize[ startComp ] && !palettePredictionFinished && NumPredictedPaletteEntries < maxNumPaletteEntries; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |

8.4.5.3	Decoding process for palette mode

Inputs to this process are:

- a location ( xCbComp, yCbComp ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current coding block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of treeType, the variables startComp, numComps and maxNumPalettePredictorSize are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(430)

numComps = ChromaArrayType = = 0 ? 1 : 3		(431)

maxNumPalettePredictorSize = 63		(432)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(433)

numComps = 1		(434)

maxNumPalettePredictorSize = 31		(435)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(436)

numComps = 2		(437)

maxNumPalettePredictorSize = 31		(438)

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is greater than 0 and startComp is equal to 0, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

- Otherwise, nSubWidth is set to 1 and nSubHeight is set to 1.

The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCbComp, yCbComp ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:

- The variables xL, yL, xCbL, and yCbL are derived as follows:

xL = x * nSubWidth		(439)

yL = y * nSubHeight		(440)

xCbL = xCbComp * nSubWidth		(441)

yCbL = yCbComp * nSubHeight		(442)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCbL + xL ][ yCbL + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCbL + xL ][ yCbL + yL ] ]	(443)

- Otherwise (bIsEscapeSample is equal to 1), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( QpPrimeTsMin, Qp′Y )		(444)

- Otherwise, if cIdx is equal to 1,

qP = Max( QpPrimeTsMin, Qp′Cb )		(445)

- Otherwise (cIdx is equal to 2),

qP = Max( QpPrimeTsMin, Qp′Cr )		(446)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCbL + xL ][ yCbL + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(447)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, tmpVal )	(448)

The variable localDualTree is derived as follows:

localDualTree = treeType  !=  SINGLE_TREE  &&  
( slice_type  !=  I  | |  ( slice_type = = I  &&  qtbtt_dual_tree_intra_flag = = 0 ) ) ? 1 : 0	(449)

When localDualTree is equal to 1, the following applies:

- If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(450)

CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(451)

- Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(452)

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = 31		(456)

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 2;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ )
	for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )
		newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize[ startComp ]
for( i = 0; i < PredictorPaletteSize[ startComp ]  &&  newPredictorPaletteSize < maxNumPalettePredictorSize; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )	(457)
			newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ cIdx ][ i ]
		newPredictorPaletteSize++
	}
for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]
PredictorPaletteSize[ startComp ] = newPredictorPaletteSize

When qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies:

PredictorPaletteSize[ 1 ] = newPredictorPaletteSize		(458)

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to maxNumPalettePredictorSize, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).