| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0412-v1 |
| --- | --- |

| Title: | AHG11: On maximum palette size and palette predictor size under local dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Wei-Jung Chien, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reduce maximum palette and palette predictor size under local dual tree, to be unified with the design in dual tree slices and to reduce the pipeline latency.

The results show very minor impact in terms of coding efficiency on top of the palette mode in VVC draft 8:

Class F: 0.00% AI, 0.00% RA and x.xx% LDB

Class TGM: 0.00% AI, 0.00% RA, and x.xx% LDB

# Proposal

In VVC draft 8, the palette size and palette predictor size in dual tree slices are reducd to half the sizes in the single trees slice [1] due to the fact that the complexity of the palette generation and predictor update in separate tree is roughly doubled from the case of single tree. However, such change was not applied in the local dual tree, where luma and chroma palette are also generated separately and the palette predictor is updated in both luma and chroma local dual tree, making palette mode under local dual tree the worst case in terms of pipeline latency among palette coded blocks.

In this contribution, we propose to reduce the palette size to 15 and palette predictor size to 31 (same size as in dual tree slices) under local dual tree, to address the complexity issue identified in last meeting.

# Results

## The proposed methods are evaluated for YUV420 sequences according to the SDR common test condition [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. Besides YUV420.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -1.24% | -0.43% | -0.47% | 100% | 91% | 0.00% | 0.00% | 0.00% | 98% | 101% |
| TGM | -6.50% | -5.18% | -4.94% | 99% | 93% | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class F | -1.29% | -0.47% | -0.55% | 103% | 98% | 0.00% | 0.00% | 0.01% | 98% | 98% |
| TGM | -3.78% | -2.43% | -2.48% | 100% | 96% | 0.00% | 0.02% | -0.06% | 99% | 101% |

# References

- M. G. Sarwer, Y. Ye, J. Luo, R. -L. Liao, “CE2-related: On maximum palette size of VVC”, JVET-Q0291.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).