| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0415 |
| --- | --- |

| Title: | AHG12: A summary of proposals on subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd San Diego, CA 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution intended to provide a summary of the proposals on subpictures submitted to the 18th JVET meeting.

# List of proposed items

- Condition sps_independent_subpics_flag on "sps_num_subpics_minus1 > 0". (JVET-R0071 #1, JVET-R0156 #4, JVET-R0284 #1)

- When sps_independent_subpics_flag is not present, it is inferred to be equal to 1 (JVET-R0071 #1, JVET-R0156 #4, JVET-R0136 #1)

- Change the inference of subpic_treated_as_pic_flag[ i ] when not present.

  - Infer it to be equal to 1 (JVET-R0071 #2)

  - Keep the current inference (JVET-R0284 #1

- Change the inference of loop_filter_across_subpic_enabled_pic_flag[ i ] when not present.

  - Infer it to be equal to 0 (JVET-R0071 #3)

- Infer single_slice_per_subpic_flag to be equal to 1 when no_pic_partition_flag is equal to 1 (JVET-R0071 #4)

- Condition the presence of sps_ref_wraparound_enabled_flag such as it is persent onlyh when sps_independent_subpics_flag is equal to 0. When not present, infer the value to be equal to 0.

- Order of slices in PPS signalling and in picture. It is asserted that there is problem since the order of slices signalled in PPS may be different from the order of slices in actually decoding order.

Is it a problem? if yes, the following are proposed fixes:

  - Introduce constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are the same. (JVET-R0091 #1)

The proposed constraint:

The signalling order of slices in the PPS shall follow the decoding order of slice NAL units. Let slice A be signalled by the syntax elements slice_width_in_tiles_minus1[ sA ], slice_height_in_tiles_minus1[ sA ], num_exp_slices_in_tile[ sA ] and exp_slice_height_in_ctus_minus1[ sA ] and let slice B be signalled by the syntax elements slice_width_in_tiles_minus1[ sB ], slice_height_in_tiles_minus1[ sB ], num_exp_slices_in_tile[ sB ] and exp_slice_height_in_ctus_minus1[ sB ]. If coded slice NAL unit A precedes coded slice NAL unit B in the bitstream, then sA shall be less than sB.

In order for item 1 to work, the following constraints are also needed:

One or both of the following conditions shall be fulfilled for each subpicture and tile:

–	All CTUs in a subpicture belong to the same tile

–	All CTUs in a tile belong to the same subpicture

  - Introduce a mapping between the two indexing orders (JVET-R0091 #2, JVET-R0238). In addition, definition of subpicture-level slice index is updated in the spec text (JVET-R0238)

- Alignment on subpic_treated_as_pic_flag value across layers (JVET-R0118 #2, JVET-R0186 #3)

- Move the signalling of no_pic_partition_flag to be earlier than the signalling of pps_num_subpics_minus1. When the value of no_pic_partition_flag is equal to 1, pps_num_subpics_minus1 is not present and inferred to be equal to 0 (JVET-R0186 #1)

- Constrain the value of single_slice_per_subpic_flag to be equal to 0 when no_pic_partition_flag is equal to 0, number of tiles in picture is equal to 1, and the number of subpicture is equal to 1 (JVET-R0186 #2)

- When the maximum picture width and height are both less than or equal to one CTB size, sps_num_subpics_minus1 is not signaled and inferred to be 0 (JVET-R0239 #5)

- Signal a flag sps_raster_scan_order_subpics_flag in the SPS to specify whether subpictures are ordered in raster scan order in the bitstream. (JVET-R0257 #1)

This flag is asserted can be a shortcut for a mode to specify a raster scan order for the subpictures while conforming to the availability rule.

Raster scan ordering of subpictures, which is also a unique ordering of subpictures in the bitstream, is claimed to be useful for extraction and merge purposes and to provide a hook for, e.g., external use.

- Use the above flag to skip the signalling of the top-left position of the subpictures in the SPS when sps_raster_scan_order_subpics_flag is equal to 1. (JVET-R0257 #2)

- Enable signalling of subpicture with filler slice. Decoding process for the filler slice in subpicture can be normative or non-normative (JVET-R0337)

  - A flag sps_filler_slice_present_flag is signalled when subpic_info_present_flag is equal to 1.

  - When sps_filler_slice_present_flag is equal to 1, signal subpic_treated_as_filler_slice_flag[ i ]

- Signaling pps_num_subpics_minus1 in PPS as mandatory, to avoid asserted parsing dependency on SPS (on single_slice_per_subpic_flag when pps_num_subpics_minus1 is not present). (JVET-R0117 #1)

- Change the signalling of subpicture layout in unit of integer multiples units of CtbSizeY (JVET-R0135)

  - Option 1: signal subpic_unit_num_ctus_minus1 syntax element that is the number of CtbSizeY in the subpicture layout. All syntax elements of subpicture layout use the same in units.

  - Option2: signal subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax elements that are the number of CtbSizeY in the subpicture layout. The first syntax element indicate the units for the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ], and the second for the subpic_ctu_top_left_y[ i ] and subpic_height_minus1[ i ]

  - Option3: combine Option1 with Option2 and two options can be selected.

- Change the signalling of slice_subpic_id as follows: (JVET-R0087)

  - Add a flag called slice_subpic_info_present_flag

  - Replace subpic_info_present_flag to condition the presence of slice_subpic_id

  - The value of slice_subpic_info_present_flag is the same as subpic_info_present_flag

- Move the signalling of subpic_id_mapping_in_pps_flag, pps_num_subpics_minus1, pps_subpic_id_len_minus1, and pps_subpic_id[ i ] to be present only when no_pic_partition_flag is equal to 0 (JVET-R0088)

- Add a constraint that the value of subpic_treated_as_pic_flag[] shall be equal to 1, when the value of SubpicIdVal[] of the subpicture is changed from the previous picture (JVET-R0126)

It is intended to guarantee that only independently coded subpictures can be relocated by subpicture ID remapping in PPS.

- Add subpicture ID mapping signalling override mechanism (JVET-R0265)

  - When subpic_id_mapping_explicitly_signalled_flag is equal to 1, the mapping is present in SPS

  - Add a flag subpic_id_mapping_override_in_pps_flag in PPS. When it is equal to 1, signal subpicture Id mapping in PPS.

- On subpicture Id and subpicture Idx in sub-bitstream extraction

  - Use the subpicture index instead of the subpicture ID in the subpicture sub-bitstream extraction process (JVET-R0068 #5)

  - Derive subpicIdx similar to CurrSubpicIdx for each slice, right after the definition of subpicId (JVET-R0294)

- On subpicture size and picture size rewriting for sub-bitstream extraction.

A bug is asserted exist in the current spec for rewriting of picture size during sub-bitstream extraction process. The root of the problem is when the subpicture is located at the bottom and/or right border of a picture that has a size that is not a multiple of the CTU size because subpicture size (i.e., width and height) is expressed in CtbSize, instead of luma samples.

  - Update the sub-bitstream extraction process with different calculation for picture size when the subpicture is the right most subpicture or the bottom subpicture in the original bitstream (JVET-R0092)

  - Derive the subpicture width and height in luma samples and update the rewriting process of picture width and height (JVET-R0294)

- Add a constraint such that no subpicture can be located completely outside of the conformance cropping window. (JVET-R0093 #1, JVET-R0294)

- Define rewriting process for conformance cropping window for sub-bitstream extranction process (JVET-R0093 #2, JVET-R0294)

  - The conformance cropping window offsets of the full picture are kept or not depending on where the subpicture is located within the full picture. If the subpicture is located in the middle of the picture, the conformance cropping window offsets for the subpicture are set to zero (JVET-R0093 #2)

  - Copy all offset values that cross the subpicture to be extracted. If a subpicture lies completely inside the conformance window, no conformance window shall be signalled (JVET-R0294)

- Handling of decoded piocture has SEI msg (JVET-R0294:

  - Option 1: The following applies:

    - Decoded picture hash SEI messages are removed during extraction

    - Decoded picture hash SEI messages are allowed to be nested inside of scalable nesting SEI messages, if subpicture nesting is signalled in the scalable nesting SEI message

    - Decoded pictures hash SEI messages that are nested in a scalable nesting SEI message and associated with subpicId are extracted into the output bitstream

  - Option 2: extended decoded pciture hash SEI msg with hashes for each subpicture

- Information contribution on successful experiments carried out for implementation of subpicture-based system. The experimen included the following steps:

  - Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder

  - Merging selected encoded bitstreams into a bitstream with multiple subpictures, using a merger software developed by the authors

  - Decoding the bitstream having multiple subpictures, using the VTM decoder

# List of proposals

### Subpictures (23)

      - General and misc. subpicture aspects (12)

JVET-R0071 AHG12: Some cleanups on subpicture signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

This contribution proposed the following changes on subpicture signalling:

- Condition sps_independent_subpics_flag on "sps_num_subpics_minus1 > 0". Accordingly, when sps_independent_subpics_flag is not present, it is inferred to be equal to 1.

- Infer subpic_treated_as_pic_flag[ i ] to be equal to 1 when not present.

- Infer loop_filter_across_subpic_enabled_pic_flag[ i ] to be equal to 0 when not present.

- Infer single_slice_per_subpic_flag to be equal to 1 when no_pic_partition_flag is equal to 1.

JVET-R0091 AHG9: Issue with slice indexing [V. Drugeon (Panasonic)]

It is asserted that there are currently two different indexing orders for slices: the order in which slices are signalled within the PPS and the order in which slices are coded within the bitstream. Unfortunately, the current draft specification may fail when these two orders differ from each other, which may easily happen when subpictures are used.

This proposal suggested two alternative solutions to solve this issue:

- Introduce constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are the same.

- Introduce a mapping between the two indexing orders.

The proponent has a preference for the first solution, since it is asserted to simplify implementation and remove corner use cases that have the potential to cause headaches and bugs in subpictures-based implementations.

The constraint proposed in solution 1 is:

The signalling order of slices in the PPS shall follow the decoding order of slice NAL units. Let slice A be signalled by the syntax elements slice_width_in_tiles_minus1[ sA ], slice_height_in_tiles_minus1[ sA ], num_exp_slices_in_tile[ sA ] and exp_slice_height_in_ctus_minus1[ sA ] and let slice B be signalled by the syntax elements slice_width_in_tiles_minus1[ sB ], slice_height_in_tiles_minus1[ sB ], num_exp_slices_in_tile[ sB ] and exp_slice_height_in_ctus_minus1[ sB ]. If coded slice NAL unit A precedes coded slice NAL unit B in the bitstream, then sA shall be less than sB.

In order for item 1 to work, the following constraints are also needed:

One or both of the following conditions shall be fulfilled for each subpicture and tile:

–	All CTUs in a subpicture belong to the same tile

–	All CTUs in a tile belong to the same subpicture

JVET-R0118 AHG9/AHG12: On signalling of subpicture partitinging in SPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 2 of this contribution belongs to this category.

This contribution proposed the following HLS clean-ups in SPS:

- Proposal 1: Signaling sps_picture_header_in_slice_header_flag

- Proposal 2: Alignment on subpic_treated_as_pic_flag value across layers

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 4 of this contribution belongs to this category.

- The signaling of sps_independent_subpics_flag is enabled only when there are at least two subpictures, and add a constraint to the semantics of subpic_info_present_flag

The proposed added constraint to the semantics of subpic_info_present_flag has been discussed. See note in item 3 of JVET-R0343.

JVET-R0238 AHG12: A fix on subpicture-level slice indexing [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, K. Fan, J. Xu, H. Liu (Bytedance)]

In the current VVC text, to derive the value of the variable NumCtusInCurrSlice, which relies on a conversion from the subpicture-level slice index to the picture-level slice index of a slice, it is assumed that the order of increasing picture-level slice index values is the same as the decoding order of slices. However, when some slices are resulted from splitting one tile, it is claimed that the assumption may be violated. Consequenlty, the parsing of slice data would be problematic and the decoded sample values won't be correct.

It is proposed to fix the problem by deriving a mapping between the picture-level slice index and the subpicture-level slice index and using the mapping in the derivation of the value of the variable NumCtusInCurrSlice.

Definition of subpicture-level slice index is updated in the proposed spec text.

The case being argued is illustrated by the figure below. In the shown case the following is asserted: according to the current VVC text, the picture-level slice index values of the four slices in slice raster scan order would be 0, 2, 1, 3, while the decoding order index values of the four slices in slice raster scan order would be 0, 1, 2, 3. Consequently, the derivation of NumCtusInCurrSlice would be incorrect, and in turn the parsing of slice data would be problematic, the decoded sample values won't be correct.

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 1 of this contribution belongs to this category.

The current VVC draft is asserted to be silent on how to deal with a bitstream that uses both a conformance cropping window and subpictures. In order to ensure that extraction of all subpictures of a picture is possible, a constraint is proposed such that no subpicture can be located completely outside of the conformance cropping window.

It is also proposed to extend the subpicture sub-bitstream extraction process to clarify how to rewrite the conformance cropping window offsets when extracting a subpicture.

JVET-R0136 AHG9/AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

Item 1 of this contribution belongs to this category.

This contribution proposes two improvements, including

- When sps_independent_subpics_flag is not present, sps_independent_subpics_flag should be inferred to be equal to 1 (=treat as picture).

- To apply the current nal_unit_type constraints to a bitstream of a subpicture sequence from single-layer bitstream which mixed_nalu_types_in_pic_flag is equal to 1.

JVET-R0186 AHG12: On misc updates for picture partitioning signalling [Hendry, S. Paluri, S. Kim (LGE)]

This contribution proposed the following changes:

- Move the signalling of no_pic_partition_flag to be earlier than the signalling of pps_num_subpics_minus1. When the value of no_pic_partition_flag is equal to 1, pps_num_subpics_minus1 is not present and inferred to be equal to 0.

- Constrain the value of single_slice_per_subpic_flag to be equal to 0 when no_pic_partition_flag is equal to 0, number of tiles in picture is equal to 1, and the number of subpicture is equal to 1.

- Constrain the subpic_treated_as_pic_flag[ i ] equal to 1 to align the VVC specification text with the picture size, subpicture layout, and the value of loop_filter_across_subpic_enabled_flag[ i ] when the bitstream has multi-layers and the pictures within each layer has more than one subpicture, thus making bitstreams independently extractable.

JVET-R0239 AHG9: Cleanups on signalling of tiles, slices, and subpictures [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, J. Xu, H. Liu (Bytedance)]

Item 5 of this contribution belongs to this category.

- When the maximum picture width and height are both less than or equal to one CTB size, sps_num_subpics_minus1 is not signaled and inferred to be 0.

JVET-R0257 AHG12: Raster scan order flag for subpictures [M. Damghanian, R. Sjöberg, M. Pettersson, Z. Zhang, J. Enhorn, J. Ström, R. Yu (Ericsson)]

This contribution proposed to add an sps_raster_scan_order_subpics_flag in the SPS of VVC that specifies that subpictures are ordered in raster scan order in the bitstream. This contribution further proposes to skip the signalling of the top-left position of the subpictures in the SPS when sps_raster_scan_order_subpics_flag is equal to 1.

Proposal 1, SPS flag:

- Add an sps_raster_scan_order_subpics_flag to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1 inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint.

Proposal 2, Subpicture signalling:

- Condition the signalling of the top-left position of the subpictures on sps_raster_scan_order_subpics_flag being equal to 0.

Proposal 1 is assessed by the proponents to be independent from proposal 2.

The proponents claim that the current design generally does not provide a unique order of the subpictures in the bitstream, and that by introducing a raster scan order flag, a shortcut is provided for a mode to specify a raster scan order for the subpictures while conforming to the availability rule. Raster scan ordering of subpictures, which is also a unique ordering of subpictures in the bitstream, is claimed to be useful for extraction and merge purposes and to provide a hook for, e.g., external use.

JVET-R0284 AHG12/AHG9: On independent subpicture signalling [Y.-J. Chang, Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This proposal fixes for the asserted unintended design on independent subpicture signalling when there is only one subpicture. There are two proposed changes:

- sps_independent_subpics_flag conditioned on the number of subpictures

- sps_ref_wraparound_enabled_flag conditioned on (!sps_independent_subpics_flag)

JVET-R0337 AHG12: Subpicture with filler slice for merged stream [K. Kawamura, S. Naito (KDDI)]

This contribution proposed to enable signalling subpicture with filler slice for simple and flexible subpicture merging. For enabling filler subpicture, a filler subpicture flag is introduced. A decoding process of the filler slice is defined as normative or non-normative approaches.

      - Subpicture layout signalling (2)

JVET-R0117 AHG9/AHG12: On signalling of subpicture and slice in PPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 1 of this contribution belongs to this category.

This contribution proposed the following clean-ups in PPS:

- Proposal 1: Signaling pps_num_subpics_minus1 in PPS as mandatory, to avoid asserted parsing dependency on SPS (on single_slice_per_subpic_flag when pps_num_subpics_minus1 is not present).

- Proposal 2: Replacing num_slices_in_pic_minus1 with num_slices_in_pic_minus_num_sub_pics, for efficient signaling.

JVET-R0135 AHG12: On subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

This contribution proposed that in addition to the current subpicture layout in the unit of CtbSizeY, subpicture layout can be signalled in integer multiples units of CtbSizeY for saving bits.

Option1: signal subpic_unit_num_ctus_minus1 syntax element that is the number of CtbSizeY in the subpicture layout.  All syntax elements of subpicture layout use the same in units.

Option2: signal subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax elements that are the number of CtbSizeY in the subpicture layout.  The first syntax element indicate the units for the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ], and the second for the subpic_ctu_top_left_y[ i ] and subpic_heihgt_minus1[ i ].

Option3: combine Option1 with Option2 and two options can be selected.

      - Subpicture ID signalling (4)

JVET-R0087 AHG12: Modification of subpicture information in slice header [W. Lim, G. Bang (ETRI)]

This contribution proposed to modify syntax elements related to subpicture in slice header. VVC slice header refers to subpic_info_present_flag signaled in SPS and parsing dependency may exist. To remove the dependency, slice level subpicture-related syntax element, slice_subpic_info_present_flag, is proposed.

Currently, the presence of syntax element slice_subpic_id is conditioned upon the value of subpic_info_present_flag. This contribution proposed the following:

- Add a flag called slice_subpic_info_present_flag

- Replace subpic_info_present_flag to condition the presence of slice_subpic_id

- The value of slice_subpic_info_present_flag is the same as subpic_info_present_flag

JVET-R0088 AHG12: Modification of subpicture information in PPS [W. Lim, G. Bang (ETRI)]

Item 1 of this contribution belongs to this category.

Proposal 1) Subpicture-related syntax elements are signalled in PPS when a picture partitioned which refers to the PPS.

The signalling of subpic_id_mapping_in_pps_flag, pps_num_subpics_minus1, pps_subpic_id_len_minus1, and pps_subpic_id[ i ] are moved such that they are present only when no_pic_partition_flag is equal to 0.

JVET-R0126 AHG9/AHG12: On signalling of subpicture ID [B. Choi, S. Wenger, S. Liu (Tencent)]

This contribution proposed a constraint that the value of subpic_treated_as_pic_flag[] shall be equal to 1, when the value of SubpicIdVal[] of the subpicture is changed from the previous picture. It is intended to guarantee that only independently coded subpictures can be relocated by subpicture ID remapping in PPS.

JVET-R0265 AHG9/AHG12: On subpicture ID mapping signalling [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

The contribution proposed subpicture ID mapping signaling override design to avoid signaling IDs for all subpictures in PPS.

      - Subpicture based bitstream extraction and merging (5)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 5 of this contribution belongs to this category.

- Use the subpicture index instead of the subpicture ID in the subpicture sub-bitstream extraction process.

It is asserted that when the subpicture ID mapping is signalled in the PPS instead and can be dynamic within a CLVS, there would be a problem as the sub-bitstream extraction is supposed to extract a sequence of collocated subpictures.

JVET-R0092 AHG12: Subpicture size calculation for subpicture extraction [V. Drugeon (Panasonic)]

It is asserted that there is a bug in the current subpicture extraction process whereby the size for a subpicture is claimed to be wrongly calculated when the subpicture is located at the bottom and/or right border of a picture that has a size that is not a multiple of the CTU size. It is suggested to solve this bug by calculating the correct size of the subpictures in luma samples based on the position of the subpicture to extract within the picture.

When extracting a subpicture from a bitstream, currently the width and height of picture is rewritten to be the same as the width and height of the extracted subpicture. However, the variable for width and height of subpicture is in the unit of CtbY so that there is problem when the subpicture is at the right most or at the bottom of the picture since the actual subpicture size may not be multiplication of CtbY.

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 2 of this contribution belongs to this category.

The current VVC draft is silent on how to deal with a bitstream that uses both a conformance cropping window and subpictures. In order to ensure that extraction of all subpictures of a picture is possible, a constraint is proposed such that no subpicture can be located completely outside of the conformance cropping window.

It is also proposed to extend the subpicture sub-bitstream extraction process to clarify how to rewrite the conformance cropping window offsets when extracting a subpicture.

It is also proposed to add a description to the subpicture sub-bitstream extraction process in clause C.7 to clarify how to rewrite the conformance cropping window offsets in the SPS and PPS.

In particular, the conformance cropping window offsets of the full picture are kept or not depending on where the subpicture is located within the full picture. If the subpicture is located in the middle of the picture, the conformance cropping window offsets for the subpicture are set to zero.

JVET-R0294 AHG12: On subpicture extraction [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

This contribution proposes the following changes to the subpicture extraction process:

- rewriting of conformance window

- replace decoded picture hash SEI messages

- bug fixes for subpicture width and height derivation and derivation of the subpicture index

The contribution also proposed to disallow subpictures to be located completely outside the conformance window.

For conformance window:

- Copy all offset values that cross the subpicture to be extracted

- If a subpicture lies completely inside the conformance window, no conformance window shall be signalled

- Disallow subpictures that do not cover samples inside the conformance window

For decoded picture hash SEI msg:

Option 1:

- Decoded picture hash SEI messages are removed during extraction

- Decoded picture hash SEI messages are allowed to be nested inside of scalable nesting SEI messages, if subpicture nesting is signalled in the scalable nesting SEI message

- Decoded pictures hash SEI messages that are nested in a scalable nesting SEI message and associated with subpicId are extracted into the output bitstream

Option 2: extended decoded pciture hash SEI msg with hashes for each subpicture

Asserted bug fix #1: derive subpicIdx similar to CurrSubpicIdx for each slice, right after the definition of subpicId.

Asserted bug fix#2: for sub-bitstream extraction process, derive the subpicture width and height in luma samples and update the rewriting process of picture width and height.

JVET-R0148 AHG12: Subpicture merging experiments [A. Hallapuro, M. Homayouni, A. Aminlou, M. M. Hannuksela (Nokia)]

This contribution reported successful experiments carried out with the following subpicture-based system:

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using a merger software developed by the authors.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

The implemented merger software and the performed experiments give the authors high confidence that both the subpicture feature in the VVC specification and the subpicture implementation in the VTM software are mature.

- Figure 1. System for selective subpicture-based operation.