| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April. 2020 | Document: JVET-R0426 |
| --- | --- |

| Title: | Crosscheck of JVET-R0056 (LFNST complexity reduction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma | Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-R0056 (LFNST complexity reduction). The crosscheck results match the ones provided by the proponent.

# Simulation Results

Simulation results under AI and RA configurations over VTM-8.0 anchor are shown below. The simulation runtime is not accurate.

Table 1. Summary results of the proposed method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.55% | 1.96% | 100% | 98% |
| Class A2 | -0.04% | 1.54% | 1.62% | 101% | 102% |
| Class B | 0.13% | 1.12% | 1.47% | 97% | 97% |
| Class C | 0.27% | 0.73% | 0.96% | 102% | 98% |
| Class E | 0.25% | 1.19% | 0.85% | 98% | 99% |
| Overall | 0.14% | 1.19% | 1.36% | 99% | 99% |
| Class D | 0.25% | 0.90% | 1.01% | 99% | 95% |
| Class F | 0.22% | 0.50% | 0.45% | 98% | 93% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.45% | 0.47% | 101% | 100% |
| Class A2 | 0.03% | 0.45% | 0.59% | 98% | 98% |
| Class B | 0.06% | 0.62% | 0.79% | 96% | 95% |
| Class C | 0.16% | -0.45% | -0.46% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.26% | 0.35% | 99% | 98% |
| Class D | 0.20% | 0.20% | 0.39% | 102% | 103% |
| Class F | 0.19% | -0.25% | 0.12% | 98% | 97% |

Table 2. Summary results of the proposed method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.74% | 0.60% | 99% | 100% |
| Class A2 | 0.01% | 0.45% | 0.43% | 100% | 98% |
| Class B | 0.05% | 0.27% | 0.68% | 98% | 100% |
| Class C | 0.08% | 0.25% | 0.55% | 98% | 101% |
| Class E | 0.03% | 0.29% | 0.73% | 98% | 104% |
| Overall | 0.04% | 0.38% | 0.60% | 98% | 101% |
| Class D | 0.07% | 0.55% | 0.63% | 100% | 98% |
| Class F | 0.03% | 0.26% | 0.50% | 97% | 93% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.20% | 0.27% | 102% | 100% |
| Class A2 | 0.01% | 0.02% | 0.23% | 99% | 99% |
| Class B | 0.01% | 0.09% | 0.38% | 98% | 99% |
| Class C | 0.00% | 0.20% | 0.18% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.13% | 0.27% | 99% | 99% |
| Class D | 0.06% | 0.54% | 0.41% | 99% | 100% |
| Class F | 0.09% | -0.10% | 0.35% | 98% | 99% |