| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0433-r1 |
| --- | --- |

| Title: | AHG 9: Combination of JVET-R0177/R0301 and JVET-R0074 on APS Signaling and Semantics Cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec Tangi Poirier, Philippe de Lagrange (InterDigital) Ling Li, Xiang Li, Byeongdoo Choi Stephan Wenger, Shan Liu (Tencent) Zhipin Deng, Li Zhang, Ye-Kui Wang, Kai Zhang (Bytedance) Shunsuke Iwamura, Shimpei Nemoto, Atsuro Ichigaya (NHK) | Tel: Email: | karam.naser@interdigital.com {aurali, xlxiangli, bdchoi, swenger, shanl }@tencent.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com iwamura.s-gc@nhk.or.jp |
| Source: | InterDigital, Tencent, Bytedance Inc. and NHK |  |  |

_____________________________

# Abstract

This contribution proposes removing redundant signaling inside the APS by skipping the chroma related information (as in JVET-R0177/R0301) and removing the semantics dependency of APS on SPS (as in JVET-R0074).

# Introduction

The syntax element scaling_list_chroma_present_flag in VVC Draft 8 signaled in APS indicates whether chroma is present.

Table 1. Scaling list data syntax in VVC Draft 8

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| … |  |
| } |  |

It is noticed that other parts in APS, namely ALF and LMCS, also contains chroma only related syntax elements. To avoid redundancy, it is proposed to move existing scaling_list_chroma_present_flag in APS forward such that the syntax element could be shared by ALF, LMCS and scaling list related syntax elements.

Further, it is asserted that there exists semantics dependency between APS and SPS. Specifically, the value of the APS syntax element scaling_list_chroma_present_flag is constrained based on ChromaArrayType derived from SPS syntax elements chroma_format_idc and separate_colour_plane_flag, phrased as follows: scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0. It is proposed here to remove such a dependency by setting the chroma related APS syntax element in PH/SH when the APS TemporalId is signaled.

# Proposed Text Changes

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data() |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data() |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data() |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| alf_data() { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| if ( aps_chroma_present_flag ){ |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| … |  |
| } |  |

| lmcs_data() { | Descriptor |
| --- | --- |
| … |  |
| if( aps_chroma_present_flag ) |  |
| lmcs_delta_abs_crs | u(3) |
| … |  |
| } |  |

| scaling_list_data() { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if(scaling_list aps_chroma_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| … |  |
| } |  |

#### 7.4.3.5 Adaptation parameter set semantics

…

aps_chroma_present_flag equal to 1 specifies that the APS NAL unit may include chroma related syntax elements. aps_chroma_present_flag equal to 0 specifies that the APS NAL unit does not include chroma related syntax elements.

...

#### 7.4.3.7 Picture header structure semantics

…

ph_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

When ph_alf_aps_id_luma[ i ] is present, the following applies:

- The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be equal to 1.

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the picture associated with the PH.

- When ChromaArrayType is equal to 0, the value of aps_chroma_present_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be equal to 0.

…

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to.

When ph_lmcs_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

- When ChromaArrayType is equal to 0, the value of aps_chroma_present_flag of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be equal to 0.

…

ph_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS.

When ph_scaling_list_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

- The value of aps_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to ChromaArrayType  = =  0 ? 0 : 1.

### 7.3.7.1 General slice header semantics

…

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ].

When slice_alf_aps_id_luma[ i ] is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

- The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

- When ChromaArrayType is equal to 0, the value of aps_chroma_present_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 0.

…

### 7.4.3.19 Adaptive loop filter data semantics

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0 not present, the value of alf_chroma_filter_signal_flag is inferred to shall be equal to 0.

…

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When ChromaArrayType is equal to 0 not present, the value of alf_cc_cb_filter_signal_flag is inferred to shall be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When ChromaArrayType is equal to 0 not present, the value of alf_cc_cr_filter_signal_flag is inferred to shall be equal to 0.

### 7.4.3.21 Scaling list data semantics

...

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

...

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).