| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0434-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0375 (AHG2/AHG16: CCLM bug fix in luma reference down-sampling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Pfaff | Tel: Email: | +49 jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for JVET-R0375 (AHG2/AHG16: CCLM bug fix in luma reference down-sampling). In this document, it is asserted that in the current specification text, the derivation of the availability of top-left reference samples used for the CCLM downsampling process can be incorrect for certain cases. As a solution, it is proposed to completely remove the checking of the availability of top-left reference samples and to always use padded samples for these samples in the CCLM downsampling. The crosschecker reports that the software and specification text provided match this description. He also reports that the simulation results reported below match with those of the document.

# Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,02% | -0,02% | 95% | 99% |
| Class A2 | 0,00% | 0,04% | 0,03% | 96% | 99% |
| Class B | 0,00% | 0,01% | 0,08% | 100% | 96% |
| Class C | 0,01% | -0,06% | 0,00% | 101% | 92% |
| Class E | 0,00% | 0,01% | -0,04% | 100% | 92% |
| Overall | 0,00% | 0,00% | 0,02% | 99% | 95% |
| Class D | 0,00% | -0,05% | -0,11% | 101% | 99% |
| Class F | 0,02% | 0,02% | 0,02% | 100% | 94% |
|  | 0,00% | 0,02% | -0,02% | 95% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,08% | 0,00% | 101% | 102% |
| Class A2 | -0,02% | -0,06% | 0,14% | 103% | 99% |
| Class B | 0,00% | -0,11% | -0,06% | 102% | 95% |
| Class C | -0,02% | -0,11% | -0,14% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,09% | -0,03% | 101% | 97% |
| Class D | 0,02% | 0,07% | 0,05% | 101% | 98% |
| Class F | 0,04% | -0,01% | -0,02% | 99% | 94% |
|  | -0,01% | -0,08% | 0,00% | 101% | 102% |