| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0436-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0352 (On LFNST in shared tree) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Moonmo Koo 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This is cross-check report for JVET-R0352 (On LFNST in shared tree). The cross-check result matched with what is provided by proponent in JVET-R0352.

Introduction

JVET-R0352 proposes the following two aspects related to restriction on LFNST to the chroma blocks in shared tree.

- Aspect 1 (only SW change): In VTM-8.0, primary transform (DCT-II) coefficients of the chroma blocks in shared tree are zeroed-out even though LFNST is not applied to the chroma blocks. It is proposed not to zero-out the coefficients of the chroma blocks in shared tree.

- Aspect 2: In the current VVC draft, signalling of LFNST index is conditioned on the last scan positions of the transform blocks. Since LFNST is not applied to the chroma blocks in shared tree and chroma coefficients are the primary coefficients, there is no need to check the position of the chroma blocks. It is proposed to remove the chroma-dependent signalling of LFNST index in shared tree.

For aspect 1, SW was changed in such a way that both quantization and transform parts do not apply LFNST zero-out to Chroma components in shared tree. For aspect 2, residual_coding syntax table was modified as shown in Table 1.

Table 1  The proposed spec text change in residual_coding syntax table

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx, treeType ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 && !( treeType = = SINGLE_TREE && cIdx > 0 ) ) |  |
| LfnstDcOnly = 0 |  |
| if( ( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) && !( treeType = = SINGLE_TREE && cIdx > 0 ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| QState = 0 |  |
| } |  |  |

As presented in Table 1, LfnstDcOnly and LfnstZeroOutSigCoeffFlag can be updated (determined) only by Luma component in shared tree, which frees the restriction on LFNST to the Chroma blocks and aligns with the SW change as aspect 1.

Source code analysis

It was confirmed that code implementation is aligned with what JVET-R0352 describes.

Experimental results

The software package was obtained from the proponent. Code is compiled and tested following JVET common test condition [1], and compared with the result of VTM 8.0 as an anchor. The available BD-rate results and run-time ones match with what is provided by proponent. RA simulation was only performed because JVET-R0352 only affects shared tree case.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010, March 2019.