| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0437-v2 |
| --- | --- |

| Title: | Combination of JVET-R0168 and JVET-R0228 on deblocking filter boundary strength setting |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Mohammed Golam Sarwer (Alibaba) Kiyofumi Abe, Tadamasa Toma (Panasonic) | Tel: Email: | {ruling.lrl, yan.ye, m.sarwer}@alibaba-inc.com {abe.kiyo, toma.tadamasa}@jp.panasonic.com |
| Source: | Alibaba Group and Panasonic |  |  |

_____________________________

- Abstract

This contribution is to clarify the interaction between JVET-R0168 and JVET-R0228. It is proposed to set the boundary strength to 1 for luma edge and set to 0 for chroma edge when one of blocks is coded in inter, IBC or palette mode and the other blocks is coded in different mode. It is reported that there is no coding performance impact under CTC. When comparing to VTM-8.0 with palette on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is {0.00%, 0.00%, 0.00%} for AI, {0.01%, -0.01%, -0.01%} for RA, {0.04%, 0.01%, 0.02%} for LB. For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is {0.00%, 0.00%, 0.00%} for AI, {-0.01%, -0.02%, 0.05%} for RA, {-0.08%, 0.36%, 0.42%} for LB. For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is {-0.01%, 0.00%, 0.00%} for AI, {0.03%, 0.09%, 0.03%} for RA, {0.02%, 0.06%, -0.02%} for LB.

Introduction

Two problems are reported in this contribution. Firstly, there is a mismatch between VVC draft 8 and VTM-8.0 when deriving the boundary strength (bS) for a chroma edge. In VVC draft 8, when one of blocks is coded in IBC and the other blocks is coded in inter, bS is set equal to 1 for both luma and chroma edge. However, it is set to 1 for luma edge and is set to 0 for chroma edge in the aforementioned case. Secondly, for both specification and software, bS is not clearly defined when one of blocks is coded in palette and the other blocks is coded in IBC or inter.

Proposed method

For the first problem, it is proposed to align the specification to software. For the second problem, it is proposed to set bS to 1 for a luma edge and set bS to 0 for a chroma edge when palette is used at the other side of the block.

Specification changes

8.8.3.5	Derivation process of boundary filtering strength

….

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a coding block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0.

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

….

Simulation results

The proposed methods are implemented on top of VTM-8.0. Table 1 shows the simulation results under CTC, which is identical to VTM-8.0. Table 2 shows the simulation results for 4:2:0 color format, and palette is on for both anchor and test. Table 3 shows the simulation results for 4:4:4 color format, and palette mode is on and dual tree is off for both anchor and test.

Table 1. Results under CTC

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 98% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 95% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 96% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 98% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 98% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 96% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 98% |  |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 99% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 99% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 96% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 94% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 97% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 96% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 95% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 97% |  | 93% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 98% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 91% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 90% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 94% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 92% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 95% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 97% |  |

Table 2. Results for 4:2:0 color format with palette on

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  |  | DecT |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 100% |  |  | 96% |
| Class TGM | -0.01% | 0.00% |  | 0.00% |  | 100% |  |  | 98% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  |  | DecT |
| Class F | -0.01% | -0.02% |  | 0.05% |  | 100% |  |  | 96% |
| Class TGM | 0.03% | 0.09% |  | 0.03% |  | 99% |  |  | 96% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  |  | DecT |
| Class F | -0.08% | 0.36% |  | 0.42% |  | 100% |  |  | 93% |
| Class TGM | 0.02% | 0.06% |  | -0.02% |  | 99% |  |  | 96% |

Table 3. Results for 4:4:4 color format with palette on and dual tree off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 98% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | -0.01% | -0.04% | 99% | 98% |
| TGM 720p | 0.00% | 0.00% | 0.02% | 100% | 98% |
| Animation | 0.04% | 0.02% | 0.01% | 100% | 98% |
| Mixed content | 0.00% | -0.02% | -0.03% | 100% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.01% | -0.01% | -0.01% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.04% | 0.02% | 0.00% | 99% | 98% |
| TGM 720p | 0.07% | 0.03% | -0.17% | 99% | 97% |
| Animation | 0.03% | 0.01% | 0.02% | 100% | 97% |
| Mixed content | 0.00% | -0.03% | 0.32% | 99% | 96% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall | 0.04% | 0.01% | 0.02% | 99% | 97% |

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).