| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0438-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0333: AHG11: Mismatches related to palette prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the crosscheck summary for the methods and results in JVET-Q0333. The proposal suggested three changes in the handlation of local dual tree for the palette mode. This report confirms that the crosscheck results match the results provided by the proponent. In addition, the spec changes are aligned with the implementation in the software proivided.

# Introduction

This document reports the crosscheck report for proposal JVET-R0333. In the proposal, two mismatches between spec and software regarding the handlation of local dual tree for the palette modethe are identified in the current VVC draft 8 spec. the first mismatch is related to palette table generation in the case of local dual tree, and the 2nd mismatch is related to the palette predictor update in the case of local dual tree. This report confirms that the two mismatches do exist in the current VVC draft 8 spec and the suggested changes are aligned with what was done in the VTM software. Besides the bugfixes suggested, Document JVET-R0333 also propose changes in the palette table generation in the case of local dual tree which provides simplification in the spec with negligible effect in coding efficiency. The crosscheck results match the one provided by the proponents.

# Simulation Results

The first two suggestions only fix the mismatch in the current VVC draft 8 spec to align with VTM software. For the third suggestion, the crosscheck simulation is conducted for YUV4:2:0 sequences in class F and class TGM. The IBC, HashME, BDPCM, and PLT are turned on for all the sequences.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 + PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM8.0 + PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | 0.00% | -0.04% | -0.02% | 100% | 100% |
| Class TGM | 0.11% | 0.08% | 0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM8.0 + PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | -0.06% | 0.44% | 0.35% | 100% | 100% |
| Class TGM | 0.05% | 0.08% | 0.04% | 100% | 100% |