| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0444 |
| --- | --- |

| Title: | Crosscheck of JVET-R0291 (AHG16: On ALF attenuation near virtual boundaries) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides a crosscheck report of JVET-R0291. The simulations were carried out based on the common test condition (CTC) with VTM-8.0.

Test results matches the results provided in the contribution. In addition, the software matches the proposed spec text in the contribution.

# Introduction

At the 17th JVET meeting ALF attenuation near virtual boundaries was introduced. Two modifications are proposed in JVET-R0291: a) rounding is modified such as to match the increased shift amount when attenuation is applied b) attenuation is extended to CC-ALF, where attenuation factors are further increased to limit the amount of required line buffers when using 4:2:2 and 4:4:4 chroma sampling formats.

# Simulation Results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.07% | 100% | 100% |
| Class A2 | 0.00% | 0.09% | 0.12% | 103% | 100% |
| Class B | 0.00% | 0.11% | 0.08% | 99% | 99% |
| Class C | 0.00% | 0.02% | 0.03% | 100% | 99% |
| Class E | 0.00% | 0.03% | 0.06% | 99% | 100% |
| Overall | 0.00% | 0.07% | 0.07% | 100% | 100% |
| Class D | 0.00% | 0.02% | 0.02% | 100% | 102% |
| Class F | 0.00% | 0.04% | 0.04% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.02% | 102% | 99% |
| Class A2 | 0.02% | -0.04% | 0.21% | 100% | 101% |
| Class B | 0.01% | 0.05% | 0.04% | 100% | 101% |
| Class C | 0.00% | 0.03% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.14% | -0.08% | 99% | 99% |
| Class F | 0.00% | 0.00% | -0.08% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.06% | 0.26% | 100% | 99% |
| Class C | -0.06% | -0.15% | -0.12% | 100% | 100% |
| Class E | -0.04% | 0.47% | -0.08% | 101% | 99% |
| Overall | -0.04% | 0.09% | 0.05% | 100% | 99% |
| Class D | -0.06% | -0.18% | -0.42% | 99% | 101% |
| Class F | -0.06% | 0.49% | 0.37% | 99% | 101% |