| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0446-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0259 (AHG7: On CCALF filtering of chroma sample location type-2 content) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangjun Pu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | +1 408 330 3281 Fangjun.Pu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for contribution JVET-R0259 on CCALF filtering of chroma sample location type-2 content. The cross-checking tests were performed for both HDR and SDR content under CTC test conditions [3][4], in AI, RA and LDB configurations. The results are reported to match the ones provided by the proponents in JVET-R0259. Encoding and decoding runtimes are also reported to be similar order.

# Introduction

In VVC, the CCALF filter is designed based on chroma sample location type-0 mainly for SDR content. However, for HDR type-2 videos, current CCALF filter is not symmetric, which may not achieve optimal coding performance. JVET-R0259 proposes three cross shaped CCALF filters to improve the coding performance for chroma sample location type-2 content.

# Experimental Results

For HDR content, the tests are conducted under HDR CTC test conditions [3]. H1 (PQ) content are with chroma sample location type-2, H2 (HLG) content are with chroma sample location type-0. Table 1-3 show the BD-rate performance of H1 and H2 CTC test sequences for 9-tap, 13-tap, and 8-tap filter, respectively.

Table 1. BD-rate performance of HDR CTC testsets for 9-tap CCALF filter

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.29% | 0.01% | 0.01% | -0.61% | -0.63% | 0.01% | -0.77% | -0.81% | 101% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.53% | 0.17% | 95% | 97% |
| Overall | -0.29% | 0.01% | 0.01% | -0.61% | -0.63% | 0.01% | -0.29% | -0.45% | 99% | 98% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.62% | 0.08% | 0.07% | -1.94% | -2.40% | 0.07% | -1.99% | -1.95% | 96% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | -0.02% | -0.03% | 97% | 99% |
| Overall | -0.62% | 0.08% | 0.07% | -1.94% | -2.40% | 0.04% | -1.27% | -1.25% | 97% | 99% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -3.05% | 0.17% | 0.19% | -6.12% | -9.67% | 0.19% | -5.85% | -9.09% | 98% | 99% |
| Class H2 |  |  |  |  |  | -0.01% | -1.91% | -0.64% | 97% | 102% |
| Overall | -3.05% | 0.17% | 0.19% | -6.12% | -9.67% | 0.11% | -4.41% | -6.02% | 98% | 100% |

Table 2. BD-rate performance of HDR CTC testsets for 13-tap CCALF filter

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.39% | 0.04% | 0.05% | -1.06% | -1.12% | 0.05% | -1.40% | -1.34% | 98% | 98% |
| Class H2 |  |  |  |  |  | 0.02% | -0.23% | -0.23% | 96% | 101% |
| Overall | -0.39% | 0.04% | 0.05% | -1.06% | -1.12% | 0.03% | -0.98% | -0.94% | 98% | 99% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -1.22% | 0.05% | 0.04% | -2.68% | -3.66% | 0.04% | -2.85% | -3.22% | 96% | 99% |
| Class H2 |  |  |  |  |  | 0.01% | -1.46% | -1.11% | 97% | 98% |
| Overall | -1.22% | 0.05% | 0.04% | -2.68% | -3.66% | 0.03% | -2.34% | -2.45% | 96% | 98% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -4.18% | 0.06% | 0.10% | -7.19% | -11.70% | 0.10% | -7.28% | -11.34% | 97% | 98% |
| Class H2 |  |  |  |  |  | -0.03% | -4.95% | -4.05% | 100% | 97% |
| Overall | -4.18% | 0.06% | 0.10% | -7.19% | -11.70% | 0.05% | -6.43% | -8.69% | 98% | 97% |

Table 3. BD-rate performance of HDR CTC testsets for 8-tap CCALF filter

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.11% | 0.00% | 0.00% | -0.16% | -0.24% | 0.00% | -0.35% | -0.39% | 100% | 98% |
| Class H2 |  |  |  |  |  | 0.00% | 0.73% | 0.28% | 98% | 97% |
| Overall | -0.11% | 0.00% | 0.00% | -0.16% | -0.24% | 0.00% | 0.04% | -0.15% | 99% | 98% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.27% | 0.06% | 0.05% | -1.28% | -1.85% | 0.05% | -1.43% | -1.52% | 99% | 98% |
| Class H2 |  |  |  |  |  | 0.00% | 0.58% | 0.09% | 98% | 98% |
| Overall | -0.27% | 0.06% | 0.05% | -1.28% | -1.85% | 0.03% | -0.70% | -0.93% | 98% | 98% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -2.14% | 0.13% | 0.17% | -3.99% | -7.62% | 0.17% | -4.10% | -7.04% | 97% | 98% |
| Class H2 |  |  |  |  |  | 0.01% | -0.92% | 0.04% | 98% | 100% |
| Overall | -2.14% | 0.13% | 0.17% | -3.99% | -7.62% | 0.11% | -2.95% | -4.46% | 98% | 99% |

For SDR content, the tests are conducted under SDR CTC test conditions [4]. All SDR content are with chroma sample location type-0. Table 4-6 show the BD-rate performance of SDR CTC test sequences for 9-tap, 13-tap, and 8-tap filter, respectively.

Table 4. BD-rate performance of SDR CTC testsets for 9-tap CCALF filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.25% | -0.89% | 98% | 102% |
| Class A2 | 0.00% | -0.47% | -0.04% | 99% | 98% |
| Class B | 0.01% | -0.35% | -0.71% | 105% | 97% |
| Class C | 0.02% | -0.34% | -0.38% | 101% | 92% |
| Class E | 0.04% | 0.20% | -1.46% | 102% | 97% |
| Overall | 0.02% | -0.26% | -0.68% | 101% | 97% |
| Class D | 0.02% | -0.09% | -0.65% | 87% | 76% |
| Class F | 0.02% | 0.22% | 0.44% | 104% | 95% |
| Class SCC | 0.01% | 0.87% | 0.81% | 102% | 93% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.27% | -0.63% | 101% | 106% |
| Class A2 | -0.02% | -0.60% | 0.35% | 95% | 107% |
| Class B | 0.02% | -0.37% | -0.86% | 98% | 97% |
| Class C | 0.00% | -0.32% | -0.53% | 102% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.39% | -0.48% | 99% | 99% |
| Class D | 0.02% | -0.19% | -0.69% | 87% | 96% |
| Class F | 0.02% | 0.22% | 0.20% | 97% | 97% |
| Class SCC | -0.02% | 0.88% | 0.76% | 95% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -1.25% | -1.28% | 91% | 91% |
| Class C | -0.08% | -1.56% | -0.69% | 107% | 94% |
| Class E | 0.00% | -0.08% | -1.71% | 102% | 107% |
| Overall | -0.04% | -1.06% | -1.19% | 99% | 96% |
| Class D | 0.01% | -1.55% | -1.84% | 105% | 99% |
| Class F | -0.02% | 0.05% | -0.53% | 101% | 105% |
| Class SCC | 0.00% | 1.09% | 0.73% | 99% | 98% |

Table 5. BD-rate performance of SDR CTC testsets for 13-tap CCALF filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.57% | -1.43% | 95% | 105% |
| Class A2 | 0.01% | -1.43% | -0.52% | 95% | 99% |
| Class B | 0.02% | -0.99% | -1.19% | 97% | 95% |
| Class C | 0.06% | -0.72% | -0.68% | 101% | 97% |
| Class E | 0.10% | -0.32% | -2.14% | 106% | 104% |
| Overall | 0.04% | -0.82% | -1.16% | 98% | 99% |
| Class D | 0.03% | -0.22% | -0.42% | 103% | 98% |
| Class F | 0.06% | -0.19% | 0.13% | 102% | 97% |
| Class SCC | 0.02% | 0.84% | 0.80% | 100% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.63% | -1.58% | 99% | 107% |
| Class A2 | -0.04% | -1.87% | -1.30% | 97% | 105% |
| Class B | 0.02% | -1.61% | -1.48% | 99% | 99% |
| Class C | -0.01% | -1.00% | -0.77% | 100% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -1.30% | -1.28% | 99% | 100% |
| Class D | 0.00% | -0.42% | -0.80% | 100% | 96% |
| Class F | 0.03% | -0.04% | -0.07% | 96% | 103% |
| Class SCC | 0.00% | 0.77% | 0.65% | 97% | 106% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -4.45% | -3.28% | 101% | 98% |
| Class C | -0.10% | -3.34% | -2.00% | 98% | 93% |
| Class E | 0.09% | -1.15% | -3.08% | 93% | 107% |
| Overall | -0.02% | -3.25% | -2.80% | 98% | 98% |
| Class D | 0.00% | -2.99% | -2.00% | 109% | 104% |
| Class F | -0.02% | -0.67% | -0.61% | 99% | 113% |
| Class SCC | 0.02% | 0.71% | 0.48% | 103% | 110% |

Table 6. BD-rate performance of SDR CTC testsets for 8-tap CCALF filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | -0.76% | 92% | 103% |
| Class A2 | 0.00% | -0.19% | 0.08% | 96% | 101% |
| Class B | 0.00% | -0.23% | -0.52% | 99% | 99% |
| Class C | 0.01% | -0.18% | -0.26% | 100% | 96% |
| Class E | 0.03% | 0.38% | -1.43% | 101% | 102% |
| Overall | 0.01% | -0.09% | -0.55% | 98% | 100% |
| Class D | 0.01% | -0.10% | -0.67% | 102% | 98% |
| Class F | 0.01% | 0.35% | 0.54% | 102% | 97% |
| Class SCC | 0.00% | 0.88% | 0.82% | 101% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.17% | -0.47% | 97% | 110% |
| Class A2 | -0.03% | -0.22% | 0.52% | 97% | 105% |
| Class B | 0.01% | -0.21% | -0.71% | 102% | 96% |
| Class C | -0.03% | -0.17% | -0.30% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.19% | -0.31% | 99% | 100% |
| Class D | 0.03% | -0.19% | -0.94% | 98% | 94% |
| Class F | 0.01% | 0.21% | 0.26% | 94% | 103% |
| Class SCC | -0.03% | 0.94% | 0.81% | 93% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.77% | -0.72% | 100% | 96% |
| Class C | -0.06% | -1.36% | -0.59% | 104% | 97% |
| Class E | 0.07% | 0.89% | -1.64% | 101% | 111% |
| Overall | -0.01% | -0.55% | -0.91% | 102% | 100% |
| Class D | 0.01% | -1.50% | -1.38% | 105% | 105% |
| Class F | 0.12% | 0.76% | 0.17% | 100% | 115% |
| Class SCC | 0.04% | 1.16% | 0.88% | 99% | 104% |

# References

M. G. sarwer, Y. Ye, J. Luo, “AHG7: On CCALF filtering of chroma sample location type-2 content”, JVET-R0259.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011.

F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,”, JVET-N1010.

VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).