| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0455 |
| --- | --- |

| Title: | AHG17: Cross-check report of JVET-R0359 on Illustration of the film grain characteristics SEI message for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | P. de Lagrange E. François | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document is a cross-check report of contribution JVET-R0359 on Illustration of the film grain characteristics SEI message for VVC. It is reported that the VTM8 software implementation proposed in JVET-R0359 has been checked, and that the VTM8 simulation results match those reported in JVET-R0359 for the 5 proposed test configurations. The cross-checker also reports partial visual checking confirming a benefit from using film grain for the test sequence proposed in JVET-R0359, especially at low and medium bitrates.

# Cross-check report

Contribution JVET-R0359 from Dolby proposes a software implementation to illustrate the film grain characteristics SEI message for VVC specified in H.SEI | ISO/IEC 23002-7, and also found in AVC and HEVC specifications.

This implementation is said to comply with the SMPTE RDD 5 document, which restricts the value of some syntax elements of above-mentioned SEI and provides a bit-exact film grain synthesis specification with low complexity.

It deviates however in one minor aspect, relaxing the constraint on the range of comp_model_value to (0 – (2bitDepth – 1)) instead of (0 – 255). This extended range is exercised in test configuration #5, but may not be needed as the values could be lowered by acting on log2_scale_factor.

The contribution contains a general description of SMPTE RDD 5 film grain synthesis, a software implementation on top of VTM8, various test configuration files, and instructions to run tests.

Software has been carefully checked to accurately implement SMPTE RDD 5. The code can be qualified as mature, complete, and clean. It was compiled and run successfully, and the results match with the contributor.

The software changes from VTM can be divided in two categories:

- SEI message generation from configuration parameters and encoder side, and parsing at decoder side. This enhances existing film grain SEI message handling currently present in VTM 8, by supporting the actual effective parameters (e.g. num_intensity_intervals_minus1, num_model_values_minus1, comp_model_value), and allowing it to be sent every picture. This has also an impact on parcat utility.

- Film grain synthesis itself, implemented in a new file (SEIFilmGrainSynthesizer.cpp), and invoked from a new function after picture reconstruction (Picture::getDisplayBuf). It is noted that this architecture could enable other post-processing features to co-exist.

Some visual checking has been done, verifying subjective value of film grain synthesis in some scenarios, as illustrated in the following figure:

Figure 1: ArenaOfValor, QP37, test #5. Left: raw decoded picture, right: adding film grain synthesis

InterDigital thanks Dolby for providing this valuable reference implementation of SMPTE RRD 5 compliant film grain synthesis in VTM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).