| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0457 |
| --- | --- |

| Title: | Crosscheck of JVET-R0345 (Unified primary transform kernel for ISP mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-R0345, which proposed to use a unified primary transform kernel, i.e., DCT-2, for ISP mode when sps_lfnst_enabled_flag is equal to 1. The software implementation is provided by the proponent of JVET-R0345, and simulations results are reportedly matching with the results provided by proponent of JVET-R0345.

# Crosscheck report

It is reported in JVET-R0345 that, in the latest VVC spec text [1], when a CU is coded as ISP mode, an implicit transform scheme may be applied. Specifically, DCT-2 or DST-7 is selected as the primary transform kernel according to the following condition:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? DST-7 : DCT-2		(1)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? DST-7 : DCT-2		(2)

In the proposed method, when sps_lfnst_enabled_flag is equal to 1, the primary transform kernel of ISP mode is set as DCT-2 regardless of the lfnst index being signalled.

Simulated are performed using common test condition (CTC) [2]. The anchor is VTM-8.0, and the test is the JVET-R0345. Below tables summarize the results. Simulations are performed on heterogeneous clusters and run-time may not be reliable.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | -0.09% | 99% | 100% |
| Class A2 | 0.05% | 0.06% | 0.10% | 98% | 99% |
| Class B | 0.06% | 0.15% | 0.18% | 98% | 98% |
| Class C | 0.05% | 0.12% | 0.05% | 97% | 95% |
| Class E | 0.15% | 0.24% | 0.32% | 98% | 96% |
| Overall | 0.06% | 0.13% | 0.12% | 98% | 97% |
| Class D | 0.04% | 0.09% | 0.05% | 98% | 95% |
| Class F | -0.04% | 0.06% | 0.00% | 99% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | -0.03% | 87% | 90% |
| Class A2 | 0.01% | 0.05% | 0.22% | 88% | 91% |
| Class B | 0.04% | 0.02% | 0.07% | 98% | 97% |
| Class C | 0.02% | 0.02% | 0.09% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.08% | 94% | 94% |
| Class D | 0.08% | 0.12% | 0.19% | 99% | 97% |
| Class F | 0.04% | 0.00% | 0.04% | 99% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.09% | 0.04% | 88% | 94% |
| Class C | -0.08% | -0.20% | -0.20% | 99% | 97% |
| Class E | 0.05% | -0.12% | -0.19% | 99% | 95% |
| Overall | 0.00% | -0.06% | -0.09% | 94% | 95% |
| Class D | 0.00% | 0.34% | 0.25% | 99% | 95% |
| Class F | -0.01% | -0.09% | -0.29% | 99% | 98% |

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Document JVET-Q2001, Version 14 , Brussels, Belgium, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.