| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0458 |
| --- | --- |

| Title: | Crosscheck of JVET-R0174 (LFNST index signaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-R0174. The proposal removes the redundant checking of the significant coefficient positions for single tree chroma coding blocks regarding LFNST index signalling.

Introduction

In WD8 and VTM-8.0, the signalling of LFNST index is conditioned by significant coefficient positions of both luma and chroma coding blocks in single tree even though there is no need to check those of chroma coding blocks since LFSNT is prohibited in chroma coding blocks. JVET-R0174 [1] proposes to remove the redundant checking of the significant coefficient positions for single tree chroma coding blocks regarding LFNST index signalling. In detail, two checking regarding last significant position and zero out positon, i.e., LfnstDcOnly and LfnstZeroOutSigCoeff, are modified not to check.

An implementation of the provided software is consistent with the description in the contribution.

The anchor result was generated by using the common test condition [2]. Only RA configuration is evaluated since the proposed method does not affect AI and LDB configuration results since dual tree is enabled by default in AI configuration and LFNST is disabled by default in LDB configuration. The provided software was compiled on 64-bit CentOS environment. The experiments were performed on the cluster PCs.

Table 1 Random access configuration results of JVET-R0174

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | 0.07% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.04% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.03% | 100% | 100% |
| Class D | -0.01% | 0.04% | -0.17% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.02% | 100% | 100% |

Conclusion

We had verified results reported in JVET-Q0174. The results of the cross-check matched with proponent’s report.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- C. Rosewarne and J. Gan, “LFNST index signaling”, JVET-R0174, by teleconference, 15–24 April 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.