| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0459 |
| --- | --- |

| Title: | Crosscheck of JVET-R0318 (Alternative methods of LFNST index signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-R0318 Method 2 and Method 3. Method 2 proposes to enable LFNST to be applied into single tree chroma coding blocks which LFNST is prohibited in WD8 and VTM-8.0. Method 3 proposes to move the LFNST index parsing position from CU-based to TU-based signalling processes, and to remove the redundant checking of the significant coefficient position and transform skip flag for single tree chroma coding blocks regarding LFNST index signalling.

Introduction

In WD8 and VTM-8.0, the signalling of LFNST index is conditioned by significant coefficient positions of both luma and chroma coding blocks in single tree even though there is no need to check those of chroma coding blocks since LFSNT is prohibited in chroma coding blocks. JVET-R0318 [1] proposes three alternative methods of LFNST index signalling. In this contribution, Method 2 and Method 3 are evaluated. Method 2 proposes to enable LFNST to be applied into single tree chroma coding blocks which LFNST is prohibited in WD8 and VTM-8.0. In detail, LFNST is applied only into the TU which is equal to or larger than 4x4 pixels. Method 3 proposes to move the LFNST index parsing position from CU-based to TU-based signalling processes, and to remove the redundant checking of the significant coefficient position and transform skip flag for single tree chroma coding blocks regarding LFNST index signalling. Furthermore, along with the signalling modification, redundant checking in RDO, unnecessary prohibition of transform skip and zeroing for single tree chroma coding blocks when LFNST index is signaled, are fixed.

An implementation of the provided software is consistent with the description in the contribution.

The anchor result was generated by using the common test condition [2]. Only RA configuration is evaluated since the proposed method does not affect AI and LDB configuration results since dual tree is enabled by default in AI configuration and LFNST is disabled by default in LDB configuration. The provided software was compiled on 64-bit CentOS environment. The experiments were performed on the cluster PCs.

Method 2

Table 1 Random access configuration results of JVET-R0318 Method 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.33% | -0.40% | 100% | 100% |
| Class A2 | -0.03% | -0.10% | -0.11% | 100% | 100% |
| Class B | -0.03% | -0.23% | -0.26% | 100% | 100% |
| Class C | -0.06% | -0.09% | -0.32% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.19% | -0.27% | 100% | 100% |
| Class D | 0.02% | 0.01% | -0.29% | 100% | 101% |
| Class F | 0.01% | -0.10% | -0.14% | 100% | 100% |

Method 3

Table 1 Random access configuration results of JVET-R0318 Method 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.21% | -0.21% | 100% | 99% |
| Class A2 | 0.00% | -0.06% | 0.07% | 100% | 99% |
| Class B | -0.01% | -0.01% | -0.05% | 101% | 99% |
| Class C | 0.00% | 0.07% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.06% | 100% | 100% |
| Class D | 0.01% | 0.10% | -0.06% | 100% | 101% |
| Class F | 0.01% | 0.03% | 0.00% | 101% | 100% |

Conclusion

We had verified results reported in JVET-Q0318. The results of the cross-check matched with proponent’s report.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- M. Koo, M. Salehifar, J. Lim, S. Kim, “Alternative methods of LFNST index signalling”, JVET-R0318, by teleconference, 15–24 April 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.