| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0468-v1 |
| --- | --- |

| Title: | AHG13: On RGB common test condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Wei-Jung Chien, Marta Karczewicz Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang | Tel: Email:yunghsua@qti.qualcomm.com xiaoyuxiu@kwai.com |  |
| Source: | Qualcomm Incorporated, Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to disable chroma separate tree (--DualITree=0) for RGB sequences in the non-420 common test condition. The results of different combinations of ACT and chroma separate tree setting are presented in the document. From the results, enabling ACT and disabling chroma separate tree gives the best performance for RGB sequences for both natural and screen content.

Disabling chroma separate tree versus enabling chroma separate tree for ACT:

Natural content: -13.59% AI, x.xx% RA, x.xx% LDB

Screen Content: x.xx% AI, x.xx% RA, x.xx% LDB

# Proposal

In the non-420 common test condition [1], adaptive colour transform (ACT) is enabled in SPS for RGB format, for both natural content and screen content sequences. On the other hand, chroma separate tree (CST) is enabled for natural content and disabled for screen content. However, ACT is not applicable to CUs when CST is used for a slice. Such combination may result in sub-optimal coding performance for RGB natural contents. In this contribution, the results of different combinations of ACT and CST setting are compared. From the results, enabling ACT and disabling CST gives the best performance for RGB sequences. Therefore, it is suggested to disable CST for RGB natural content in the CTC.

# Results

## The results are evaluated on VTM8.0 with merge request #1488 and #1492, for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. Except the setting of ACT (--ColorTransform) and CST (--DualITree), the parameter setting follows common test condition of the non-420 format [1]. Since the ACT cannot be enabled when maximum TU size is equal to 64 (--Log2MaxTbSize=6), ACT and --Log2MaxTbSize=5 are coupled in the parameter setting.

Table 1-2 shows the performance when ACT is enabled (--ColorTransform=1, --Log2MaxTbSize=5) and CST is disabled (--DualITree=0). The results are versus ACT and CST enabled (--ColorTransform=1, --Log2MaxTbSize=5, --DualITree=1).

Table 1: Natural content results of disabling CST for ACT

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM8.0 + MR#1488 and #1492 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Traffic | -21.81% |  | 3.85% |  | -15.35% |  | 243% |  | 96% |
| Kimono | -15.34% |  | 0.05% |  | -6.71% |  | 251% |  | 87% |
| EBULupoCandlelight | -19.43% |  | 2.31% |  | -8.72% |  | 168% |  | 104% |
| EBURainFruits | -13.37% |  | -0.16% |  | -2.22% |  | 151% |  | 104% |
| VenueVu | -12.15% |  | -3.50% |  | -1.00% |  | 194% |  | 93% |
| DucksAndLegs | -4.18% |  | 0.92% |  | -2.09% |  | 324% |  | 88% |
| OldTownCross | -10.41% |  | 2.70% |  | -1.84% |  | 262% |  | 92% |
| ParkScene | -12.05% |  | 0.49% |  | -4.89% |  | 258% |  | 93% |
| Overall | -13.59% |  | 0.83% |  | -5.35% |  | 189% |  | 94% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | VTM8.0 + MR#1488 and #1492 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Traffic | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Kimono | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| EBULupoCandlelight | -16.12% | -5.10% |  | -7.19% |  | 103% |  | 101% |  |
| EBURainFruits | -3.85% | 0.37% |  | 1.07% |  | 102% |  | 99% |  |
| VenueVu | -0.87% | 0.15% |  | 0.63% |  | 103% |  | 101% |  |
| DucksAndLegs | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| OldTownCross | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| ParkScene | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |

# References

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013.

# Patent rights declaration(s)

Qualcomm Incorporated and Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).