| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0471-v2 |
| --- | --- |

| Title: | On CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

Numerous issues are identified in the CCLM-related text of VVC draft 8 (version E) and changes are proposed to resolve these.

# Proposed solution

A number of previously unidentified issues are addressed in the attached modified draft text. The changes are believed to be consistent with previously agreed design intent. Addressed issues include:

- Samples pY[ −1 ][ y ] may be needed in equations (370) and (371). However, no value is assigned to them if numSampL is 0 and availL is TRUE;

- Samples pY[ x ][ −1] may be needed in equation (370). However, no value is assigned to them if numSampT is 0 and availT is TRUE;

- sps_chroma_vertical_collocated_flag may be equal to 0 only when the chroma format is 4:2:0. The filter F4 is used only when sps_chroma_vertical_collocated_flag is 0. Defining F4 for chroma formats other than 4:2:0 is unnecessary. Such unnecessary definition is removed;

- Horizontal filtering when the chroma format is 4:2:2 is correctly defined;

- The luma locations used to derive numTopRight and numLeftBelow are corrected to include consideration for SubWidthC and SubHeightC; and

- Various fixes found in other input contributions such as JVET-R0314, JVET-R0356, JVET-R0375 and JVET-R0452 are included.

Regarding the 4:2:2 case, it should be noted that the VTM software appears to incorrectly set sps_chroma_vertical_collocated_flag to 0 but nevertheless correctly applies a (1 2 1) horizontal filter on luma samples. When correcting the flag value to 1, filter definitions should be updated in VTM (in accordance to the text changes listed above).

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).