| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0473-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0347 (AHG2: On minimum QT size, maximum BT size and maximum TT size) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This document reports the results of crosschecking JVET-R0347 proposed by Qualcomm. JVET-R0347 proposes to (1) change the upper limit of minimum QT size, maximum BT size and maximum TT size under dual-tree condition to 64, and (2) set the upper limit of minimum QT size and maximum TT size under single-tree condition to 64. The changes in the provided software and changes in the provided VVC Draft are aligned. The verification task was done, and the BD-rate results match those in JVET-R0347.

Verification

Qualcomm provided the source code of their proposed method described in JVET-R0347 [1]. Three tests were conducted under the common test conditions (CTC) defined in [2] using the VTM8.0 software [3]. Table 1 and Table 2 show the results of Test 1 and Test 2 using VTM8.0 as the anchor, respectively. Table 3 shows the results of Test 3 using Test 1 as the anchor. The BD-rates of the test from our experiment match those in JVET- R0347.

Table 1.  The results of Test1 using VTM8.0 as anchor. Note that the runtime is not accurate.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.04% | 30.20% | 32.89% | 109% | 100% |
| Class A2 | 3.93% | 14.91% | 10.05% | 82% | 85% |
| Class B | 5.92% | 14.12% | 16.64% | 74% | 84% |
| Class C | 13.23% | 23.83% | 25.82% | 63% | 75% |
| Class E | 8.98% | 7.52% | 8.38% | 74% | 94% |
| Overall | 7.74% | 17.99% | 18.91% | 78% | 86% |
| Class D | 12.34% | 21.20% | 22.26% | 60% | 86% |
| Class F | 25.63% | 34.10% | 39.98% | 46% | 82% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 14.84% | 21.05% | 24.63% | 33% | 92% |
| Class A2 | 23.72% | 32.21% | 27.40% | 34% | 91% |
| Class B | 29.81% | 31.52% | 32.73% | 33% | 91% |
| Class C | 62.52% | 54.00% | 61.77% | 41% | 74% |
| Class E |  |  |  |  |  |
| Overall | 34.32% | 35.56% | 37.79% | 35% | 86% |
| Class D | 53.11% | 50.54% | 52.56% | 55% | 77% |
| Class F | 56.60% | 54.26% | 59.36% | 42% | 88% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 29.54% | 24.06% | 24.41% | 31% | 87% |
| Class C | 70.26% | 45.04% | 50.74% | 43% | 93% |
| Class E | 30.19% | 22.93% | 24.56% | 46% | 98% |
| Overall | 43.27% | 30.77% | 33.23% | 38% | 92% |
| Class D | 59.61% | 42.48% | 47.68% | 59% | 101% |
| Class F | 97.28% | 68.84% | 73.67% | 41% | 91% |

Table 2.  The results of Test 2 using VTM8.0 as anchor. Note that the runtime is not accurate.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.05% | 30.20% | 32.89% | 105% | 90% |
| Class A2 | 3.93% | 14.91% | 10.05% | 76% | 79% |
| Class B | 5.92% | 14.13% | 16.64% | 71% | 79% |
| Class C | 13.23% | 23.83% | 25.83% | 61% | 73% |
| Class E | 8.99% | 7.54% | 8.39% | 76% | 99% |
| Overall | 7.75% | 17.99% | 18.92% | 75% | 82% |
| Class D | 12.36% | 21.23% | 22.28% | 58% | 83% |
| Class F | 25.63% | 34.10% | 39.99% | 47% | 82% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.70% | 6.97% | 7.33% | 76% | 105% |
| Class A2 | 3.84% | 10.41% | 7.46% | 76% | 94% |
| Class B | 5.05% | 10.50% | 10.64% | 77% | 88% |
| Class C | 12.33% | 16.32% | 18.44% | 76% | 81% |
| Class E |  |  |  |  |  |
| Overall | 6.28% | 11.33% | 11.42% | 77% | 90% |
| Class D | 13.32% | 20.95% | 22.27% | 78% | 81% |
| Class F | 21.68% | 24.39% | 27.32% | 73% | 96% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 8.00% | 9.98% | 11.33% | 75% | 93% |
| Class E | 3.05% | 3.50% | 5.03% | 83% | 96% |
| Overall |  |  |  |  |  |
| Class D | 9.54% | 11.91% | 13.72% | 77% | 100% |
| Class F | 13.50% | 15.08% | 16.45% | 76% | 91% |

Table 3.  The results of Test 3 using Test 1 as anchor. Note that the runtime is not accurate.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.52% | -14.26% | -15.69% | 14% | 97% |
| Class A2 | 1.20% | -0.61% | 1.97% | 13% | 108% |
| Class B | 0.64% | -0.18% | -0.71% | 13% | 111% |
| Class C | -3.21% | -8.63% | -8.88% | 12% | 122% |
| Class E | 1.38% | 5.51% | 4.83% | 15% | 116% |
| Overall | -0.36% | -3.53% | -3.65% | 13% | 111% |
| Class D | -3.13% | -6.62% | -7.32% | 14% | 118% |
| Class F | -6.65% | -10.48% | -13.20% | 22% | 123% |
|  |  |  | Random access Main10 |  |  |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.48% | -7.03% | -7.48% | 46% | 109% |
| Class A2 | -8.41% | -8.66% | -6.90% | 49% | 106% |
| Class B | -13.60% | -8.62% | -8.60% | 49% | 102% |
| Class C | -29.49% | -23.62% | -26.11% | 36% | 126% |
| Class E |  |  |  |  |  |
| Overall | -14.97% | -12.31% | -12.71% | 45% | 110% |
| Class D | -25.80% | -19.57% | -20.25% | 35% | 123% |
| Class F | -22.61% | -19.97% | -21.87% | 52% | 123% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -14.32% | -4.15% | -3.77% | 54% | 121% |
| Class C | -33.70% | -19.45% | -21.19% | 40% | 116% |
| Class E | -13.73% | -4.88% | -5.01% | 64% | 108% |
| Overall | -20.63% | -9.43% | -9.89% | 51% | 116% |
| Class D | -29.84% | -15.85% | -17.55% | 37% | 113% |
| Class F | -35.06% | -22.73% | -24.09% | 67% | 112% |

References

[1] H. Huang, J. Chen, W.-J. Chien, and M. Karczewicz, “AHG2: On minimum QT size, maximum BT size and maximum TT size,” Document of Joint Video Experts Team, JVET-R0347.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).