| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0480-v1 |
| --- | --- |

| Title: | AHG9: Restricted maximum numbers of ALF and CC-ALF filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang1, Ye-Kui Wang1, Kai Zhang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contributions proposes the following aspects regarding restrictions of the maximum numbers of ALF and CC-ALF filters:

- Replace the constraints on the number of ALF APSs with constraints on the number of filters, more specifically, the following constrain are proposed to be added:

The total number of adpative loop filter classes for luma component, the total number of alternative filters for chroma components, and the total number of cross-component filters in all APS NAL units for a PU shall be less than or equal to 200, 64, and 64, respectively.

- On top of item 1), further change the coding of the APS ID in the APS syntax from u(5) to u(v), with the length being 9, 2, and 3, respectively for ALF, LMCS, and scaling list APSs, respectively.

- On top of item 1), further change the coding of ALF APS indices and the number of ALF APSs in PH and SH from u(v) to ue(v).

# Introduction

In current VVC specification, the filtering information for ALF and CC-ALF are signalled in ALF APSs. The corresponding ALF data syntax is defined as follows:

# 7.3.2.19	Adaptive loop filter data syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | ue(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| if( alf_luma_clip_flag ) |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | ue(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag ) |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| if( alf_cc_cb_filter_signal_flag ) { |  |
| alf_cc_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cb_filters_signalled_minus1 + 1; k++ ) { |  |
| for( j = 0; j < 7; j++ ) { |  |
| alf_cc_cb_mapped_coeff_abs[ k ][ j ] | u(3) |
| if( alf_cc_cb_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cb_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_cc_cr_filter_signal_flag ) { |  |
| alf_cc_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < alf_cc_cr_filters_signalled_minus1 + 1; k++ ) { |  |
| for( j = 0; j < 7; j++ ) { |  |
| alf_cc_cr_mapped_coeff_abs[ k ][ j ] | u(3) |
| if( alf_cc_cr_mapped_coeff_abs[ k ][ j ] ) |  |
| alf_cc_cr_coeff_sign[ k ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

Based on the above syntax, the requried momery for storing filtering information within a ALF APS is about (3000 + 480 + 112 + 112) = 3704 bits. And for a PU, there are up to 8 ALF APSs, which means more than 3704×8bits are required (there are a few more bits for storing APS indices required) for handling a PU with the loop filtering process. Detailed information about how to get the number is tabulated in Table 1.

Table 1. Memory requirement for an ALF APS

|  | Max. Number of filters | Number of coefficients per filter | Bits associated with each coefficient | Total (bits) |
| --- | --- | --- | --- | --- |
| Luma ALF | 25 | 12 | 8 (coeff) + 2 (clipping index) = 10 | 3000 |
| Chroma ALF | 8 | 6 | 8 (coeff) + 2 (clipping index) = 10 | 480 |
| CC-ALF for Cb | 4 | 7 | 4 | 112 |
| CC-ALF for Cr | 4 | 7 | 4 | 112 |
| Total | 3704 bits |  |  |  |

To make a balance between coding performance of ALF/CC-ALF and memory requirement for storing the filters, currently up to 8 ALF APSs are allowed for a PU. However, such a restriction may have the following problems:

- The limitation of 8 ALF APSs may restrict the coding efficiency for multiple layers/slices/subpicture cases, especially when the number of slices or subpictures within a PU is greater than 8. Furthermore, this essentially disallows practically meaningful use of ALF in parallel live encoding when each picture is split into more a large number of (e.g., 96 as in OMAF use cases) subpictures before encoding, then separately encoded, and later merged into single coded picture with multile coded subpictures.

- In an ALF APS, up to 25 luma ALF filters could be signalled. However, according to statistics, the probability of signalling a large number of filters (e.g., more than 16 filters) within an ALF APS is relatively low. Therefore, restricting the total number of ALF APSs is less efficient since the bits of the actual transmitted ALF APSs may be less than the maximum allowed bits.

# Proposal

## Summary of the proposal

To resolve the above issues, it is proposed to replace the constraints on the number of ALF APSs with constraints on the number of filters, more specifically, the following constrain are added:

The total number of adpative loop filter classes for luma component, the total number of alternative filters for chroma components, and the total number of cross-component filters in all APS NAL units for a PU shall be less than or equal to 200 (25×8), 64 (8×8), and 64 (8×8), respectively.

In addition, the coding of APS index in the APS syntax, ALF APS indices and the number of ALF APSs in PH/SH are further revised accordingly.

Meanwhile, to make sure the total required meomry does not exceed the current design, the maximum number of APSs is derived to be (200 + 64 + 64) = 328, assuming that the minimum number of filters in an ALF APS is only one.

It is claimed that, with the suggested changes, the encoder is allowed to choose from a larger number of ALF APSs for coding a picture/slice. Therefore, higher coding performance could be expected and practically meaningful use of ALF in parallel live encoding with a large number of subpictures per picture is eanbled. In addition, the total memeory is smaller than the current design in the worst case.

## Proposed text changes

Note that all of the following changes are based on the assumption that up to 328 ALF APSs is allowed for a PU. However, the maximum value of allowed ALF APSs could be discussed.

7.3.2.5	Adaptation parameter set RBSP syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| adaptation_parameter_set_id | u(v) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && alf_info_in_ph_flag ) { |  |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | ue(v3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | ue(v3) |
| if( ChromaArrayType != 0 ) |  |
| ph_alf_chroma_idc | u(2) |
| if( ph_alf_chroma_idc > 0 ) |  |
| ph_alf_aps_id_chroma | ue(v3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | ue(v3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | ue(v3) |
| } |  |
| } |  |
| } |  |
| … |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !alf_info_in_ph_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | ue(v3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | ue(v3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | ue(v3) |
| if( sps_ccalf_enabled_flag ) { |  |
| slice_cc_alf_cb_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | ue(v3) |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | ue(v3) |
| } |  |
| } |  |
| } |  |
| … |  |

7.4.3.5	Adaptation parameter set semantics

Each APS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements. The length of the syntax element adaptation_parameter_set_id, in bits, is aps_params_type   =  ALF_APS ? 9 : (aps_params_type  = =  LMCS_APS ? 2 : 3 ). [Ed.: Move the semantics of adaptation_parameter_set_id to be after the semantics of aps_params_type.]

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7 327, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Let apsLayerId be the value of the nuh_layer_id of a particular APS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular APS NAL unit unless apsLayerId is less than or equal to vclLayerId and the layer with nuh_layer_id equal to apsLayerId is included in at least one OLS that includes the layer with nuh_layer_id equal to vclLayerId.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

…

It is a requirement of bistream conformance that, the total number of adpative loop filter classes for luma component, the total number of alternative filters for chroma components, and the total number of cross-component filters in all APS NAL units with a PU shall be less than or equal to 200, 64, and 64, respectively.

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id.

7.4.3.7	Picture header structure semantics

ph_num_alf_aps_ids_luma specifies the number of ALF APSs that the slices associated with the PH refers to. The value of ph_num_alf_aps_ids_luma shall be in the range of 0 to 200, inclusive.

ph_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to. The value of ph_alf_aps_id_luma[ i ] shall be in the range of 0 to 327, inclusive.

ph_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to. The value of ph_alf_aps_id_chroma[ i ] shall be in the range of 0 to 327, inclusive.

ph_cc_alf_cb_aps_id specifies the adaptation_parameter_set_id of the ALF APS that the Cb colour component of the slices associated with the PH refers to. The value of ph_alf_aps_id_chroma[ i ] shall be in the range of 0 to 327, inclusive.

ph_cc_alf_cr_aps_id specifies the adaptation_parameter_set_id of the ALF APS that the Cr colour component of the slices associated with the PH refers to. The value of ph_alf_aps_id_chroma[ i ] shall be in the range of 0 to 327, inclusive.

…

7.4.8.1	General slice header semantics

slice_num_alf_aps_ids_luma specifies the number of ALF APSs that the slice refers to. When slice_alf_enabled_flag is equal to 1 and slice_num_alf_aps_ids_luma is not present, the value of slice_num_alf_aps_ids_luma is inferred to be equal to the value of ph_num_alf_aps_ids_luma. The value of slice_num_alf_aps_ids_luma shall be in the range of 0 to 200, inclusive.

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ]. The value of slice_alf_aps_id_luma[ i ] shall be in the range of 0 to 327, inclusive.

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of ph_alf_aps_id_chroma. The value of slice_alf_aps_id_chroma shall be in the range of 0 to 327, inclusive.

slice_cc_alf_cb_aps_id specifies the adaptation_parameter_set_id that the Cb colour component of the slice refers to.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_cc_alf_cb_enabled_flag is equal to 1 and slice_cc_alf_cb_aps_id is not present, the value of slice_cc_alf_cb_aps_id is inferred to be equal to the value of ph_cc_alf_cb_aps_id. The value of slice_cc_alf_cb_aps_id shall be in the range of 0 to 327, inclusive.

The value of alf_cc_cb_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be equal to 1.

slice_cc_alf_cr_aps_id specifies the adaptation_parameter_set_id that the Cr colour component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_cc_alf_cr_enabled_flag is equal to 1 and slice_cc_alf_cr_aps_id is not present, the value of slice_cc_alf_cr_aps_id is inferred to be equal to the value of ph_cc_alf_cr_aps_id. The value of slice_cc_alf_cr_aps_id shall be in the range of 0 to 327, inclusive.

The value of alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be equal to 1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).