| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0489 |
| --- | --- |

| Title: | Cross-check of JVET-R0076 (AHG9/AHG15: Chroma QP mapping table cleanups) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian 5775 Morehouse Dr San Diego, CA, USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document provides a partial cross-check of the results presented in Section 2.4 of JVET-R0076-v3. The cross-check covers the proposal to modify the chroma QP mapping table specified for the AI, RA and LDB common test conditions. For simulations that are completed, it is reported that the results of the cross-checker match that of the authors provided in Section 2.4 of JVET-R0076-v3 and corresponding Excel files (excluding some negligible differences due to rounding in the RA configuration).

# Introduction

The JVET CTC uses chroma QP mapping tables to derive the chroma QP value from the luma QP values. The mapping table is signalled in the bitstream. The VTM-8.0 parameters the chroma QP mapping tables are specified in the configuration files as follows:

- AI:

  - QpInValCb: 1 31 43
QpOutValCb: 1 32 41

- RA/LDB/LDP

  - QpInValCb: 32 44
QpOutValCb: 32 41

The same mapping table is used for both the chroma components.

# Test

Section 2.4 of JVET-R0076-v3 proposes to modify the chroma QP mapping tables for the SDR CTC. Two sets of parameters are proposed: Setting1 and Setting2. The tables used for the AI configuration are the same in both Setting1 and Setting2. The corresponding VTM parameters are as follows:

Setting 1

- AI:

  - QpInValCb: 0 20 33 37
QpOutValCb: 0 22 35 38

- RA/LDB/LDP

  - QpInValCb: 12 60
QpOutValCb: 12 60

Setting 2

- AI:

  - QpInValCb: 0 20 33 37
QpOutValCb: 0 22 35 38

- RA/LDB/LDP

  - QpInValCb: 29 30
QpOutValCb: 29 31

An illustration of the mapping tables are provided in the following figures. The figures show that the chroma QPs are slightly higher for the proposed settings compared to the anchor, which would indicate that the proposed configurations will have relatively more bits (or rather ratio of bits) spent for luma than chroma when compared to the anchoranchor; the BD-rate will also be affected accordingly.

Figure 1: Comparison of chroma QP mapping table for the AI configuration

Figure 2: Comparison of chroma QP mapping table for the AI configuration

# Simulation results

The simulation results obtained by testing the configurations Setting1 and Setting2 on VTM-8.0 for AI, RA and LD configurations. The bitrate and PSNR for the rate points that are completed in the cross-checker’s simulation match the results provided by the authors of JVET-R0076-v3; the PSNR values of few points in the RA configuration have shown some difference in the fourth decimal place but the cross-checkers assert that this is mostly due to difference in rounding calculations and a not a fundamental difference in the results. These difference would not modify the BD-rate values. The attached Excel file provides the rate points that are completed by the cross-checker.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).