| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R2009 |
| --- | --- |

| Title: | Draft plan for VVC verification testing (Draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft Verification Test Plan text |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Andrew Segall Yan Ye | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com asegall@sharplabs.com yan.ye@alibaba-inc.com |
| Source: | AHG |  |  |

_____________________________

# Introduction

This document contains the draft plan for the video verification test to be conducted to verify the coding performance of the VVC Main profile. A formal subjective evaluation will be conducted comparing the VVC Main profile to the HEVC Main 10 profile.

Verification testing is planned with first priority for the following categories:

- SDR HD, SDR UHD

- HDR HD, HDR UHD

- 360° (extracted viewports)

Verification testing is planned with secondary priority for the following categories:

- Screen content (perhaps with 2 HEVC anchors – Main 10 and SCC profiles)

- Scalability

- 4:4:4 content

# Schedule

The first formal test is planned to be completed by October 2020.

Test Sequences Identified: T-X weeks

Encoding of sequences completed: T

Subjective evaluation starts: T+1 weeks

Subjective evaluation completed: T+2 weeks

Subjective evaluation results available: T +4 weeks.

# Test sites

[tbd]

# Test categories, coding conditions, and test sequences

Test cases for standard dynamic range (SDR), high dynamic range (HDR), and 360° video content are defined for the verification tests. The test conditions and configurations for these cases are defined below.

## Standard Dynamic Range

### Coding conditions

The following test conditions will be used for the VVC verification test.

- Number of sequences and video resolutions:

  - X sequences for HD (1920x1080) and UHD (3840x2160) resolutions each

- Bitstreams

  - Generated with VTM X.Y for VVC bitstreams

  - Generated with HM 16.20 for HEVC bitstreams

  - In addition to a. and b., other VVC and/or HEVC bitstreams generated with encoders that are optimized for subjective quality may be tested if available.

- Encoding parameters

  - Fixed QP.

    - 4 bit rate points per sequences covering the whole MOS range as much as possible with QP values for the HM and the VTM. The QPs are to be selected such that the subjectively assessed quality is comparable between the two test models.

  - Bit depth of 10bits for all video resolutions

  - Coding structure depending on the nature of the sequence.

    - UHD using Random access, RA (Storage/Streaming)

      - Intra refresh at approximately 1 second intervals.

      - Picture reordering allowed.

    - HD using Low delay, LD (Video conferencing)

      - No Intra refresh.

      - Without picture reordering.

  - Other settings as in the configuration files

    - VTM: 	cfg/encoder_randomaccess_vtm.cfg, and cfg/ encoder_lowdelay_vtm.cfg

    - HM: 	cfg/encoder_randomaccess_main10.cfg and encoder_lowdelay_main.cfg

### Test sequences

A set of test sequence candidates in UHD resolution for the SDR category as established at the 18th JVET meeting is listed in the table below.

Table 1– SDR UHD test sequences under consideration

| No. | Test sequence | Frames |
| --- | --- | --- |
| 01 | Marathon_3840x2160_30fps_8bit_420pf.yuv | 0:299 (all) |
| 02 | MountainBay2_3840x2160_30fps_420_10bit.yuv | 0:299 (all) |
| 03 | Netflix_DrivingPOV_4096x2160_60fps_10bit_420.yuv | 0:599 |
| 04 | Runners_3840x2160_30fps_8bit_420pf.yuv | 0:299 (all) |
| 05 | TallBuildings_3840x2160_30fps_8bit_420pf.yuv1) | 0:299 (all) |
| 06 | TrafficFlow_3840x2160_30fps_8bit_420pf.yuv1) | 0:299 (all) |
| 07 | RaceNight_3840x2160_50fps_10bit.yuv | 0:499 (all) |
| 08 | BodeMuseum_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |
| 09 | NeptuneFountain3_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |
| 10 | OberbaumSpree_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |
| 11 | QuadrigaTree_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |
| 12 | SubwayTree_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |
| 13 | TiergartenParkway_3840x2160_60fps_420_10bit_qp262) | 0:599 (all) |

1)  For the sequences TallBuildings and TrafficFlow, additional results for 10 bit versions of the sequences were provided.
2) Sequences made available during the 18th JVET meeting by HHI

Indicative rate-distortion plots showing luma PSNR over bitrate for HM-16.20 encodings using RA configuration for QP=24:1:46 for all sequences are provided below. HM and VTM bitstreams of these sequences are made available on the JVET ftp site in the directory /ahg/verificationtests/JVET-R_VT/.

Figure 1 – Rate-distortion plot for test sequences 01-07. PSNR-Y over log bitrate

Figure 2 – Rate-distortion plot for test sequences 08-13. PSNR-Y over log bitrate

### Timeline and methodology for selection of candidate test sequences

For each candidate test sequence, the rate points to be considered shall be selected such that the full quality range from close-to-transparent (expected DMOS ~8) down to almost bad quality (expected DMOS ~3) is covered. Two types of comparison are considered: Quality improvement at identical rate, and rate saving at identical quality.

For quality improvement comparison at same rate, the lowest point should be selected such that VTM still has somewhat acceptable quality (as would be used by typical application), and HM would show more prominent impairments at the same rate. The highest point should be that VTM starts becoming transparent, and HM still shows artifacts. For the rate comparison, the lowest/highest quality point selection should be identical, but as HM should have somewhat similar quality as VTM, the HM quality could be used as starting point.

The flowing steps are planned to be pursued for achieving this task

- Encoding. Bitstreams for HM-16.20 and VTM-8.0 in Random Access configuration with the IRAP interval set to 24,32,48,64,128 for sequences at 24/25Hz, 30Hz, 50Hz, 60Hz, and 120Hz, respectively. An initial QP range of QP=24 to QP=50 with a step size of 1 for the HM and QP=26 to QP=50 with a step size of 4 for the VTM is considered. This range may be extended and refined depending on the visual assessment of the sequences.

- Selection. Based on data available at the 18th JVET meeting, the test coordinators preselect rate points for the sequences covering the specified quality and rate ranges.

- Packing. Viewing of raw YUV files under laboratory conditions is not expected to be available under self-isolation rules established in most countries. Therefore, the HM and VTM bitstreams are to be packed in a format suitable for remote assessment without the need for professional viewing equipment.

  - HM bistreams can be packed into mp4 container e.g. using ffmpeg with the following command line
ffmpeg -y -r <fps> -i <hevc.bin> -vcodec copy <out.mp4>

  - VTM bitstreams need to be transcoded at sufficiently high quality into a format decodable by conventional video players on a state-of-the-art PC. A possible transcoding operation can be achieved with ffmpeg using the following command line (note that mkv used)
ffmpeg -video_size <pix>x<lin> -pix_fmt yuv420p10le -r <fps> -i <VTMyuvoutput> -c:v libx265 -crf 5 <out.mkv>

- Viewing. Packed files are prepared by the test coordinators and distributed to participating experts who are equipped with a computer capable of providing fluid playback of the video sequences on a monitor (or TV set) of at least 32” size; the recommended set-up includes a UHD 55” OLED TV set (regardless the brand). The viewing is performed during an online meeting with moderation by the test coordinators. Observations and recommendations are collected and documented to form the basis for the selection of candidate sequences and candidate rate points.

- Dry Run. A dry run with the recommended candidate sequences on the candidate rate points is to be performed (in a way permitted by local self-isolation rules) for confirmation of the suitability of the proposed selection.

- Report. A report on the activity is produced as input to the next meeting.

Timeline

- Selection completed: May 5th 2020

- (Re-)Encoding completed: May 11th 2020

- Packing completed: May 12th 2020

- Online Viewing Sessions with invited experts: during May 12-15th 2020, duration 2h max, precise time and schedule tbd and agreed among participants. One or more viewing sessions are expected to be scheduled.

- An AHG session on the results is scheduled on Fri 15th 2020.

- Dry run with 4 test sequences coded at 4 bitrates: Conducted in Week June 1-5th 2020
Conduction of additional encoding and calibration tasks may be necessary in preparation of the dry run.

- Report on activity by input document to the 19th JVET meeting.

## High Dynamic Range

The goal of the High Dynamic Range verification test is to evaluate and verify the performance of VVC for content represented with both HLG and SMPTE ST 2084 transfer functions.

### Coding conditions

The following test conditions will be used for the High Dynamic Range verification test.

- Number of sequences and video resolutions:

  - 15 sequences represented with the HLG transfer function and with a spatial resolution of either HD (1920x1080) or UHD (3840x2160)

  - 29 sequences represented with the SMPTE ST 2084 transfer function and with a spatial resolution of either HD (1920x1080) or UHD (3840x2160)

- Bitstreams

  - Generated with VTM 8.2 for VVC bitstreams, where the VTM is configured for HDR content according to the configuration defined in JVET-P2011.

  - Generated with HM 16.20 for HEVC bitstreams, where the HM is configured for HDR content according to the configuration defined in ISO/IEC TR 23008-14:2018.

  - In addition to a. and b., other VVC and/or HEVC bitstreams generated with encoders that are optimized for subjective quality may be tested if available.

- Encoding parameters

  - Fixed QP.

    - 4 bit rate points per sequences covering the whole MOS range as much as possible with QP values for the HM and the VTM. The QPs are to be selected such that the subjectively assessed quality is comparable between the two test models.

  - Bit depth of 10bits for all video resolutions

  - Coding structure depending on the nature of the sequence.

    - Random access, RA (Storage/Streaming)

      - Intra refresh at approximately 1 second intervals.

      - Picture reordering allowed.

  - Other settings as in the configuration files

    - VTM: 	Configured for HDR content according to the configuration defined in JVET-2011.

    - HM: 	Configured for HDR content according to the configuration defined in ISO/IEC TR 23008-14:2018.

### Test sequences

A set of test sequence candidates for the HDR category as established at the 18th JVET meeting is listed in the table below.

Table 1 – HDR UHD test sequences under consideration

| No. | Test sequence | Transfer Function |
| --- | --- | --- |
| 01 | Chimera HDR1 | SMPTE ST 2084 |
| 02 | Chimera HDR2 | SMPTE ST 2084 |
| 03 | Chimera HDR3 | SMPTE ST 2084 |
| 04 | Chimera HDR4 | SMPTE ST 2084 |
| 05 | Chimera HDR5 | SMPTE ST 2084 |
| 06 | Chimera HDR6 | SMPTE ST 2084 |
| 07 | Chimera HDR7 | SMPTE ST 2084 |
| 08 | Chimera HDR8 | SMPTE ST 2084 |
| 09 | Chimera HDR9 | SMPTE ST 2084 |
| 10 | Cosmos HDR1 | SMPTE ST 2084 |
| 11 | Cosmos HDR2 | SMPTE ST 2084 |
| 12 | Cosmos HDR3 | SMPTE ST 2084 |
| 13 | Cosmos HDR4 | SMPTE ST 2084 |
| 14 | Cosmos HDR5 | SMPTE ST 2084 |
| 15 | Cosmos HDR6 | SMPTE ST 2084 |
| 16 | Cosmos HDR7 | SMPTE ST 2084 |
| 17 | CosmosLaundromatHDR 1 | SMPTE ST 2084 |
| 18 | CosmosLaundromatHDR 2 | SMPTE ST 2084 |
| 19 | CosmosLaundromatHDR 3 | SMPTE ST 2084 |
| 20 | CosmosLaundromatHDR 4 | SMPTE ST 2084 |
| 21 | CosmosLaundromatHDR 5 | SMPTE ST 2084 |
| 22 | CosmosLaundromatHDR 6 | SMPTE ST 2084 |
| 23 | HLG1_10s | HLG |
| 24 | HLG2_10s | HLG |
| 25 | HLG3_10s | HLG |
| 26 | HLG4_10s | HLG |
| 27 | HLG5_10s | HLG |
| 28 | HLG6_10s | HLG |
| 29 | HLG7_10s | HLG |
| 30 | Meridian HDR 1 | SMPTE ST 2084 |
| 31 | Meridian HDR2 | SMPTE ST 2084 |
| 32 | Meridian HDR3 | SMPTE ST 2084 |
| 33 | Meridian HDR4 | SMPTE ST 2084 |
| 34 | Meridian HDR5 | SMPTE ST 2084 |
| 35 | Meridian HDR6 | SMPTE ST 2084 |
| 36 | Meridian HDR7 | SMPTE ST 2084 |
| 37 | HLG_NightStreet | HLG |
| 38 | HLG_SeaSurface | HLG |
| 39 | HLG_StainedGlass | HLG |
| 40 | H3S01 | HLG |
| 41 | H3S02 | HLG |
| 42 | H3S03 | HLG |
| 43 | H3S04 | HLG |
| 44 | H3S05 | HLG |

### Timeline and methodology for selection of candidate test sequences

For each candidate test sequence, the rate points to be considered shall be selected such that the full quality range from close-to-transparent (expected DMOS ~8) down to almost bad quality (expected DMOS ~3) is covered. Two types of comparison are considered: Quality improvement at identical rate, and rate saving at identical quality.

For quality improvement comparison at same rate, the lowest point should be selected such that VTM still has somewhat acceptable quality (as would be used by typical application), and HM would show more prominent impairments at the same rate. The highest point should be that VTM starts becoming transparent, and HM still shows artifacts. For the rate comparison, the lowest/highest quality point selection should be identical, but as HM should have somewhat similar quality as VTM, the HM quality could be used as starting point.

The flowing steps are planned to be pursued for achieving this task

- Initial Encoding. Bitstreams for HM-16.20 in Random Access configuration with the IRAP interval set to 24,32,48,64,128 for sequences at 24/25Hz, 30Hz, 50Hz, 60Hz, and 120Hz, respectively. An initial QP range of QP=22 to QP=37 with a step size of 5 for the HM is considered. This range may be extended and refined depending on the visual assessment of the sequences.

- Pre-Screening.  Using the objective generated by the initial encoding, the sequences will be pre-screened and at least 5 SMPTE ST 2084 and 5 HLG sequences will be selected for further investigation.

- Encoding. Bitstreams for HM-16.20 and VTM-8.0 in Random Access configuration with the IRAP interval set to 24,32,48,64,128 for the sequences selected during pre-screening at 24/25Hz, 30Hz, 50Hz, 60Hz, and 120Hz, respectively. An initial QP range of QP=24 to QP=46 with a step size of 1 for the HM and QP=26 to QP=46 with a step size of 4 for the VTM is considered. This range may be extended and refined depending on the visual assessment of the sequences.

- Selection. Based on data generated by the encoding, the test coordinators preselect rate points for the sequences covering the specified quality and rate ranges.

- Packing. Viewing of raw YUV files under laboratory conditions is not expected to be available under self-isolation rules established in most countries. Therefore, the HM and VTM bitstreams are to be packed in a format suitable for remote assessment without the need for professional viewing equipment.

- Viewing. Packed files are prepared by the test coordinators and distributed to participating experts who are equipped with a computer capable of providing fluid playback of the video sequences on a monitor (or TV set) of at least 32” size; the recommended set-up will be defined as part of the workplan but is anticipated to a UHD OLED TV with HDR capabilities.  The viewing is performed during an online meeting with moderation by the test coordinators. Observations and recommendations are collected and documented to form the basis for the selection of candidate sequences and candidate rate points.

- Report. A report on the activity is produced as input to the next meeting.

Timeline

- Initial encoding completed: May 8th 2020

- Pre-Screening completed: May 13th 2020

- Encoding completed: May 20th 2020

- Selection completed: May 27th 2020

- Teleconference: May 28th 2020

- (Re-)Encoding and packing completed: May 29th 2020

- Initial viewing completed: June 15th 2020

- Completion of report to be input to the 19th JVET meeting: June 16th 2020

## 360° Video

### Coding conditions

The following test conditions will be used for the VVC verification test.

- Number of sequences and video resolutions:

  - 9 sequences at different input resolutions, 8192x4096, 4096x2048, 4320x2160, and 6144x3072

- Bitstreams

  - Generated with VTM 8.0 and 360Lib 10.1 for VVC bitstreams, in two projection formats, padded equirectangular (PERP) and padded hybrid equiangular cubemap (PHEC)

  - Generated with HM 16.20 and patched version of 360Lib 5.0 for HEVC bitstreams, in two projection formats, PERP and cubemap (CMP)

- Encoding parameters

  - Fixed QP.

    - 4 bit rate points per sequence covering the whole MOS range as much as possible with QP values for the HM and the VTM. The QPs are to be selected such that the subjectively assessed quality is comparable between the two test models.

  - Coding bit depth of 10 bits for all video resolutions, coding resolution depends on the input resolution and coding projection format, as listed in Table 2:

Table 2– Coding resolution for different input resolutions and coding projection formats

|  | CMP/HEC (before padding) | ERP (before padding) |
| --- | --- | --- |
| 4K (4096x2048 & 4320x2160) | 1184 x 1184 per face --CodingFaceWidth=1184 --CodingFaceHeight=1184 | 4096x2048 --CodingFaceWidth=4096 --CodingFaceHeight=2048 --WrapAroundOffset=4096 (only for VTM) |
| 6K & 8K | 1280 x 1280 per face --CodingFaceWidth=1280 --CodingFaceHeight=1280 | 4432x 2216 --CodingFaceWidth=4432 --CodingFaceHeight=2216 --WrapAroundOffset=4432 (only for VTM) |

  - Random access, RA (Storage/Streaming) coding structure will be used:

      - Intra refresh at approximately 1 second intervals.

      - Picture reordering allowed.

  - Other settings as in the configuration files

    - VTM: 	cfg/encoder_randomaccess_vtm.cfg

    - HM: 	cfg/encoder_randomaccess_main10.cfg

    - 360Lib: cfg-360Lib/ encoder_360_PERP.cfg,

cfg-360Lib/ encoder_360_CMP.cfg,

cfg-360Lib/ encoder_360_HEC.cfg

### Test sequences

A set of	360° video test sequence candidates as established at the 18th JVET meeting is listed in Table 3.

Table 3– 360 video test sequences under consideration

| No. | Sequence | Frame rate | Resolution | Provider | BD | Camera |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | skateboarding | 60 | 4096x2048 | GoPro | 8 | static |
| 02 | GT_Sheriff | 30 | 4320x2160 | Nokia | 8 | static |
| 03 | basketball | 30 | 8192x4096 | GoPro | 10 | static |
| 04 | jam_session | 30 | 8192x4096 | GoPro | 8 | static |
| 05 | SkateboardTrick | 60 | 8192x4096 | GoPro | 8 | static |
| 06 | Train | 60 | 8192x4096 | GoPro | 8 | static |
| 07 | SkateBoardAtBridge | 30 | 6144x3072 | GoPro | 8 | moving |
| 08 | HarborBiking | 30 | 8192x4096 | InterDigital | 8 | moving |
| 09 | KiteFliteWalking | 30 | 8192x4096 | InterDigital | 8 | moving |

Indicative rate-distortion plots showing end-to-end weighted spherical PSNR (E2E WSPSNR) over bitrate for HM-16.20 encodings using RA configuration for QP=24:1:46 for all sequences are to be provided.

Static and dynamic viewports with 78.1×49.1 degrees of field of view will be used to generate viewport video at the resolution of 1920×1080 and viewed on conventional HD displays.

### Timeline and methodology for selection of candidate sequences

The general methodology for sequence selection as outlined in 4.1.3 applies to the 360° video category as well, except for the following additional considerations that have been given to the 360° video category:

- Encoding. Bitstreams for HM-16.20 and VTM-8.0 in Random Access configuration with the IRAP interval set to 32 and 64 for sequences at 30Hz and 60Hz, respectively. An initial QP range of QP=24 to QP=46 with a step size of 1 for the HM and QP=26 to QP=46 with a step size of 4 for the VTM is considered. For projection format conversion and E2E WSPSNR calculation, 360Lib-10.1 is used fpr VTM-8.0 and a patched version of 360Lib-5.0 is used for HM-16.20. Coding resolution depends on input resolution and projection format, as listed in Table 2. Regardless of input bit depth, the coding bit depth is set to10 bits.

- Sequence, rate and viewport selection. Upon completion of encoding, the test coordinators preselect rate points for the sequences covering the specified quality and rate ranges. The viewport selection volunteers will select two appropriate viewports (one static and one dynamic) per sequence to generate viewport videos at resolution of 1920x1080.

- Packing. The same protocols as in 4.1.3 will be used.

- Viewing. The same viewing protocols as those for HD SDR sequences are planned to be used for 360° video.

- Report. The activity will be included in the 360 video part of the report document at the next meeting.

Timeline

- Encoding completed: May 8, 2020

- Preliminary viewport selection completed: May 22, 2020

- Teleconference: May 27th, 2020

- (Re-)Encoding completed (if necessary): June 5, 2020

- Viewing: June 8-12th, 2020

- Completion of report to be input to the 19th JVET meeting: June 16th 2020

# Encoding results

By the release of this document, preliminary rate-distortion data for the UHD SDR category were available. A collection of these rate-distortion measurements is provided in the accompanying Excel sheet. The data is further provided in xml format to be used for display with the RDplot tool (https://github.com/IENT/RDPlot). With this tool, rate-distortion plots and indicative BD-rate and BD-PSNR measurements can be generated.

A comprehensive set of simulation results and rate-distortion data will be provided in the verification test preparation report to be input to the next meeting.

# Annex A – Encoding tasks and candidate selection

## SDR

Thanks to Alibaba, ByteDance, Ericsson, HHI, Huawei, and Tencent are expressed for volunteering for the encoding tasks.

## HDR

Thanks to Dolby, Ericsson, InterDigital, NHK, Qualcomm, Sharp and SONY are expressed for volunteering for the encoding tasks.

## 360° Video

Thanks to Alibaba, Bytedance, InterDigital, MediaTek, and RWTH Aachen University are expressed for volunteering for the encoding tasks.

Thanks to Intel, RWTH Aachen University, and VABTECH are expressed for volunteering for the viewport selection tasks.