| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0055 |
| --- | --- |

| Title: | AHG9: On signaling IRAP and GDR pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The following semantics modifications on ph_gdr_or_irap_pic_flag are proposed:

- The constraints of the semantics of ph_gdr_or_irap_pic_flag are specified as two way, to mandate the presence of ph_no_output_of_prior_pics_flag, when the current picture is an IRAP picture.

- The value of ph_gdr_or_irap_pic_flag is set equal to aud_irap_or_gdr_au_flag, when aud_irap_or_gdr_au_flag is present.

# Proposal-1

In the latest VVC draft (JVET-R2001-v10), the flag ph_no_output_of_prior_pics_flag is signaled in picture header and used for output and removal process of pictures from the DPB. However, ph_no_output_of_prior_pics_flag is present in picture header, only when ph_gdr_or_irap_pic_flag is equal to 1. The asserted problem is that an IRAP picture may have ph_gdr_or_irap_pic_flag equal to 0, because the constraint specified by the semantics of ph_gdr_or_irap_pic_flag is one way, as follows:

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture.

When the value of ph_gdr_or_irap_pic_flag of an IRAP picture is equal to 0, the value of ph_no_output_of_prior_pics_flag is used for DPB operations, without signaling or any inference rule.

To address this issue, it is proposed to specify the constraints of the semantics of ph_gdr_or_irap_pic_flag as two way, to mandate the presence of ph_no_output_of_prior_pics_flag, when the current picture is an IRAP picture, as follows:

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR or IRAP picture. and may or may not be an IRAP picture.

As an alternative way, an inference rule of the ph_no_output_of_prior_pics_flag value can be specified, as follows:

ph_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C. When not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0.

However, it is recommended to adopt the first method that specifies the constraints of the semantics of ph_gdr_or_irap_pic_flag as two way, so that the value of ph_gdr_or_irap_pic_flag can be used for detection of the GDR or IRAP picture.

Proposed Text (option 1):

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR or IRAP picture. and may or may not be an IRAP picture.

Proposed Text (option 2):

ph_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C. When not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0.

# Proposal-2

The flag aud_irap_or_gdr_au_flag that indicates the presence of IRAP or GDR AU is present in AUD, while ph_gdr_or_irap_pic_flag is present in PH, to indicate the presence of IRAP or GDR picture. It is asserted that the value of ph_gdr_or_irap_pic_flag shall be algined to the value of aud_irap_or_gdr_au_flag, to avoid any conflict or confusion between IRAP or GDR AU indication/detection and IRAP or GDR picture indication/detection. Note that the flag aud_irap_or_gdr_au_flag of an IRAP or GDR AU shall be present, when the bitstream has multiple layers, with sps_video_parameter_set_id is greater than 0.

Proposed Text:

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture.

When aud_irap_or_gdr_au_flag is present and the value of aud_irap_or_gdr_au_flag is equal to 1, the value of ph_gdr_or_irap_pic_flag shall be equal to 1.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).