| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0068-v1 |
| --- | --- |

| Title: | AHG9: Editorial improvements on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Olena Chubach, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, olena.chubach, lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following editorial improvements on high-level syntax:

- Remove a redundant check for signalling the syntax element sh_sign_data_hiding_used_flag.

- Remove a redundant check for deriving the variable signHidden.

- Rename the syntax element sps_internal_bit_depth_minus_input_bit_depth as sps_min_qp_prime_ts_idc.

# Proposals

## Proposal 1: Removal of redundant check for signalling sh_sign_data_hiding_used_flag

In VVC Draft 9 [1], the syntax element sh_sign_data_hiding_used_flag is signalled only when the value of the syntax element sps_sign_data_hiding_enabled_flag is equal to 1 and the value of the syntax element  sh_dep_quant_used_flag is equal to 0. However, under the syntax constraints of VVC Draft 9, the value of sh_dep_quant_used_flag shall be equal to 0 when sps_sign_data_hiding_enabled_flag is equal to 1. In this contribution, it is proposed that the condition for siganlling sh_sign_data_hiding_used_flag is simplified by removing the redundant check on whether sh_dep_quant_used_flag is equal to 0. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

#### 7.3.2.7 Picture header structure syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| sh_picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && ! sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |
| …. |  |
| } |  |

## Proposal 2: Removal of redundant check for deriving signHidden

In VVC Draft 9, the value of the variable signHidden is set equal to 0 when sh_dep_quant_used_flag is equal to 1 or sh_sign_data_hiding_used_flag is equal to 0. However, under the syntax constraints of VVC Draft 9, the value of sh_sign_data_hiding_enabled_flag shall be equal to 0 when sh_dep_quant_used_flag is equal to 1. Therefore, the check “!sh_sign_data_hiding_used_flag” is redundant in the residual_coding() syntax table, and the derivation of signHidden syntax element can be rewritten in a more compact way. In the proposed modification, an editorial improvent is suggested, in which the derivation process of the value of signHidden is simplified by removing the redundant check on whether sh_dep_quant_used_flag is equal to 1. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

#### 7.3.10.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sh_dep_quant_used_flag ¦ ¦ !sh_sign_data_hiding_used_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| signHidden = sh_sign_data_hiding_used_flag && ( lastSigScanPosSb − firstSigScanPosSb > 3 ) ? 1 : 0 |  |

## Proposal 3: Renaming sps_internal_bit_depth_minus_input_bit_depth

In VVC Draft 9, the syntax element sps_internal_bit_depth_minus_input_bit_depth in the SPS is signalled for deriving the the minimum allowed quantization parameter for transform skip mode. The name of the syntax element indicates that the value of the syntax element is equal to the value of the internal bit depth minus the input bit depth. As follows from the adoption, the sps_internal_bit_depth_minus_input_bit_depth syntax element was designed for lossless coding. However, the current syntax name is misleading, since the coded value of sps_internal_bit_depth_minus_input_bit_depth may have nothing to do with the internal and input bit depths. For example, when internal bit depth is less than the input bit depth, the value of sps_internal_bit_depth_minus_input_bit_depth, if computed directly, shall become negative, while, according to the specification, the value of sps_internal_bit_depth_minus_input_bit_depth is only allowed to be greater than or equal to 0. Thus, a video encoder may choose any value of sps_internal_bit_depth_minus_input_bit_depth that is unrelated to the internal and input bit depths and within the specified valid value range. This contribution proposes to rename sps_internal_bit_depth_minus_input_bit_depth as sps_min_qp_prime_ts_idc. The proposed modifications to VVC Draft 9 with modified parts highlighted in turquoise are provided as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| sps_internal_bit_depth_minus_input_bit_depth | ue(v) |
| sps_min_qp_prime_ts_idc | ue(v) |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_internal_bit_depth_minus_input_bit_depth sps_min_qp_prime_ts_idc specifies the minimum allowed quantization parameter for transform skip mode as follows:

QpPrimeTsMin = 4 + 6 * sps_internal_bit_depth_minus_input_bit_depth sps_min_qp_prime_ts_idc	(67)

The value of sps_internal_bit_depth_minus_input_bit_depth sps_min_qp_prime_ts_idc shall be in the range of 0 to 8, inclusive.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001-vA, 18th Meeting: by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).