| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0073-v1 |
| --- | --- |

| Title: | AHG9: Bug-fix of the constraint flag of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes two aspects.

The first aspect of this contribution proposes a bug-fix to the  no_aps_constraint_flag. In the current design, ALF and CCALF process can be enabled when the value of no_aps_constraint_flag is equal to 1. However, it is asserted that the chroma ALF and CCALF process cannot be executed without signaling of filter sets in APS. The aspect 1 of this contribution proposes to disable chroma ALF and CCALF when the value of no_aps_constraint_flag is equal to 1. Three options are proposed. In the first option, a semantic constraint is applied to the no_aps_constraint_flag to disable CCALF and chroma ALF. In the second option, a new SPS level flag is introduced to control chroma ALF process and the new SPS flag is disabled if no_aps_constraint_flag == 1. In the third option, both ALF and CCALF processes are disabled in SPS level.

The second aspect is related to a bug-fix of the no_tsrc_constraint_flag. In VVC draft 9, TS residual coding can only be disabled if both dependent quantization (DQ) and sign data hiding (SDH) are disabled. However, in current design, DQ or SDH can be enabled independent of the value of no_tsrc_constraint_flag. When no_tsrc_constraint_flag == 1 and DQ or SDH is enabled,  there is no way to actually disable TSRC. Aspect 2 of this contribution proposes to apply semantic constraint to disable DQ and SDH when no_tsrc_constraint_flag is equal to 1. Three options are proposed. In the first option, the DQ and SDH are disabled in SPS level; whereas in the second option, DQ and SDH are disabled in slice level. In the third option, transform-skip mode is disabled, which is equivalent to removing no_tsrc_constraint_flag.

# Problem Statement

In VVC, constraint flags are used to disable different coding tools to define profile, tier, and level. It is reported that the current VVC design has following issues related to constraint flags no_aps_constraint_flag and no_tsrc_constraint_flag.

# Issues related to no_aps_constraint_flag

In VVC9, the no_aps_constraint_flag specifies whether APS NAL unit presents in the bit-stream or not. Following is the semantic of the no_aps_constraint_flag.

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and the sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall both be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint.

From the above semantic, it is shown that if no_aps_constraint_flag is equal to 1, the coding tools LMCS and scaling list are disabled. However, ALF and CCALF can still being enabled even if no APS is signaled ( i.e. no_aps_constraint_flag is equal to 1).

In current VVC design, the ALF process of luma component can be enabled by using fixed set of filter sets (i.e. without sending the APS). However, there is no such fixed sets of filters for chroma ALF and CCALF process. Therefore, it is asserted that the chroma ALF and CCALF process cannot be executed without signaling of filter sets through APS.

In addition to disabling LMCS and scaling list, it is proposed that following constraint need to be applied when no_aps_constraint_flag == 1.

- During ALF process, in order to use fixed sets of filter for luma component, the value of number of ALF APS of luma (sh_num_alf_aps_ids_luma) need to set to be equal to 0.

- Since chroma ALF and CCALF do not have fixed set of filters, it is requiring to disable chroma ALF and CCALF process when no_aps_constraint_flag == 1.

Three  options are proposed.

## Option 1

In option 1, a semantic constraint is applied to the no_aps_constraint_flag to disable CCALF and chroma ALF. The semantic of the no_aps_constraint_flag is proposed as follows:

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and the sps_ccalf_enabled_flag, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall both all be equal to 0. If no_aps_constraint_flag is equal to 1, the value of  sh_num_alf_aps_ids_luma, sh_alf_cb_flag and sh_alf_cr_flag of all slices in OlsInScope shall be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint.

## Option 2

In option 2, a new SPS level flag is introduced to control chroma ALF process and the new SPS flag is disabled if no_aps_constraint_flag == 1. The semantic of the proposed sps_chroma_alf_enabled_flag is proposed as follows:

sps_chroma_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied in decoding of chroma component of the pictures in the CLVS. sps_chroma_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied in decoding of the chroma component of the pictures in the CLVS. When not present, the value of sps_chroma_alf_enabled_flag is inferred to be equal to 0.

Table 1: SPS syntax table of aspect 1: option 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ){ |  |
| sps_chroma_alf_enabled_flag | u(1) |
| sps_ccalf_enabled_flag | u(1) |
| } |  |
| } |  |

Table 1 shows the SPS syntax table of the option 2 of the aspect 1, where the proposed sps_chroma_alf_enabled_flag is signaled if sps_alf_enabled_flag is 1.

Upon introducing the new SPS level flag sps_chroma_alf_enabled_flag, the semantic of the no_aps_constraint_flag is defined as follows:

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and the sps_chroma_alf_enabled_flag, sps_ccalf_enabled_flag, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall both all be equal to 0. If no_aps_constraint_flag is equal to 1, the value of  sh_num_alf_aps_ids_luma, of all slices in OlsInScope shall be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint.

## Option 3

In option 3 , a semantic constraint is applied to the no_aps_constraint_flag to disable both ALF and CCALF. The semantic of the no_aps_constraint_flag is proposed as follows:

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and the sps_alf_enabled_flag, sps_ccalf_enabled_flag, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall both all be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint.

# Issues related to no_tsrc_constraint_flag

The no_tsrc_constraint_flag specifies whether TS residual coding is disabled or not. Below is the semantic of the no_tsrc_constraint_flag.

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

In VVC draft 9, TS residual coding can only be disabled through explicit signaling when both dependent quantization (DQ) and sign data hiding (SDH) are disabled. However, in current design, DQ or SDH can be enabled even if no_tsrc_constraint_flag == 1, which can result to disallow disabling of TSRC.

In this contribution, it is proposed to disable both DQ and SDH when no_tsrc_constraint_flag is equal to 1.

## Option 1

In option 1, a semantic constraint is applied to no_tsrc_constraint_flag to disable DQ and SDH in SPS level. Therefore, the semantic of the no_tsrc_constraint_flag is proposed as follows:

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are both equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

## Option 2

In option 2, a semantic constraint is applied to no_tsrc_constraint_flag to disable DQ and SDH in all of the slices of the CLVS.  Therefore, the semantic of the no_tsrc_constraint_flag is proposed as follows:

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sh_dep_quant_used_flag and sh_sign_data_hiding_used_flag are both equal to 0 for all slices. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

## Option 3

It is noted that disabling DQ or SDH may impact coding efficiency. It is further noted that if transform skip is disabled at the SPS level, the problem of having TSRC no longer exists.  In current VVC specification, there is another constraint flag “no_transform_skip_constraint_flag” to disable transform skip mode. no_transform_skip_constraint_flag equal to 1 specifies that transform skip mode is disabled. When no_transform_skip_constraint_flag == 1, since there is no transform skip mode, TSRC is implicitly disabled. Therefore, if disabling DQ and SDH is not desirable,  the functionality of no_tsrc_constraint_flag is overlapped with that of no_transform_skip_constraint_flag,  and no_tsrc_constraint_flag can be removed. Table 2 shows the general constraint syntax of the option 3.

Table 2: General constraint syntax of the option 3

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| } |  |

# Conclusion

This contribution proposes to fix of bugs related to no_aps_constraint_flag and no_tsrc_constraint_flag. In the aspect 1, it is proposed to disable chroma ALF and CCALF process when the value of no_aps_constraint_flag is equal to be 1. In aspect 2, this contribution proposes to disable DQ and SDH process when the value of no_tsrc_constraint_flag is equal to 1, or simply remove no_tsrc_constraint_flag.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).