| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0081 |
| --- | --- |

| Title: | AHG9: On non-referenced picture and POC derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Junghak Nam Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com hm.jang@lge.com junghak.nam@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current VVC spec text, it is asserted that the POC decoding process has a problem related to the non-referenced picture. In the decoding process, PicOrderCntMsb is inferred based on the POC value of the picture that is designated as prevTid0Pic. The picture that is assigned to be prevTid0Pic for the current picture should be the same picture during encoding and decoding in order to derive the POC of the current picture consistently. However in the current draft, the assignment of prevTid0Pic does not take into account the value of ph_non_ref_pic_flag. Currently, the prevTid0Pic can be a picture that may be removed by a system which is possible when the value of ph_non_ref_pic_flag of the picture that is assigned to be prevTid0Pic by encoder is equal to 1.

To solve the asserted problem, in this contribution, the following bitstream conformance constraint is proposed:

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

–	The values of nuh_layer_id of picA and picB are the same.

–	picA and picB are not RADL picture or RASL picture.

–	The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

–	picB is the first picture with TemporalId equal to 0 that follows picA in output order.

# Problem statement

In the current VVC spec text, the derivation process for POC does not take into account whether reference picture that it is assigned as prevTid0Pic may be removed by a system (e.g., in a media aware network that is capable of dropping picture in in router in case of network congestion). When the prevTid0Pic is dropped by the system, decoder will unknowingly use different prevTid0Pic from the one that was used by encoder for POC derivation of the current picture. It may consequently cause the decoder to derive wrong POC value.

The figure below depicts an example of the problem described above. During POC derivation for the current picture, the picture with POC equal to 16 is designated as prevTid0Pic. If the picture with POC 16 has ph_non_ref_pic_flag equal to 1, it may be removed from the bitstream by system and when it happens it will cause wrong POC derivation.

Figure 1. Example for a problem statement when the prevTid0Pic is dropped by system entity.

# Proposal and spec text changes

To solve the problem above, we proposed the following bitstream conformance constraint such that any two consecutive picture with TemporalId equal to 0 that may be used as prevTid0Pic shall be POC difference that is not greater than half of MaxPicOrderCntLsb.

The spec text changes for implementing the above proposed changes is as follows:

7.4.2.2	NAL unit header semantics

…

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The value of nuh_temporal_id_plus1 shall not be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(36)

When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_12, inclusive, TemporalId shall be equal to 0.

When nal_unit_type is equal to STSA_NUT and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an AU. The value of TemporalId of a coded picture, a PU, or an AU is the value of the TemporalId of the VCL NAL units of the coded picture, PU, or AU. The value of TemporalId of a sublayer representation is the greatest value of TemporalId of all VCL NAL units in the sublayer representation.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to DCI_NUT, VPS_NUT, or SPS_NUT, TemporalId shall be equal to 0 and the TemporalId of the AU containing the NAL unit shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to PH_NUT, TemporalId shall be equal to the TemporalId of the PU containing the NAL unit.

–	Otherwise, if nal_unit_type is equal to EOS_NUT or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT, FD_NUT, PREFIX_SEI_NUT, or SUFFIX_SEI_NUT, TemporalId shall be equal to the TemporalId of the AU containing the NAL unit.

–	Otherwise, when nal_unit_type is equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT, TemporalId shall be greater than or equal to the TemporalId of the PU containing the NAL unit.

NOTE 7 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all AUs to which the non-VCL NAL unit applies. When nal_unit_type is equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing AU, as all PPSs and APSs may be included in the beginning of the bitstream (e.g., when they are transported out-of-band, and the receiver places them at the beginning of the bitstream), wherein the first coded picture has TemporalId equal to 0.

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

–	The values of nuh_layer_id of picA and picB are the same.

–	picA and picB are not RADL picture or RASL picture.

–	The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

–	picB is the first picture with TemporalId equal to 0 that follows picA in output order.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).