| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0085-v2 |
| --- | --- |

| Title: | AHG9: On semantics related to reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

It is reported that several syntax elements and variables related to the reference picture lists may be set to out of range values or undefined values. There are three problems. The first problem is that when sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag is equal to 0 and rpl_idx[ 0 ] is equal to or greater than sps_num_ref_pic_lists[ 1 ], the value of rpl_idx[ 1 ] may be out of range. The second problem is that since the variable RplsIdx[ 1 ] before the derivation process is used, the value of RplsIdx[ 1 ] is undefined. The third problem is that when sps_num_ref_pic_lists[ 0 ] is equal to 1, the syntax element rpl_idx[ 0 ] may be undefined. In this contribution, a set of methods to solve the above problems is proposed. All proposals only improve the semantics without changing the syntax.

Problem Statement

There are several problems with the current semantics of the syntax elements related to ref_pic_lists( ), which defines reference picture lists.

Problem 1

The first problem is that the reference picture list 1 may be undefined under the following conditions

- sps_rpl1_same_as_rpl0_flag is equal to 0

- sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ].

- pps_rpl1_idx_present_flag is equal to 0.

- rpl_sps_flag[ 0 ] is equal to 1.

- rpl_idx[ 0 ] is equal to or greater than sps_num_ref_pic_lists[ 1 ].

On the above condition, since pps_rpl1_idx_present_flagis equal to 0, rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] are inferred from rpl_sps_flag[ 0 ] and rpl_idx[ 0 ] respectively. Since rpl_idx[ 0 ] is equal to or greater than sps_num_ref_pic_lists[ 1 ], rpl_idx[ 1 ] is out of range of 0 to sps_num_ref_pic_lists[ 1 ] − 1. The variable RplsIdx[ 1 ] is set to the value of rpl_idx[ 1 ] because rpl_sps_flag[ 1 ] is equal to 1. As a result, RplsIdx[ 1 ] may indicate an undefined reference picture list.

For example, assume the following case:

- sps_rpl1_same_as_rpl0_flag = 0

- sps_num_ref_pic_lists[ 0 ] = 5

- sps_num_ref_pic_lists[ 1 ] = 3

- pps_rpl1_idx_present_flag = 0

- rpl_sps_flag[ 0 ] = 1

- rpl_idx[ 0 ] = 4

It is decoded that rpl_sps_flag[ 1 ] is equal to 1, rpl_idx[ 1 ] is equal to 4 and RplsIdx[ 1 ] is equal to 4. Since sps_num_ref_pic_lists[ 1 ] is equal to 3, there is no such a reference picture list.

The following is the related syntax and semantics text. The sky-blue highlights should be noted.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < sps_rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| sps_num_ref_pic_lists[ i ] | ue(v) |
| for( j = 0; j < sps_num_ref_pic_lists[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| … |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_rpl1_idx_present_flag | u(1) |
| … |  |

| ref_pic_lists( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 2; i++ ) { |  |
| if( sps_num_ref_pic_lists[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && pps_rpl1_idx_present_flag ) ) ) |  |
| rpl_sps_flag[ i ] | u(1) |
| if( rpl_sps_flag[ i ] ) { |  |
| if( sps_num_ref_pic_lists[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && pps_rpl1_idx_present_flag ) ) ) |  |
| rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, sps_num_ref_pic_lists[ i ] ) |  |
| … |  |
| } |  |
| } |  |
| } |  |

- 7.4.3.3  Sequence parameter set RBSP semantics

sps_rpl1_same_as_rpl0_flag equal to 1 specifies that the syntax element sps_num_ref_pic_lists[ 1 ] and the syntax structure ref_pic_list_struct( 1, rplsIdx ) are not present and the following applies:

–	The value of sps_num_ref_pic_lists[ 1 ] is inferred to be equal to the value of sps_num_ref_pic_lists[ 0 ].

–	The value of each of syntax elements in ref_pic_list_struct( 1, rplsIdx ) is inferred to be equal to the value of corresponding syntax element in ref_pic_list_struct( 0, rplsIdx ) for rplsIdx ranging from 0 to sps_num_ref_pic_lists[ 0 ] − 1.

sps_num_ref_pic_lists[ i ] specifies the number of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS. The value of sps_num_ref_pic_lists[ i ] shall be in the range of 0 to 64, inclusive.

NOTE 6 – For each value of listIdx (equal to 0 or 1), a decoder should allocate memory for a total number of sps_num_ref_pic_lists[ i ] + 1 ref_pic_list_struct( listIdx, rplsIdx ) syntax structures since there may be one ref_pic_list_struct( listIdx, rplsIdx ) syntax structure directly signalled in the slice headers of a current picture.

- 7.4.3.4  Picture parameter set RBSP semantics

pps_rpl1_idx_present_flag equal to 0 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] are not present in the PH syntax structures or the slice headers for pictures referring to the PPS. pps_rpl1_idx_present_flag equal to 1 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] may be present in the PH syntax structures or the slice headers for pictures referring to the PPS.

- 7.4.9  Reference picture lists semantics

The ref_pic_lists( ) syntax structure may be present in the PH syntax structure or the slice header.

rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i in ref_pic_lists( ) is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the picture is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in ref_pic_lists( ).

When rpl_sps_flag[ i ] is not present, it is inferred as follows:

- If sps_num_ref_pic_lists[ i ] is equal to 0, the value of rpl_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise (sps_num_ref_pic_lists[ i ] is greater than 0), when pps_rpl1_idx_present_flag is equal to 0 and i is equal to 1, the value of rpl_sps_flag[ 1 ] is inferred to be equal to rpl_sps_flag[ 0 ].

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits. The value of rpl_idx[ i ] shall be in the range of 0 to sps_num_ref_pic_lists[ i ] − 1, inclusive. When not present, if rpl_sps_flag[ i ] is equal to 1 and pps_rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ 1 ] is inferred to be equal to 0.

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : sps_num_ref_pic_lists[ i ]	(150)

Problem 2

The second problem is that the variable RplsIdx[ i ] may be undefined because if pps_rpl_info_in_ph_flag is equal to 0, the variable RplsIdx[ 1 ] is used before ref_pic_list( ) is called. The condition for decoding ph_mvd_l1_zero_flag in the picture header is that pps_rpl_info_in_ph_flag is equal to 0 or num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] is greater than 0. When pps_rpl_info_in_ph_flag is equal to 0, ref_pic_lists( ) is not called, so the value of the variable RplsIdx[ 1 ] becomes undefined. The value of num_ref_entries[ 1 ] [ RplsIdx[ 1 ] ] is also undefined.

The following is the related syntax and semantics text. The sky-blue highlights should be noted.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_rpl_info_in_ph_flag ) |  |
| ref_pic_lists( ) |  |
| … |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … |  |
| if( !pps_rpl_info_in_ph_flag ¦ ¦ num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |
| } |  |
| } |  |

Problem 3

The third problem is that when sps_num_ref_pic_lists[ 0 ] is equal to 1, the value of rpl_idx[ 0 ] is not signaled, but the inferred value is not defined.

The following is the related syntax and semantics text. The sky-blue highlights should be noted.

| ref_pic_lists( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 2; i++ ) { |  |
| if( sps_num_ref_pic_lists[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && pps_rpl1_idx_present_flag ) ) ) |  |
| rpl_sps_flag[ i ] | u(1) |
| if( rpl_sps_flag[ i ] ) { |  |
| if( sps_num_ref_pic_lists[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && pps_rpl1_idx_present_flag ) ) ) |  |
| rpl_idx[ i ] | u(v) |
| } else |  |
| … |  |
| } |  |

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits. The value of rpl_idx[ i ] shall be in the range of 0 to sps_num_ref_pic_lists[ i ] − 1, inclusive. When not present, if rpl_sps_flag[ i ] is equal to 1 and pps_rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ 1 ] is inferred to be equal to 0.

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : sps_num_ref_pic_lists[ i ]	(150)

Proposal

Solutions for problem 1

There may be multiple solutions to problem 1. In this contribution, two straightforward options are suggested.

Option 1

The semantics of pps_rpl1_idx_present_flag should be restricted as the following (the yellow highlight is the changed part).

pps_rpl1_idx_present_flag equal to 0 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] are not present in the PH syntax structures or the slice headers for pictures referring to the PPS. pps_rpl1_idx_present_flag equal to 1 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] may be present in the PH syntax structures or the slice headers for pictures referring to the PPS. When sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag shall not be equal to 0.

Option 2

The semantics of rpl_idx[ i ] should be improved as the following (the yellow highlight is the changed part).

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. When pps_rpl1_idx_present_flag is equal to 0, the syntax element rpl_idx[ 0 ] shall be in the range of 0 to min(sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ]) − 1 and the syntax element rpl_idx[ 0 ] is represented by Ceil( Log2(min( sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ] ) ) ) bits, otherwise (pps_rpl1_idx_present_flag is equal to 1) the syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits. The value of rpl_idx[ i ] shall be in the range of 0 to sps_num_ref_pic_lists[ i ] − 1, inclusive. When not present, if rpl_sps_flag[ i ] is equal to 1 and pps_rpl1_idx_present_flag is equal to 0, the value of rpl_idx[ 1 ] is inferred to be equal to rpl_idx[ 0 ], otherwise the value of rpl_idx[ 1 ] is inferred to be equal to 0.

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : sps_num_ref_pic_lists[ i ]	(150)

Solution for problem 2

The semantics of rpl_sps_flag[ i ] and num_ref_entries[ listIdx ][ rplsIdx ] should be improved as the following (the yellow highlight is the changed part).

rpl_sps_flag[ i ] equal to 1 specifies that reference picture list i in ref_pic_lists( ) is derived based on one of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i in the SPS. rpl_sps_flag[ i ] equal to 0 specifies that reference picture list i of the picture is derived based on the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is directly included in ref_pic_lists( ).

When rpl_sps_flag[ i ] is not present, it is inferred as follows:

- If sps_num_ref_pic_lists[ i ] is equal to 0, the value of rpl_sps_flag[ i ] is inferred to be equal to 0.

- Otherwise (sps_num_ref_pic_lists[ i ] is greater than 0), when pps_rpl1_idx_present_flag is equal to 0 and i is equal to 1, the value of rpl_sps_flag[ 1 ] is inferred to be equal to rpl_sps_flag[ 0 ] ,otherwise rpl_sps_flag[ i ] is inferred to be equal to 0.

num_ref_entries[ listIdx ][ rplsIdx ] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of num_ref_entries[ listIdx ][ rplsIdx ] shall be in the range of 0 to MaxDpbSize + 13, inclusive, where MaxDpbSize is as specified in clause A.4.2. num_ref_entries[ listIdx ][ sps_num_ref_pic_lists[ listIdx ] ] is inferred to be equal to 0.

Solution for problem 3

The semantics of rpl_idx[ i ] should be improved as the following (the yellow highlight is the changed part).

rpl_idx[ i ] specifies the index, into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures with listIdx equal to i included in the SPS, of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure with listIdx equal to i that is used for derivation of reference picture list i of the current picture. The syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits. The value of rpl_idx[ i ] shall be in the range of 0 to sps_num_ref_pic_lists[ i ] − 1, inclusive. When not present, if sps_num_ref_pic_lists[ i ] is equal to or less than 1, rpl_idx[ i ] is inferred to be equal to 0, otherwise (sps_num_ref_pic_lists[ i ] is greater than 1) if i is equal to 1, pps_rpl1_idx_present_flag is equal to 0 and rpl_sps_flag[ 0 ] is equal to 1, the value of rpl_sps_flag[ 1 ] is inferred to be equal to rpl_sps_flag[ 0 ].

The variable RplsIdx[ i ] is derived as follows:

RplsIdx[ i ] = rpl_sps_flag[ i ] ? rpl_idx[ i ] : sps_num_ref_pic_lists[ i ]	(150)

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).