| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0090 |
| --- | --- |

| Title: | AHG8/AHG9: On single_layer_constraint_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Takahiro Nishi Tadamasa Toma Virginie Drugeon 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com nishi.takahiro@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

According to the general constraint information semantics of the current VVC draft, if the value of single_layer_constraint_flag is equal to 1, sps_video_parameter_set_id shall be equal to 0. It means that a single layer bitstream using this constraint flag is prohibited from referring to any VPS. However, VVC also supports single layer bitstreams that refer to a VPS with a syntax indicating only one layer. This contribution proposes to modify the description of the single_layer_constraint_flag semantics to support both single layer structures with and without VPS referencing.

Problem Statement

In VVC Draft 9, single_layer_constraint_flag signalled in the general constraint information syntax structure has the semantics shown below. According to this semantics, if the value of single_layer_constraint_flag is equal to 1, sps_video_parameter_set_id shall be equal to 0. It means that a single layer bitstream using this constraint flag is prohibited from referring to any VPS. However, VVC also supports single layer bitstreams that refer to a VPS with a syntax indicating only one layer.

7.4.4.2	General constraint information semantics

single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1.

Proposed changes

Two options are proposed. The suggested specification changes are described on top of VVC draft of JVET-R2001[1]. The newly added parts are highlighted in yellow and the text to be removed is marked as red.

- Option 1:

The proposed modification specifies two kinds of single layer conditions. The first one is a single layer structure that does not refer to any VPS (i.e. sps_video_parameter_set_id is equal to 0), and the second one is a single layer bitstream that refers to a VPS with a syntax indicating only one layer.

7.4.4.2	General constraint information semantics

single_layer_constraint_flag equal to 1 specifies that one of the following conditions shall be fulfilled:

- sps_video_parameter_set_id is shall be equal to 0.

- The value of vps_max_layers_minus1 in the referred VPS is equal to 0 and all VCL NAL units in the CVS have the same value of nuh_layer_id.

single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1.

- Option 2:

The proposed modification describes that all NAL units belong to one layer, and does not mention any constraint of VPS and SPS syntaxes.

7.4.4.2	General constraint information semantics

single_layer_constraint_flag equal to 1 specifies that all VCL NAL units in the CVS shall have the same value of nuh_layer_id. sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1.

Conclusion

This contribution proposes to modify the description of the single_layer_constraint_flag semantics to support both single layer structures with and without VPS referencing. Option1 proposes to specify two kinds of single layer conditions. Option2 proposes to describe that all NAL units belong to one layer, and does not mention any constraint of VPS and SPS syntaxes. The authors of this document believe that a more specific description will help in the development of encoders, so Option1 is preferred for adoption into VVC.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, JVET 18th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).