| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0107 |
| --- | --- |

| Title: | AHG9/AHG12: Recommended multi-layer composite picture SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

Two SEI messages are proposed which describe recommended composite pictures of decoded pictures from multiple layers. The layer composite SEI message is sent for each layer and contains parameters for the current layer’s decoded picture. The recommended composite layers info SEI message applies to multiple layers, and includes parameters for each output layer set (OLS).

The proposed SEI messages provide similar functionality as described in contribution JVET-R0307.

# Proposal

Two SEI messages are proposed which describe recommended composite pictures of decoded pictures from multiple layers. The layer composite SEI message is sent for each layer and contains parameters for the current layer’s decoded picture. The recommended composite layers info SEI message applies to multiple layers, and includes parameters for each output layer set (OLS). A composition process is defined which uses the syntax elements from both SEI messages.

The layer composite position info SEI message applies to the decoded picture of the current layer (with the same value of nuh_layer_id as the SEI message). It includes syntax elements to indicate the following:

- top left vertical and horizontal position of the decoded layer picture within a composite picture, in units

- height and width of the decoded layer picture within a composite picture, in units

The recommended composite layers info SEI message contains parameters for a recommended composite pictures for each OLS, which apply to multiple layers. Each recommended composite layers info SEI message contains syntax elements to indicate the following:

- flag to indicate if scaling of decoded layer pictures in the composite picture is enabled

- flag to indicate if decoded layer pictures may overlap in the composite picture

- number of OLSs

- For each OLS

  - size of the units in luma samples of the position and size syntax elements in the layer composite position info SEI message

  - size of the composite picture

  - offset to be applied to all decoded layer pictures in the composite picture

The size of each layer’s decoded picture is not required to equal its recommended display size in the composite picture, unless the flag indicates that scaling is not enabled. If not equal, scaling is applied to the layer’s decoded picture when forming the composite picture. The scaling method to be used is not specified.

The decoded pictures may overlap one another in the composite picture, with the layer with the higher value of nuh_layer_id taking precedence, unless the flag indicates that overlap is not enabled. There is no requirement that the entire composite picture be covered by decoded layer pictures, Uncovered areas in the composite picture are undefined and left to the application to define.

The offset parameters are intended to be used when some layers are not included in the OLS, without needing to change the per layer parameters in each layer composite position info SEI message.

# Discussion

The design motivation in this contribution is to provide a general and flexible solution to multi-layer picture compositing which supports bitstream extraction and merging (BEAM) functionalities, while minimizing bitrate overhead.

The design assumes that the relative positions and sizes of the individual layer pictures in a composite are consistent, but different sets of layers can be present in output layer sets (OLSs), so sub-regions covered by the “full size” composite picture are of interest. It was a design goal to minimize rewriting complexity when bitstream extraction is performed. It is expected that the layer composite position info SEI message, which is sent per layer, is not required to be rewritten in bitstream extraction.

Figures 1, 2, and 3 illustrate example usages of multiple OLSs with different offset values, resulting in a different composite picture from the same bitstream for each OLS.

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 4 | 5 | 6 | 7 |
| 8 |  | 10 |  |
| 9 |  |  |  |

Figure 1. Example layout for OLS 0

| 2 | 3 |
| --- | --- |
| 6 | 7 |
| 10 |  |

Figure 2. Example layout for OLS 1

| 5 | 6 |
| --- | --- |

Figure 3. Example layout for OLS 2

In this example, 11 layers are present in the bitstream, with values of nuh_layer_id from 0 .. 10. The recommended display sizes of the pictures of layers 0 to 7 are the same, 480x240. The recommend display size of pictures of layers 8 and 9 are 960x240. The recommended display size of the picture of layer 10 is 960x480. In this example, the unit size can be 480x240 luma samples, as that is the smallest unit of granularity of sizes and positions needed.

The sizes and positions in units are signaled for each layer in its own layer composite position info SEI message. The signaled size corresponds to the recommended display size and not the coded picture size. Scaling is recommended to be applied when the coded picture size differs from the display size, but a scaling method is not defined. Explicit signaling of the recommended display size enables use of reference picture resizing, where the coded picture size may vary but the recommended display size remains constant. Scaling of the pictures from some layers and not from others may also be desired for other applications.

Table 1. Example syntax element values in layer composite position info SEI message (common to all OLSs)

| nuh_layer_id | lcpi_top_left_ pos _in_units_ver | lcpi_top_left_pos _in_units_hor | lcpi_height_in_units | lcpi_width_in_units |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 2 | 0 | 2 | 1 | 1 |
| 3 | 0 | 3 | 1 | 1 |
| 4 | 1 | 0 | 1 | 1 |
| 5 | 1 | 1 | 1 | 1 |
| 6 | 1 | 2 | 1 | 1 |
| 7 | 1 | 3 | 1 | 1 |
| 8 | 2 | 0 | 1 | 2 |
| 9 | 3 | 0 | 1 | 2 |
| 10 | 2 | 2 | 2 | 2 |

Table 2. Example syntax elements in multiple recommended composite layers info SEI messages

| OLS idx | rcli_composite_size_ver | rcli_composite_size_hor | rcli_offset_ver | rcli_offset_hor |
| --- | --- | --- | --- | --- |
| 0 | 960 | 1920 | 0 | 0 |
| 1 | 960 | 960 | 0 | -960 |
| 2 | 480 | 960 | -240 | -480 |

Signaling of the unit size and composite picture size syntax elements in the recommended composite layers info SEI message is optional. If the syntax elements are not present, the application may derive inferred values or determine them by external means. For example, the composite picture size information may be contained within the systems layer.

# Syntax and semantics

## Layer composite position info SEI message

| layer_composite_position_info( ) { | Descriptor |
| --- | --- |
| lcpi_param_num_bits_minus1 | u(12) |
| lcpi_top_left_ pos_in_units_ver[ nuh_layer_id] | u(v) |
| lcpi_top_left_pos_in_units_hor[ nuh_layer_id] | u(v) |
| lcpi_height_in_units[ nuh_layer_id] | u(v) |
| lcpi_width_in_units[ nuh_layer_id] | u(v) |
| } |  |

The layer composite position info SEI message describes the recommended position and size of the decoded picture of the current layer within a recommended composite picture comprised of decoded pictures from multiple layers.

lcpi_param_num_bits_minus1 + 1 specifies the number of bits used to represent the lcpi_top_left_pos_ver[ nuh_layer_id ], cpi_top_left_pos_ver[ nuh_layer_id ], lcpi_width[ nuh_layer_id ], and lcpi_height[ nuh_layer_id ]  syntax elements.

lcpi_top_left_pos_ver[ nuh_layer_id] and lcpi_top_left_pos_ver[ nuh_layer_id] indicate the recommended composite display vertical and horizontal positions, respectively, for the decoded picture of the current layer. The number of bits to represent the syntax elements is lcpi_param_num_bits_minus1 + 1.

lcpi_width[ nuh_layer_id] and lcpi_height[ nuh_layer_id]  indicate the recommended composite display width and height, respectively, for the decoded picture of the current layer. The number of bits to represent the syntax elements is lcpi_param_num_bits_minus1 + 1.

## Recommended composite layers info SEI message

| recommended_composite_layers_info( ) { | Descriptor |
| --- | --- |
| rcli_cancel_flag | u(1) |
| if (!rcli_cancel_flag) { |  |
| rcli_persistence_flag | u(1) |
| rcli_layer_scaling_enabled_flag | u(1) |
| rcli_layer_overlap_enabled_flag | u(1) |
| rcli_unit_size_present_flag | u(1) |
| rcli_composite_size_present_flag | u(1) |
| rcli_offset_present_flag | u(1) |
| num_olss_minus1 | u(8) |
| for ( i = 0; i <= num_olss_minus1; i++ ) { |  |
| if (rcli_unit_size_present_flag ) { |  |
| rcli_unit_size_ver[ i ] | u(16) |
| rcli_unit_size_hor[ i ] | u(16) |
| } |  |
| if (rcli_composite_size_present_flag ) { |  |
| rcli_composite_size_ver[ i ] | u(16) |
| rcli_composite_size_hor[ i ] | u(16) |
| } |  |
| if ( rcli_offset_present_flag ) { |  |
| rcli_offset_ver[ i ] | s(16) |
| rcli_offset_hor[ i ] | s(16) |
| } |  |
| } |  |
| } |  |
| } |  |

The recommended composite layers info SEI message, with the layer composite position information SEI message, describes a layout of decoded pictures from the layers of an OLS within a recommended composite picture, according to the composition process of sub-clause X.

rcli_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous recommended composite layers info SEI message in output order.

rcli_persistence_flag specifies the persistence of recommended composite layers info SEI message.

rcli_persistence_flag equal to 0 specifies that the recommended composite layers info SEI message applies to the current AU only.

rcli_persistence_flag equal to 1 specifies that the recommended composite layers info SEI message applies to the current access unit and persists all subsequent access units in output order until one or more of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in an AU associated with a recommended composite layers info SEI message is output that follows the current picture in output order

rcli_layer_scaling_enabled_flag equal to 0 indicates that layerPicSizeInCompositeHeight[ i ] and layerPicSizeInCompositeWidth[ i ] derived in subclause X are equal to the width and height, respectively,  of the coded picture with nuh_layer_id equal to i. rcli_layer_scaling_enabled_flag equal to 1 indicates that layerPicSizeInCompositeHeight[ i ] and layerPicSizeInCompositeWidth[ i ] may differ from the width and height, respectively,  of the coded picture with nuh_layer_id equal to i.

rcli_layer_overlap_enabled_flag equal to 0 indicates that all values of Count[ y][ x ] shall be les than or equal to 1, as derived in subclause X . rcli_layer_overlap_enabled_flag equal to 1 does not impose a restriction.

rcli_unit_size_present_flag equal to 1 specifies that the rcli_unit_size_ver[ i ]  and rcli_ unit_size_hor[ i ]  syntax elements are present. rcli_unit_size_present_flag equal to 0 specifies that the rcli_ unit_size_ver[ i ] and rcli_ unit_size_hor[ i ] syntax elements are not  present.

rcli_composite_size_present_flag equal to 1 specifies that the rcli_composite_size_ver[ i ]  and rcli_composite_size_hor[ i ]  syntax elements are present. rcli_composite_size_present_flag equal to 0 specifies that the rcli_composite_size_ver[ i ] and rcli_composite_size_hor[ i ] syntax elements are not  present.

rcli_offset_present_flag equal to 1 specifies that the rcli_offset_ver[ i ]  and rcli_offset_hor[ i ]  syntax elements are present. rcli_offset_present_flag equal to 0 specifies that the rcli_offset_ver[ i ] and rcli_offset_hor[ i ] syntax elements are not  present.

num_olss_minus1 indicates the number of OLSs for which syntax elements are present in the SEI message.

rcli_unit_size_ver[ i ] and rcli_unit_size_hor[ i ] indicate vertical and horizontal unit size parameters respectively,  used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  may be determined by external means.

rcli_composite_size_ver[ i ] and rcli_composite_size_hor[ i ] indicate the vertical and horizontal size, respectively, of the recommended composite picture in luma samples used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  may be determined by external means.

rcli_offest_ver[ i ] and rcli_offset_hor[ i ] indicate vertical and horizontal offsets, respectively, of the positions of the decoded layer pictures used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  are inferred to be equal to 0.

## Recommended composition process

This subclause describes a composition process to derive sample values for a recommended composite picture, CompositePicture, for the i-th OLS.

CompositePicture[ 0]  is a 2-D sample array of size rcli_composite_size_hor x rcli_composite_size_ver, of the luma samples of CompositePicture. CompositePicture[ cIdx ]  for cIdx in 1 .. 2 are 2-D sample arrays of size rcli_composite_size_hor/ SubWidthC x rcli_composite_size_ver/ SubHeightC.

For each layer j included in the i-th OLS, if a picture is present in the AU with nuh_layer_id equal to j,   the recommended size of the representation of the decoded picture in the composite picture, scaledLayerPic[ j ], is layerPicSizeInCompositeWidth[j ] x layerPicSizeInCompositeHeight[ j ], in luma samples, as derived below.

layerPicSizeInCompositeHeight[ j ] = lcpi_height_in_units[j ] * rcli_unit_size_ver[ i ]

layerPicSizeInCompositeWidth[ j ] = lcpi_width_in_units[ j ] * rcli_unit_size_hor[ i ]

When rcli_layer_scaling_enabled_flag equal to 0, scaledLayerPic[ j ] is set to the decoded picture.

Otherwise, scaledLayerPic[ j ] is derived by scaling the decoded picture.

scaledLayerPic[ j ] is a picture of size layerPicSizeInCompositeWidth[ j ] x layerPicSizeInCompositeHeight[ j ], in the luma samples .

The sample values of CompositePicture are derived as follows:

for (y = 0; y <  rcli_composite_size_ver[ i];  y++)

for (x = 0; x < rcli_composite_size_hor[ i ]; x++)

Count[  y ][ x ] = 0

for (j = 0 ; j< 64; j++)

if (j is in the i-th OLS && a picture is present in the AU with nuh_layer_id = j)

comp_y = rcli_offset_ver[ i ] + lcpi_top_left_pos_in_units_ver[ j ] * rcli_unit_size_ver

comp_x = rcli_offset_hor[ i ] + lcpi_top_left_pos_in_units_hor[ j ] * rcli_unit_size_hor

for (y = 0; y <  layerPicSizeInCompositeHeight[ j ] * ; y++)

for (x = 0; x < layerPicSizeInCompositeWidth[ j ]; x++)

CompositePicture[ 0 ] [ comp_y + y ][ comp_x + x ] = scaledLayerPic[ 0 ][ j ][ y ][ x ]

Count[[ comp_y + y ][ comp_x + x ]]++

for (y = 0; y <  layerPicSizeInCompositeHeight[ j ]/SubWidth * ; y++)

for (x = 0; x < layerPicSizeInCompositeWidth[ j ]/SubHeight; x++)

CompositePicture[ 1 ][ comp_y/SubHeight + y ][ comp_x/SubWidth + x ] =

scaledLayerPic[ 1 ][ j ][ y ][ x ]

CompositePicture[ 2 ][ comp_y/SubHeight + y ][ comp_x/SubWidth + x ] =

scaledLayerPic[ 2 ][ j ][ y ][ x ]

The value of samples of CompositePicture[cIdx] not assigned above are undefined.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).