| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0117 |
| --- | --- |

| Title: | AHG9: On subpicture sub-bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following aspects for subpicture sub-bitstream extraction process

- Bugfix on PPS conformance rewriting process

- Rewrite subpicture ID mapping in SPS and PPS

- Rewrite virtual boundary in SPS

# Proposals

## Conformance window rewriting

VVC D9 specifies the rewriting process for subpicture sub-bitstream extraction process when there is no external means to provide replacement parameter sets, including general_level_idc, picture width and height, sps_num_subpics_minus1 and pps_num_subpics_minus1, sub-picture position, tile/slice layout and conformance window.

The conformance window rewriting is specified as follows:

- Rewrite the values of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset in all the referenced SPS NAL units and the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset in all the referenced PPS NAL units to be equal to subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset, and subpicConfWinBottomOffset, respectively.

However, VVC specifies the constraint that "When pps_pic_width_in_luma_samples is equal to sps_pic_width_max_in_luma_samples and pps_pic_height_in_luma_samples is equal to sps_pic_height_max_in_luma_samples, the value of pps_conformance_window_flag shall be equal to 0".

Since the picture resolution does not change when subpicture is present, the value of pps_conformance_window_flag shall be equal to 0 in the inBitstream. The values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

As a result, it is not possible to rewrite PPS conformance window since the relevant syntax elements are not present as pps_conformance_window_flag shall be equal to 0.

The rewriting of conformance window SEs can be updated as follows:

- Rewrite the values of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset in all the referenced SPS NAL units and the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset in all the referenced PPS NAL units to be equal to subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset, and subpicConfWinBottomOffset, respectively.

## Subpicture ID mapping rewriting

The subpicture sub-bitstream extraction is based on subpicture index instead of subpicture ID, the subpicture ID mapping to the 0-th subpicture in inBitstream may not be equal to the subpicture ID of the extracted subpicture. The subpicture IDs, when present in inBitstream, of those subpictures not being extracted shall be removed in all the referenced SPS and PPS NAL units in outBitstream as follows.

- Remove the syntax elements sps_subpic_id[ j ] in all the referenced SPS NAL units and for each j that is not equal to subpicIdx.

- Remove the syntax elements pps_subpic_id[ j ] in all the referenced PPS NAL units and for each j that is not equal to subpicIdx.

## SPS VB rewriting

The value of the sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples in all the referenced SPS NAL units may be rewritten after subpicture sub-bitstream extraction process, the corresponding virtaul boundaries, when present, in SPS may be rewritten in all the referenced SPS NAL units in outBitstream as follows.

- The variables numVerVBs, subpicVBx[ i ], numHorVBs, and subpicVBy[ i ] are derived as follows:

numVerVBs = 0; 
	for( i = 0; i < sps_num_ver_virtual_boundaries; i++) {
		if( sps_virtual_boundary_pos_x[ i ] > ( sps_subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY ÷ 8 ) &&
			sps_virtual_boundary_pos_x[ i ] < min( ( sps_subpic_ctu_top_left_x[ subpicIdx ] + 
			sps_subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY ÷ 8 , pps_pic_width_in_luma_samples ÷ 8 ) ) 
			subpicVBx[ numVerVBs++ ] = sps_virtual_boundary_pos_x[ i ] −
										sps_subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY ÷ 8 
	}

numHorVBs = 0; 
	for( i = 0; i < sps_num_hor_virtual_boundaries; i++) {
		if( sps_virtual_boundary_pos_y[ i ] > ( sps_subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY ÷ 8 ) &&
			sps_virtual_boundary_pos_y[ i ] < min( ( sps_subpic_ctu_top_left_y[ subpicIdx ] + 
			sps_subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY ÷ 8 , pps_pic_height_in_luma_samples ÷ 8)) 
			subpicVBy[ numHorVBs++ ] = sps_virtual_boundary_pos_y[ i ] −
										sps_subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY ÷ 8 
	}

- When sps_virtual_boundaries_present_flag is equal to 1, rewrite the values of the sps_num_ver_virtual_boundaries, sps_virtual_boundary_pos_x[], sps_num_hor_virtual_boundaries, and sps_virtual_boundary_pos_y[] in all the reference SPS NAL units to be equal to numVerVBs, subpicVBx[ i ], numHorVBs and subpicVBy[ j ] and i is in the range of 0 to numVerVBs, exclusive, and j is in the range of 0 to numHorVBs, exclusive. The virtual boundaries outside the extracted subpicture are removed. When both numVerVBs and numHorVBs are equal to 0, rewrite the value of sps_virtual_boundaries_present_flag in all the referenced SPS NAL units to 0 and remove the syntax elements sps_num_ver_virtual_boundaries, sps_virtual_boundary_pos_x[ i ], sps_num_hor_virtual_boundaries, and sps_virtual_boundary_pos_y[ i ].

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).