| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0135 |
| --- | --- |

| Title: | AHG9: On IRAP and GDR picture signaling in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Tsung-Chuan Ma, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, three methods are proposed to prevent signaling one picture as one IRAP or GDR picture in picture header when pps_mixed_nalu_types_in_pic_flag in PPS is equal to one: 1) adding one constraint for ph_gdr_or_irap_pic_flag to be 0 when pps_mixed_nalu_types_in_pic_flag is equal to 1; 2) conditioning the presence of ph_gdr_or_irap_pic_flag on pps_mixed_nalu_types_in_pic_flag being equal to 0; 3) adding one constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

Introduction

In VVC draft 9 [1], IRAP and GDR pictures are defined as pictures for which all the NAL units have one same NAL unit type which is equal to IDR_W_RADL, IDR_N_LP or CRA_NUT for IRAP pictures and GDR_NUT for GDR pictures. Therefore, for IRAP and GDR pictures, the feature of mixed NAL unit types should be disallowed, i.e., pps_mixed_nalu_types_in_pic_flag is equal to 0.

gradual decoding refresh (GDR) picture: A picture for which each VCL NAL unit has nal_unit_type equal to GDR_NUT.

instantaneous decoding refresh (IDR) picture: An IRAP picture for which each VCL NAL unit has nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

On the other hand, in VVC draft 9 [1], two flags, namely ph_gdr_or_irap_pic_flag and ph_gdr_pic_flag, are signaled at the beginning of picture header to indicate whether one current picture is an IRAP/GDR picture. When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0, the current picture is an IRAP picture. When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 1, the current picture is a GDR picture. However, in the current specification, there is no constraint between ph_gdr_or_irap_pic_flag and pps_mixed_nalu_types_in_pic_flag such that pps_mixed_nalu_types_in_pic_flag of one IRAP/GDR picture (i.e., when ph_gdr_or_irap_pic_flag is equal to 1) can be equal to 1.

Proposal

To resolve the above problem, three solutions are proposed in this contribution.

Solution One

In the first solution, it is proposed to add one constraint for ph_gdr_or_irap_pic_flag to be 0 when pps_mixed_nalu_types_in_pic_flag in the associated PPS is equal to 1.

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture. When pps_mixed_nalu_types_in_pic_flag is equal to 1, ph_gdr_or_irap_pic_flag shall be equal to 0.

Solution Two

In the second solution, it is proposed to make the presence of ph_gdr_or_irap_pic_flag conditioned on the value of pps_mixed_nalu_types_in_pic_flag being equal to 0, and add the inference that ph_gdr_or_irap_pic_flag is equal to 0 when not present.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| if( pps_mixed_nalu_types_in_pic_flag ) |  |
| ph_gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture. When ph_gdr_or_irap_pic_flag is not present, it is inferred to be 0.

Solution Three

In the third solution, in the same way as how IRAP picture is defined, it is proposed to add one constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

gradual decoding refresh (GDR) picture: A picture for which each VCL NAL unit has nal_unit_type equal to GDR_NUT.

NOTE – The value of pps_mixed_nalu_types_in_pic_flag for a GDR picture is equal to 0. When pps_mixed_nalu_types_in_pic_flag is equal to 0 for a picture, and any slice of the picture has nal_unit_type equal to GDR_NUT, all other slices of the picture have the same value of nal_unit_type, and the picture is known to be a GDR picture.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).