| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0138-v5 |
| --- | --- |

| Title: | AHG9: A summary of proposals on general constraints information (GCI) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yong He (Qualcomm) Ye-Kui Wang, Zhipin Deng (Bytedance) | Email: | yonghe@qti.qualcomm.com yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Qualcomm Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 31 proposals on general constraints information (GCI) (the agenda item 3.1.3 in JVET-S0137 and in JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposed items

## On the GCI syntax structure

- Approaches to enable skipping all GCI syntax elements

  - Add a present flag for general_constraint_info( ) syntax structure. (S0050 #1, S0127)

    - Infer the values of the GCI flags to 0. (S0050 #1a, S0127)

    - Infer the values of max_bitdepth_constraint_idc to 8 and the value of max_chroma_format_constraint_idc value to 3. (S0050 #1). Or infer the value of max_bitdepth_constraint_idc to 2 and max_chroma_format_constraint_idc to 1. (S0127)

    - Move the alignment bits outside the GCI syntax structure and change the GCI reserved bytes to bits (S0050 #1b).

  - Move the extension length indicator (gci_num_reserved_bytes) from last to first (gci_num_constraint_bytes) in the GCI syntax structure to enable skip all GCI SE signaling when all GCI SEs values are 0. The value of gci_num_reserved_bytes shall be equal to either 0 or 9. (S0092)

- Categorize GCI SEs into substructures

- Into 11 sub-structures (format, functionality, NALU, partition, pred, intra, inter, transform/quantization/residual, loop filter, layer, SEI) and add inference value in the semantics (S0105), each substructure is gated by a present flag and, when not present, the value of the corresponding constraint flag is inferred to be 0 (S0105 alt)

- Into 3 sub-structures (capability, coding tool, NALU) (S0111)

- Rearrange GCI SEs to put intra-related GCI flags together, and inter-related GCI flags together (S0129 #3)

- Result of an offline side activity on conditional signalling of GCI fields (S0179)

- Replace the while-loop for GCI byte alignment with 6 zero bits (S0209 #3)

## New GCI SEs

- Weighted prediction

- Two GCI flag, one for each of sps_weighted_pred_flag and sps_weighted_bipred_flag (S0050 #2, S0105, S0131)

- One GCI flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag (S0058)

- no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag (S0050 #2, S0105, S0131)

- no_explicit_scaling_list_constraint_flag/no_scaling_list_constraint_flag to constrain

- sps_explicit_scaling_list_enabled_flag shall be equal to 0 (S0050 #2, S0058 S0066, S0067, S0105)

- and aps_params_type shall not be equal to 2 (S0066)

- no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma (S0058)

- max_luma_transform_size_32_constraint_flag or no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag (S0058, S0066)

- no_long_term_ref_pic_constraint_flag to constrain sps_long_term_ref_pics_flag (S0058)

- max_log2_ctu_size_constraint_idc (u(2)) to constrain sps_log2_ctu_size_minus5 (S0066)

- max_layers_constraint_idc to constrain vps_max_layers_minus1 (S0113)

- max_sublayers_constraint_idc to constrain vps_max_sublayers_minus1 (S0113)

- max_subpics_constraint_idc to constrain sps_num_subpics_minus1 (S0113)

- no_pic_partition_constraint_flag to constrain pps_no_pic_partition_flag (S0113)

- no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag (S0113)

- one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- no_conformance_window_constraint_flag to contrain sps_conformance_window_flag (S0131)

- general_single_ols_per_layer_set_constraint_flag to constraint no two OLSs contain the same layer set (S0103#2)

- no_scalability_constraint_flag to constraint scalable and layered coding (S0105)

- no_360_video_constraint_flag to constrain sps_ref_wraparound_enabled_flag, and equirectangular projection SEI messages or generalized cubemap projection SEI messages (S0105)

- lossless_coding_constraint_flag to constrain all picture are losslessly coded (S0054 #8)

- no_scc_constraint_flag to constrain sps_ibc_enabled_flag, sps_bdpcm_enabled_flag and sps_palette_enabled_flag (S0105)

- no_VUI_constraint_flag to constrain sps_vui_parameters_present_flag (S0113)

- SEI related new GCI flags

- no_scalable_nesting_SEI_constraint_flag to constrain scalable nesting SEI (S0105)

- no_subpic_level_SEI_constraint_flag to constrain subpicture level SEI (S0105)

- no_filler_payload_SEI_constraint_flag to constrain filler payload SEI (S0105)

- no_user_data_reg_SEI_constraint_flag to constrain user data registered by Recommendation ITU T.35 SEI (S0105)

- no_user_data_unreg_SEI_constraint_flag to constrain data unregistered SEI (S0105)

- no_film_grain_SEI_constraint_flag to constrain film grain characteristics SEI (S0105)

- no_parameter_set_incl_SEI_constraint_flag to constrain parameter sets inclusion indication SEI (S0105)

- no_decoded_picture_hash_SEI_constraint_flag to constrain decoded picture hash SEI (S0105)

- no_mcdv_SEI_constraint_flag to constrain mastering display colour volume SEI (S0105)

- no_cll_SEI_constraint_flag to constrain content light level SEI (S0105)

- no_drap_sei_constraint_flag to constrain DRAP indication SEI (S0105, S0113)

- no_alt_transfer_char_SEI_constraint_flag to constrain alternative transfer characteristics SEI (S0105)

- no_ambient_view_envir_SEI_constraint_flag to constrain ambient viewing environment SEI (S0105)

- no_ccv_SEI_constraint_flag to constrain content colour volume SEI (S0105)

- no_omni_video_SEI_constraint_flag to constrain ERP, GCMP, sphere rotation, region-wise packing and omnidirectional viewport SEI (S0105, S0113)

- no_field_frame_SEI_constraint_flag to constrain frame-field information SEI (S0105)

- no_sar_SEI_constraint_flag to constrain sample aspect ratio SEI (S0105)

- general_non_HRD_constraint_flag to constrain BP, PT and DU SEI (S0113)

- Result of an offline side activity on GCI flags, for items 4-22 in the AHG report JVET-S0137 (S0161)

- Remove general_non_packed_constraint_flag and general_non_projected_constraint_flag (S0195 #1)

- Additional new GCI flags

  - no_subpic_info_constraint_flag to constrain sps_subpic_info_present_flag (S0195 #3)

  - no_idr_rpl_constraint_flag to constrain sps_idr_rpl_present_flag (S0195 #3)

  - no_hrd_constraint_flag to constrain vps_general_hrd_params_present_flag and sps_general_hrd_params_present_flag (S0195 #3)

  - no_bidirectional_prediction_constraint_flag to constrain pps_rpl1_idx_present_flag and and num_ref_entries[ 1 ][ rplsIdx ] (S0209 #1)

  - no_parallel_merge_constraint_flag to constrain sps_log2_parallel_merge_level_minus2 (S0209 #1)

  - no_switchable_cabac_init_constraint_flag to constrain pps_cabac_init_present_flag (S0209 #1)

  - no_entropy_coding_sync_constraint_flag to constrain sps_entropy_coding_sync_enabled_flag (S0230)

## GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs

Subpicture, slices, and tiles related

- When one_tile_per_pic_constraint_flag is equal to 1, NumTilesInPic shall be equal to 1. (S0050 #3b)

- When one_subpic_per_pic_constraint_flag is equal to 1,

- sps_num_subpics_minus1 shall be equal to 0. (S0050 #3c, S0112)

- sps_subpic_info_present_flag shall be equal to 0. (S0105)

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0054 #3, S0106 #5, S0112, S0131)

- When one_slice_per_pic_constraint_flag is equal to 1, pps_rect_slice_flag shall be equal to 1. (S0050#5c, S0106 #6)

- When one_slice_per_pic_constraint_flag is equal to 1, pps_num_slices_in_pic_minus1 shall be equal to 0. (S0050#5c)

- When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are both equal to 1, the value of pps_no_pic_partition_flag shall be equal to 1. (S0050 #5, S0112)

Single-layer, single-picture related

- Change the semantics of single_layer_constraint_flag equal to 1 from "single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0" to one of the following: (S0090)

- Option 1: single_layer_constraint_flag equal to 1 specifies that one of the following conditions shall be fulfilled (1) sps_video_parameter_set_id is equal to 0; (2) The value of vps_max_layers_minus1 in the referred VPS is equal to 0 and all VCL NAL units in the CVS have the same value of nuh_layer_id.

- Option 2: single_layer_constraint_flag equal to 1 specifies that all VCL NAL units in the CVS shall have the same value of nuh_layer_id.

- When single_layer_constraint_flag is equal to 1, all_layers_independent_constraint_flag shall be equal to 1. (S0050 #4, S0054 #6)

- When general_one_picture_only_constraint_flag is equal to 1,

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0050 #4)

- no_trail_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_stsa_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_rasl_constraint_flag shall be equal to 1 (S0050 #4, S0054#5)

- no_radl_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

Intra only related

- When intra_only_constraint_flag is equal to 1,

- all_layers_independent_constraint_flag shall be equal to 1 (S0050 #4, S0106 #4. S0112)

- no_ref_pic_resampling_constraint_flag shall be equal to 1 (S0050 #4, S0112, S0131)

- no_res_change_in_clvs_constraint_flag shall be equal to 1 (S0050 #4)

- no_sbt_constraint_flag shall be equal to 1 (S0050 #4, S0060, S0106 #1, S0112, S0129 #3, S0131)

- no_trail_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_stsa_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_rasl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_radl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_cra_constraint_flag shall be equal to 1 (S0106 #3)

- no_gdr_constraint_flag shall be equal to 1 (S0106 #3)

- no_idr_constraint_flag shall be equal to 0 (S0106 #3)

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0112)

- sps_weighted_pred_flag shall be equal to 0. (S0061)

- sps_weighted_bipred_flag shall be equal to 0. (S0061)

- sps_long_term_ref_pics_flag shall be equal to 0. (S0061)

- sps_inter_layer_ref_pics_present_flag shall be equal to 0. (S0061)

- sps_idr_rpl_present_flag shall be equal to 0. (S0061)

- sps_rpl1_same_as_rpl0_flag shall be equal to 0. (S0061)

- sps_num_ref_pic_lists[ i ] shall be equal to 0. (S0061)

- sps_sbtmvp_enabled_flag shall be equal to 0. (S0061)

- sps_six_minus_max_num_merge_cand shall be equal to 0. (S0061)

- sps_log2_parallel_merge_level_minus2 shall be equal to 0. (S0061)

- sps_explicit_mts_inter_enabled_flag shall be equal to 0. (S0061)

- pps_rpl1_idx_present_flag shall be equal to 0 (S0061)

- pps_rpl_info_in_ph_flag shall be equal to 0 (S0061)

- pps_ref_wraparound_enabled_flag shall be equal to 0 (S0061)

- pps_output_flag_present_flag shall be equal to 0 (S0112)

- ph_pic_output_flag shall be equal to 1 (S0112)

- ph_inter_slice_allowed_flag shall be equal to 0 (S0061, S0112)

- ph_non_ref_pic_flag shall be equal to 1 (S0112)

Random access related

- When no_cra_constraint_flag is equal to 1, no_rasl_constraint_flag shall be equal to 1. (S0054 #4, S0112)

- When no_idr_constraint_flag is equal to 1 and no_cra_constraint_flag is equal to 1, no_radl_constraint_flag shall be equal to 1. (S0054 #4)

- At least one of the no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0. (S0050 #5, S0112)

APS related

- When no_alf_constraint_flag is equal to 1, aps_params_type shall not be equal to 0. (S0066)

- When no_lmcs_constraint_flag equal to 1, aps_params_type shall not be equal to 1. (S0066)

- When no_aps_constraint_flag is equal to 1,

- no_lmcs_constraint_flag shall be equal to 1. (S0050 #4, S0067#1, S0105)

- no_ccalf_constraint_flag shall be equal to 1. (S0050 #4)

- no_explicit_scaling_list_constraint_flag shall be equal to 1. (S0050 #4, S0067 #2, S0105)

- sps_ccalf_enabled_flag, ph_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_num_alf_aps_ids_luma, sh_alf_cb_flag, and sh_alf_cr_flag all be equal to 0. (S0050 #5, S0073 #1.1)

- (alternative to 15.d and 15.f) sps_chroma_alf_enabled_flag (newly added as part of this item), sps_chroma_alf_enabled_flag and sh_num_alf_aps_ids_luma shall be equal to 0 (S0073 #1.2)

- (alternative to 15.d and 15.e) sps_alf_enabled_flag and sps_ccalf_enabled_flag shall both be equal to 0 (S0073 #1.3)

Video format related

- The value of the u(4)-coded max_bitdepth_constraint_idc shall be in the range of 0 to 8, inclusive. (S0046 #1, S0050 #3a)

- When max_chroma_format_constraint_idc is equal to 0, no_chroma_qp_offset_constraint_flag shall be equal to 1. (S0046 #3, S0060, S0112, S0131)

- When max_chroma_format_constraint_idc is equal to 0, no_act_constraint_flag shall be equal to 1. (S0131)

Others

- When no_ref_pic_resampling_constraint_flag to 1, no_res_change_in_clvs_constraint_flag shall be equal to 1. (S0054 #7, S0105, S0112, S0131)

- When no_transform_skip_constraint_flag is equal to 1, no_bdpcm_constraint_flag shall be equal to 1. (S0050 #4, S0059, S0069#1, S0105, S0112, S0131)

- When no_affine_motion_constraint_flag is equal to 1, no_prof_constraint_flag shall be equal to 1. (S0046 #2, S0112, S0131)

- When all_layers_independent_constraint_flag is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0 (S0050 #5)

## GCI and profiles

- Either change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag. (S0091 #1) or remove general_one_picture_only_constraint_flag (S0160 #7)

- Constrain the values of max_chroma_format_constraint_idc and max_bitdepth_constraint_idc in main 10 profile (S0094)

## GCI editorial changes

- Rename "max_bitdepth_constraint_idc" to "max_bitdepth_minus8_constraint_idc" (S0105)

- Rename "general_non_packed_constraint_flag" to "general_non_packed_SEI_constraint_flag", and "general_non_projected_constraint_flag" to "general_non_projected_SEI_constraint_flag "(S0105)

- Move the constraint "When general_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0" from SPS semantic to GCI semantic (S0054 #1, S0105, S0209 #2) and/or rename "general_frame_only_constraint_flag" to "no_field_seq_constraint_flag" and change the semantics to constrain sps_field_seq_flag (S0195 #2)

- Move the constraint "When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0" from PPS semantics to GCI semantics. (S0054 #2, S0209 #2)

- Prefix all syntax elements in the general constraint information with ‘gci_’ (S0209 #4)

- Rename "no_tsrc_constraint_flag" to "no_ts_rrc_constraint_flag" (S0238 late)

# List of proposals on general constraint information

JVET-S0046 AHG9: General constraint information semantics constraints [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, D. Liu (Ericsson)]

This contribution proposes the following modifications to the general constraint information semantics:

- Mandate the value of max_bitdepth_constraint_idc to be in the range of 0 to 8, inclusive. It is reported that the max_bitdepth_constraint_idc constraints the range of sps_bit_depth_minus8 to be from 0 to max_bitdepth_constraint_idc, inclusive and that sps_bit_depth_minus8 is constrained to be in the range of 0 to 8, inclusive. It is further reported that the max_bitdepth_constraint_idc is u(4) coded. It is asserted that the values of max_bitdepth_constraint_idc being 9 to 15 convey meaningless constraint information since the sps_bit_depth_minus8 is restricted to be not larger than 8 anyway. It is therefore suggested to restrict the value of max_bitdepth_constraint_idc to avoid potential confusion.

- Mandate the value of no_prof_constraint_flag to be equal to 1 when no_affine_motion_constraint_flag is equal to 1. It is reported that when no_affine_motion_constraint_flag is equal to 1, sps_affine_enabled_flag is equal to 0 which disables affine motion prediction, including PROF.

- Mandate the value of no_chroma_qp_offset_constraint_flag to be equal to 1 when max_chroma_format_constraint_idc is equal to 0. It is reported when max_chroma_format_constraint_idc is equal to 0, sps_chroma_format_idc is equal to 0 which specifies the chroma format to be monochrome.

JVET-S0050 AHG9: On general constraints information [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, K. Fan (Bytedance)]

This contribution proposes the following changes on the general constraints information (GCI):

- Add a new flag, named gci_present_flag, in the profile_tier_level( ) syntax structure to condition the presence of general_constraint_info( ) syntax structure.

  - When gci_present_flag equal to 0 for a profile_tier_level( ) syntax structure with profileTierPresentFlag equal to 1, the value of max_bitdepth_constraint_idc is inferred to be equal to 8, the value of max_chroma_format_constraint_idc is inferred to be equal to 3, and the value of the each of the other syntax elements in the general_constraint_info( ) syntax structure starting from general_non_packed_constraint_flag to no_aps_constraint_flag, inclusive, is inferred to be equal to 0.

  - To keep the starting position of general_level_idc to be a byte-aligned position, move the byte alignment check outside the general_constraint_info( ), change the GCI syntax elements gci_num_reserved_bytes to gci_num_reserved_bits, coded with u(11) instead of u(8), and change the GCI syntax element gci_reserved_byte[ i ] to gci_reserved_bit[ i ], coded with u(1) instead of u(8).

- Add the following new GCI flags in the general_constraint_info( ) syntax structure:

  - Add no_weighted_pred_constraint_flag to constrain the value of sps_weighted_pred_flag.

  - Add no_weighted_bipred_constraint_flag to constrain the value of sps_weighted_bipred_flag.

  - Add no_virtual_boundaries_constraint_flag to constrain the value of sps_virtual_boundaries_enabled_flag.

  - Add no_explicit_scaling_list_constraint_flag to constrain the value of sps_explicit_scaling_list_enabled_flag.

- Change the semantics of the following GCI flags to be as follows:

  - The value of max_bitdepth_constraint_idc is required to be in the range of 0 to 8, inclusive.

  - one_tile_per_pic_constraint_flag equal to 1 specifies that the value of NumTilesInPic that applies to each picture shall be equal to 1.

  - one_subpic_per_pic_constraint_flag equal to 1 specifies that the value of sps_num_subpics_minus1 shall be equal to 0.

- Use some GCI flags to constrain the values of other GCI flags as follows:

  - When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

  - When general_one_picture_only_constraint_flag is equal to 1, the values of no_mixed_nalu_types_in_pic_constraint_flag, no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, and no_radl_constraint_flag shall all be equal to 1.

  - When intra_only_constraint_flag is equal to 1, the values of all_layers_independent_constraint_flag, no_ref_pic_resampling_constraint_flag, no_res_change_in_clvs_constraint_flag, no_weighted_pred_constraint_flag, no_weighted_bipred_constraint_flag, and no_sbt_constraint_flag shall all be equal to 1.

  - When single_layer_constraint_flag is euqal to 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

  - When no_lmcs_constraint_flag, no_explicit_scaling_list_constraint_flag,  no_ccalf_constraint_flag is equal to 0, the value of no_aps_constraint_flag shall be equal to 0.

- Use some GCI flags to constrain the values of some SPS/PPS/PH/SH syntax elements, but specified in the GCI semantics, as follows:

  - When no_aps_constraint_flag is equal to 1, the values of ph_num_alf_aps_ids_luma, slice_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_alf_cb_flag, sh_alf_cr_flag, and sps_ccalf_enabled_flag shall all be equal to 1.

  - When all_layers_independent_constraint_flag is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0.

  - When one_slice_per_pic_constraint_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1 and the value of pps_num_slices_in_pic_minus1 shall be equal to 0.

  - When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are both equal to 1, the value of pps_no_pic_partition_flag shall be equal to 1.

- Add a constrain such that at least one of the no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0.

JVET-S0054 AHG9: Clean-ups on general constraint flags [B. Choi, S. Wenger, S. Liu (Tencent)]

The following modifications to constraint flags in general constraint information (GCI) syntax structure are proposed:

- Move the following constraint sentence from the semantics of sps_field_seq_flag to the semantics of general_frame_only_constraint_flag in GCI, to avoid any semantics dependency on GCI.

- When general_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0.

- Remove the following constraint sentence from the semantics of pps_mixed_nalu_types_in_pic_flag, to avoid any semantics dependency on GCI and remove the redundant description.

- When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0.

- Add the following constraint to the semantics of no_mixed_nalu_types_in_pic_constraint_flag.

- When one_subpic_per_pic_constraint_flag is equal to 1, the value of no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

- Add the following constraints to the semantics of no_rasl_constraint_flag and no_radl_constraint_flag, respectively.

- When no_cra_constraint_flag is equal to 1, the value of no_rasl_constraint_flag shall be equal to 1.

- When no_idr_constraint_flag is equal to 1 and no_cra_constraint_flag is equal to 1, the value of no_radl_constraint_flag shall be equal to 1.

- Add the following constraints to the semantics of no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, no_radl_constraint_flag and no_trail_constraint_flag, respectively.

- When general_one_picture_only_constraint_flag is equal to 1, no_stsa_constraint_flag shall be equal to 1.

- When general_one_picture_only_constraint_flag is equal to 1, no_rasl_constraint_flag shall be equal to 1.

- When general_one_picture_only_constraint_flag is equal to 1, no_radl_constraint_flag shall be equal to 1.

- When general_one_picture_only_constraint_flag is equal to 1, no_trail_constraint_flag shall be equal to 1.

- Add the following constraints to the semantics of all_layers_independent_constraint_flag.

- When single_layer_constraint_flag is equal to 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

- Add the following constraints to the semantics of no_res_change_in_clvs_constraint_flag.

- When no_ref_pic_resampling_constraint_flag to 1, the value of no_res_change_in_clvs_constraint_flag shall be equal to 1.

- Add a new constraint flag lossless_coding_constraint_flag that imposes a constraint that all pictures are losslessly coded in the bitstream of the OlsInScope.

JVET-S0058 AHG9: Additional General Constraint Flags [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

This contribution proposes adding general constraint flags for the following coding tools: MTT, Weighted Prediction, long term reference picture prediction, scaling lists, and maximum transform size.

JVET-S0059 AHG9: Fixing the Semantics of no_bdpcm_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

This contribution proposes imposing the value of no_bdpcm_constraint_flag to 1 when no_transform_skip_constraint_flag is 1, where BDPCM is currently disabled if transform skip is disabled. This is to be inline with other constraint flags and to reduce the conformance testing effort.

JVET-S0060 AHG9: Fixing the Semantics of no_chroma_qp_offset_constraint_flag and no_sbt_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

This contribution proposes imposing the values of no_chroma_qp_offset_constraint_flag to 1 when max_chroma_format_constraint_idc is zero, and no_sbt_constraint_flag to one when intra_only_constraint_flag is one. This is to follow the same method as for other chroma related and inter-related constraint flag.

JVET-S0061 AHG9: HLS Cleanup of All-Intra Profile [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

The general constraint flag “intra_only_constraint_flag” can be used to define a profile where all images are coded with intra prediction. With VVC draft 9, such a profile can come with several non-deactivated non-intra related tools and features. This contribution proposes imposing a constraint such that these tools/features are completely deactivated by their HLS flags when intra_only_constraint_flag is set to 1.

JVET-S0066 AHG9: On constraint flags [C. Rosewarne, J. Gan (Canon)]

The VVC draft 9 specification provides a set of constraint flags to deactivate certain coding tools, pictures types, image formats, and other categories of constraint flags. It is asserted that adding flags to also constrain certain aspects of block structure would be beneficial and in keeping with the specificity with which other coding tools are constrained. This contribution proposes adding constraint flags for CTU size and 64-pt transform availability. Also proposed are a scaling list constraint flags, and adding constraints on aps_params_type according to the constraint flags.

JVET-S0067 AHG9: General constraint information on features using APS [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes the following modifications to the general constraint information on features using APS:

- Mandate the value of no_lmcs_constraint_flag to be equal to 1 when no_aps_constraint_flag is equal to 1.

- Add a new general constraint flag no_explicit_scaling_list_constraint_flag constraining sps_explicit_scaling_list_enabled_flag to be equal to 0.

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 1 of this contribution belongs to this category.

This contribution proposes following modifications to the general constraint information of features related to transform skip mode:

- Mandate the value of no_bdpcm_constraint_flag to be equal to 1 when no_transform_skip_constraint_flag is equal to 1.

- ...

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

This contribution proposes two aspects.

The first aspect of this contribution proposes a bug-fix to the  no_aps_constraint_flag. In the current design, ALF and CCALF process can be enabled when the value of no_aps_constraint_flag is equal to 1. However, it is asserted that the chroma ALF and CCALF process cannot be executed without signaling of filter sets in APS. The aspect 1 of this contribution proposes to disable chroma ALF and CCALF when the value of no_aps_constraint_flag is equal to 1. Three options are proposed. In the first option, a semantic constraint is applied to the no_aps_constraint_flag to disable CCALF and chroma ALF. In the second option, a new SPS level flag is introduced to control chroma ALF process and the new SPS flag is disabled if no_aps_constraint_flag == 1. In the third option, both ALF and CCALF processes are disabled in SPS level.

The second aspect ...

JVET-S0090 AHG8/AHG9: On single_layer_constraint_flag [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

According to the general constraint information semantics of the current VVC draft, if the value of single_layer_constraint_flag is equal to 1, sps_video_parameter_set_id shall be equal to 0. It means that a single layer bitstream using this constraint flag is prohibited from referring to any VPS. However, VVC also supports single layer bitstreams that refer to a VPS with a syntax indicating only one layer. This contribution proposes to modify the description of the single_layer_constraint_flag semantics to support both single layer structures with and without VPS referencing.

JVET-S0091 AHG8/AHG9: On constraints of still picture [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

Aspect 2 of this contribution belongs to this category.

According to the specification of the still picture profile in Annex A, general_one_picture_only_constraint_flag is constrained to be set equal to 1. As a consequence, intra_only_constraint_flag and single_layer_constraint_flag shall also be equal to 1. If it is appropriate for VVC to support still pictures with multi layers, the description regarding these constraints should be changed. This contribution proposes to modify these constraint flags to enable the support of still pictures with multi layers.

After the conclusion of May 2020 HLS AHG meeting, the constraint related to general_one_picture_only_constraint_flag in still picture profile has been removed. However, it still has the definition that a still picture shall contain only one picture. So, the version 2 of this contribution continues to propose the following two items:

1.	Modify the definition of still picture to one AU instead of one picture.

2.	Change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag.

JVET-S0092 On constraint info signalling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution presents a proposal for a modification to the constraint info signalling of the VVC draft. It is proposed to move the extension length indicator from last to first in the constraint info syntax structure in order to allow skipping of signalling constraint info when all constraint flags are zero. The full specification text integrated in JVET-R2001-vA.docx and a software patch for VTM master branch (commit 8d9573f1ec214868e1a4e61e38991c5a07b86449) are attached to this contribution.

JVET-S0094 AHG9: Profile-level constraints on chroma format and bit depth constraint flags [K. Sharman, S. Keating, A. Browne (Sony)]

In the current draft of VVC, the bit depth (“bit_depth_minus8”) and the chroma format (“chroma_format_idc”) are constrained by the constraint flags “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc”. In addition, they are constrained in the specification of each profile.

However, there is no constraint of specification of what “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc” should be for each profile. This means that it is possible for a stream to indicate maximums of 16 bit 4:4:4, but only be a 4:2:0 10-bit profile.

The constraint flags are present in the VPS, describing each layer, and optionally present in the SPS.

Four options are explored, and it is proposed that a change is made to remove this ambiguity.

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 2 of this contribution belongs to this category.

The contribution asserts that it is an issue that the TargetOlsIdx in some cases cannot be detected from the BitstreamToDecode itself and proposes two aspects to overcome the asserted issue.

Aspect 1)	Add a rule to select among OLSs matching BitstreamToDecode.

Aspect 2)	Add a constraint flag so that each layer set shall match to one OLS

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

All aspects other than the aspect on changing the semantics of no_tsrc_constraint_flag belong to this category.

This contribution proposes modifications of syntax and semantics for signalling general constraint information (gci). The main aspect of the proposal is to signal categorized limited-scope general constraint information syntax structures within the existing general constraint information syntax structure, i.e., signal the following:

general_format_constraint_info( ) specifying only those gci flags related to formats

general_functionality_constraint_info( ) specifying only those gci flags related to functionality

general_NALU_constraint_info( ) specifying only those gci flags related to NAL unit type

general_partiton_constraint_info( ) specifying only those gci flags related to partitions

general_pred_mode_constraint_info( ) specifying only those gci flags related to prediction modes

general_intra_constraint_info( ) specifying only those gci flags related to intra prediction

general_inter_constraint_info( ) specifying only those gci flags related to inter prediction

general_tqr_constraint_info( ) specifying only those gci flags related to transform, quantization, and residual coding

general_loop_filter_constraint_info( ) specifying only those gci flags related to loop filters

general_layer_constraint_info( ) specifying only those gci flags related to layers

general_SEI_constraint_info( ) specifying only those gci flags related to SEI messages

This contribution also proposes additional general constraint information flags and minor modifications of the names or semantics of several existing general constraint information flags.

JVET-S0106 AHG9: Some cleanups on general constraint flags [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

This contribution proposes the following modifications of the general constraint information semantics:

- When intra_only_constraint_flag equal to 1, no_sbt_constraint_flag shall be equal to 1.

- When intra_only_constraint_flag equal to 1, no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, no_radl_constraint_flag shall be equal to 1.

- When intra_only_constraint_flag equal to 1, no_cra_constraint_flag and no_gdr_constraint_flag shall be equal to 1 and no_idr_constraint_flag shall be equal to 0.

- When intra_only_constraint_flag equal to 1, all_layers_independent_constraint_flag shall be equal to 1.

- When one_subpic_per_pic_constraint_flag equal to 1, no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

- When one_slice_per_pic_constraint_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1.

JVET-S0111 AHG9: On GCI syntax structure [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes to restructure GCI syntax elements into 3 subgroups, general_capability_constraint_info(), general_coding_tool_constraint_info() and general_nalu_constraint_info().

JVET-S0112 AHG9: On GCI semantic constraints [Y. He, N. Hu, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes the semantic constraints on the existing GCI flags:

- all_layers_independent_constraint_flag

- no_ref_pic_resampling_constraint_flag

- no_res_change_in_clvs_constraint_flag

- one_subpic_per_pic_constraint_flag

- no_sbt_constraint_flag

- no_prof_constraint_flag

- no_bdpcm_constraint_flag

- no_chroma_qp_offset_constraint_flag

- no_mixed_nalu_types_in_pic_constraint_flag

- no_trail_constraint_flag

- no_stsa_constraint_flag

- no_rasl_constraint_flag

- no_radl_constraint_flag

The contribution also proposes to constrain the following syntax elements with the GCI flags:

- sps_num_subpics_minus1

- pps_no_pic_partition_flag

- ph_inter_slice_allowed_flag

- pps_output_flag_present_flag

- ph_non_ref_pic_flag

- ph_pic_output_flag

JVET-S0113 AHG9: On new GCI flags [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes several new GCI flags as follows:

- max_layers_constraint_idc

- max_sublayers_constraint_idc

- max_subpics_constraint_idc

- no_pic_partition_constraint_flag

- no_rectangular_slice_constraint_flag

- no_raster_scan_slice_constraint_flag

- one_slice_per_subpicture_constraint_flag

- no_VUI_constraint_flag

- general_non_omnidirectional_video_constraint_flag

- general_non_dependent_RAP_constraint_flag

- general_non_HRD_constraint_flag

JVET-S0127 AHG9: On general constraint info signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

This contribution proposes to add a flag to signal the presence of general_constraint_info() syntax structure in profile level tier indication to enable skipping of signalling of this information when all contraint flags are equal to 0.

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 3 of this contribution belongs to this category.

The contribution proposes following changes:

Aspect 1: add inference value for syntax elements in VPS

Aspect 2: introduce sps_one_picture_only_constraint_flag in SPS to skip inter, RPL and POC related syntax elements in both SPS and PH

Aspect 3: rearrange syntax elements in GCI like SPS and PH, and add inference rules

JVET-S0131 AHG9: On general constraint information syntax [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution proposes the following modifications to the syntax related to general constraint information in the VVC draft specification version 9:

- Add conformance constraints for existing control flags

- Constrain the inter prediction-related constraint flags to be equal to 1 when intra_only_constraint_flag is equal to 1 (i.e., when slice_type is I for all slices).

- Constrain the chroma-related constraint flags to be equal to 1 when max_chroma_format_constraint_idc is equal to 0 (i.e., when there are no chroma components).

- Constrain the subpicture-related constraint flags to be equal to 1 when one_subpic_per_pic_constraint_flag is equal to 1 (i.e., when there are no subpictures).

- Constrain the affine mode-related constraint flags to be equal to 1 when no_affine_motion_constraint_flag is equal to 1 (i.e., when there is no affine mode).

- Constrain the transform skip-related constraint flags to be equal to 1 when no_transform_skip_constraint_flag is equal to 1 (i.e., when there is no transform skip mode).

- Constrain the picture resampling-related constraint flags to be equal to 1 when no_ref_pic_resampling_constraint_flag is equal to 1 (i.e., when there is no picture resampling).

- Add new control flags

- Add general constraint flags for the coding tools.

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 7 of this contribution belongs to this category.

7) Remove general_one_picture_only_constraint_flag.

JVET-S0161 AHG9: Report of side activity on GCI syntax elements [J. Boyce]

Side activity was conducted via email to make a recommendation on inclusion of Global Constraint Indicator (GCI) syntax elements, for items 4-22 in the AHG report JVET-S0137.

The recommendations are summarized as follows:

- Add syntax elements for the following:

  - no_weighted_pred_constraint_flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag (S0058)

  - no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag (S0050 #2, S0105, S0131)

  - no_explicit_scaling_list_constraint_flag to constrain sps_explicit_scaling_list_enabled_flag shall be equal to 0 (S0050 #2, S0058 S0066, S0067, S0105)

  - no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma (S0058)

  - no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag (S0058, S0066)

  - max_log2_ctu_size_constraint_idc (u(2)) to constrain sps_log2_ctu_size_minus5 (S0066)

  - no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag (S0113)

  - one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- Additional recommendations

  - Remove any constraints that existing constraint flags impose on other constraint flags

  - Rename max_bitdepth_constraint_idc to max_bitdepth_minus8_constraint_idc

  - Remove no_aps_constraint_flag

The attached document implements these changes in marked specification text, based upon JVET-R2001-vA. The attached text does not implement other GCI adoptions from the AHG, such as reordering of GCI syntax elements. No attempt was made to intelligently order the additional syntax elements in the attached text, because it is assumed that will be done as part of activity in reordering of the syntax elements.

JVET-S0179 AHG9: On conditional signalling of GCI fields [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang, K. Fan (Bytedance), J. Samuelsson, S. Deshpande, A. Segall (Sharp), M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm), Hendry, J. Nam (LGE), S. Wenger, B. Choi (Tencent), M. Zhou (Broadcom)]

This contribution presents the result of an offline side activity regarding the following minutes of the May 2020 HLS AHG meeting in JVET-S0137:

- AHG Recommendation: Agree in spirit to allow some form of skipping the GCI syntax. The exact method was agreed to be for further study to be determined later.

- It was commented that we should not have category-wise skipping. Although some interest was expressed by a proponent, no action was recommended on that aspect.

The following design is proposed: Signal the GCI syntax structure between the general PTL fields and sub-profile fields in the PTL syntax structure. Signal the gci_present_flag at the beginning of the GCI syntax structure and use it to condition the GCI fields, move the byte alignment to the end of the GCI syntax structure, and change the reserved bytes to reserved bits.

JVET-S0195 AHG9: On GCI cleanups [Y. He, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes the following GCI cleanups:

- Remove GCI flags on SEI messages, general_non_packed_constraint_flag and general_non_projected_constraint_flag

- Rename general_frame_only_constraint_flag to no_field_seq_constraint_flag and update semantic to constrain sps_field_seq_flag

- Propose 3 additional GCI flags

  - no_subpic_info_constraint_flag to constrain sps_subpic_info_present_flag

  - no_idr_rpl_constraint_flag to constrain sps_idr_rpl_present_flag

  - no_hrd_constraint_flag to constrain vps_general_hrd_params_present_flag and sps_general_hrd_params_present_flag

JVET-S0209 AHG9: On General Constraint Information [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

This contribution proposes the following items for the general constraint information in VVC:

- Add three new general constraint flags. (normative)

- no_bidirectional_prediction_constraint_flag

- no_parallel_merge_constraint_flag

- no_switchable_cabac_init_constraint_flag

- Move the restriction text for the general_frame_only_constraint_flag and no_mixed_nalu_types_in_pic_constraint_flag from the SPS and PPS semantics to only have it in the semantics of the general constraint flags. It is asserted that all other general constraint flags are specified purely in the semantics of the general constraint flags. (editorial)

- Replace the while-loop for byte alignment in general_constraint_info() with a fixed set of bits. It is asserted that the while-loop is not needed since the syntax element prior to the byte alignment are all u(n) coded. (editorial)

- Prefix all syntax elements in the general constraint information with ‘gci_’. (editorial)

JVET-S0230 AHG9: New GCI flag [T. Ikai (Sharp)]

This contribution proposes a new GCI flags as follows:

- no_entropy_coding_sync_constraint_flag

JVET-S238 AHG9: GCI for RRC [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)] (late)

In VVC draft 9, a new constraint flag “no_tsrc_constraint_flag” was introduced for defining a profile/sub-profiles where only regular residual coding (RRC) is used, i.e., transform skip residual coding (TSRC) is disabled. It is asserted that that such a profile requires both dependent quantization and sign data hiding to be disabled, which leads to significant coding loss. This contribution proposes instead to modify this flag to restrict RRC usage for transform skip coded blocks. The proposed modification to the specification is just renaming “no_tsrc_constraint_flag” to “no_ts_rrc_constraint_flag” since the current semantics of no_tsrc_constraint_flag already support this proposal.